| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0034-v2 |
| --- | --- |

| Title: | CE14: Summary report on post-reconstruction filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang Sergey Ikonin | Tel Email: | lizhang.idm@bytedance.com sergey.ikonin@huawei.com |  |
| Source: | CE14 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 14 on post-reconstruction filtering methods. The techniques are evaluated according to BD-rate gain, complexity (in both encoder and decoder).

The software basis for the mandatory test in this CE is BMS-2.0.1, for VTM based comparisons the BMS software is configured to produce VTM-2.0.1. For the optional test, BMS-2.1 is used as the anchor with BMS tools enabled. Test sequences, configurations and test conditions are according to JVET-K1010 [1] for SDR.

## Test conditions

| Sub-CE | Tool on (VTM_tool_test) | Tool off (BMS_tool_test) |
| --- | --- | --- |
| 14.1-14.3 | BMS2.0.1 with VTM configuration + sub-CE test vs BMS2.0.1 with VTM | BMS2.1 + sub-CE test vs BMS2.1 |

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 14.1.a | Turning off the filtering for 4x4 intra and inter blocks. No LUT. | JVET-L0172 |
| 14.1.b | Turning off the filtering for 4x4, 4x8 and 8x4 intra/inter blocks. No LUT. | JVET-L0172 |
| 14.1.c | Bilateral filter with LUT rows consisting of more than 16 8-bit values (provided as reference). | JVET-L0172 |
| 14.2.a | In-loop bilateral filter (also operated after block reconstruction, i.e. affecting subsequent intra prediction), weights presented with piece-wise linear model (PWL), 10 ranges (pieces) applied to blocks with min(W, H)<=8 | JVET-L0406 |
| 14.2.b | In-loop bilateral filter (also operated after block reconstruction, i.e. affecting subsequent intra prediction), weights presented with PWL, 2 ranges (pieces) applied to blocks with min(W, H)<=8 | JVET-L0406 |
| 14.2.c* | In-loop bilateral filter (also operated after block reconstruction, i.e. affecting subsequent intra prediction), weights presented with PWL, 2 ranges (pieces) applied to blocks with min(W, H)<=16, similar to tests 14.1 and 14.3 | JVET-L0406 /JVET-L0584 |
| 14.3a | Hadamard Transform Domain Filter with LUT 140 bytes | JVET-L0326 |
| 14.3b | Hadamard Transform Domain Filter with LUT size 70 bytes | JVET-L0326 |

* No results available on T3, no SW available on T3

## Comparisons of technologies

| Test | filter shape | Comp. complex. per sample* | Precis. of mult | Parallel friendly | Latency (in clock cycles) | Memory. required (bytes) | How to derive filter coeffs | Min. and max. filtered CU size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1.a | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | Intra: 4 mult 9 adds 4 checks Inter: 4 mult 23 adds 10 checks | Intra: 9×8 and 12×9 Inter: 9×8 and 12×11 | yes | At very high clock freq: Intra:10 Inter: 11 Estimation at lower clock freq: 3-4 clock cycles. | 63 | Intra: max0, kqp×ΔI+mqp Inter: max0, kqp×NL+mqp | Min: 4x8, 8x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| 14.1.b | —”— | —”— | —”— | —”— | —”— | —”— | —”— | Min: 8x8 Max: same as 14.1.a |
| 14.2.a, LUT based | 5 pixel “plus”-shape Inter: w(x) with NL average | Intra: 2 mult 8 adds 2 checks Inter: 2 mult 18 ads 5 checks | 32 bits registers | yes | Intra: 2 Inter: 3 | ROM: 120 CU level: <370*2 | Computed prior to CU: idx1= compar(x,R) idx2=x-R[idx1] max0, Widx1+d[idx1]×idx2 | Min: 4x8, 8x4 Max: Intra: 64x64 Inter: 8x64, 64x8 |
| 14.2.b LUT based | —”— | —”— | —”— | —”— | —”— | ROM: 120 CU level: <210*2 | Computed prior to CU: idx= x≫BS idx2=x-idx1 max0, Widx1+d[idx1]×idx2 | —”— |
| 14.2.c LUT based | —”— | —”— | —”— | —”— | —”— | —”— | —”— | Min: 4x8, 8x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| 14.2.a** LUT Free | —”— | Intra: 4 mult 12 adds 13 checks Inter: 4 mult 22 ads 16 checks | 32 bits registers | yes | Intra: 4 Inter: 5 | ROM: 120 CU level: 24 | idx1= compar(x,R) idx2=x-R[idx1] max0, Widx1+d[idx1]×idx2 | —”— |
| 14.2.b** LUT free | —”— | Intra: 4 mult 12 adds 4 checks Inter: 4 mult 22 ads 7 checks | —”— | —”— | Intra: 3 Inter: 4 | ROM: 120 CU level: 10 | idx= x≫BS idx2=x-idx1 max0, Widx1+d[idx1]×idx2 | —”— |
| 14.2.c ** LUT free | —”— | —”— | —”— | —”— | —”— | —”— | —”— | Min: 4x8, 8x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| 14.3.a | 3x3 | 0 mult 20 adds + 4 1-bit add for rounding 6 checks | n/a | yes | 1 clock: @770MHz 16nm @450MHz 28nm 2 clocks: @770MHz 28nm | 140 (32 7-bit values per qp group) | Precalculated in LUT | Min: 4x8 and 8x4 Max: Intra: 64x64 Inter: 16x64 or 64x16 |
| 14.3.b | —”— | —”— | —”— | —”— | —”— | 70 (16 7-bit values per qp group) | —”— |  |

## * max/min/abs are countered as checks

** Ideal results in terms of complexity analysis, simulation results missing, software is missing

## Test results and crosscheck reports for mandatory tests

*: SIMD optimization for the proposed technology

**: running time unreliable

***: different SIMD settings (other than default settings) are used

### VTM_tool_test

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 14.1.a | -0.39% | 0.12% | 0.16% | 103% | 105% | -0.67% | -0.10% | -0.14% | 105% | 102% | -0.64% | 0.59% | 0.39% | 103% | 103% |
| 14.1.b | -0.30% | 0.09% | 0.11% | 105% | 103% | -0.58% | -0.10% | -0.10% | 105% | 102% | -0.64% | 0.45% | 0.51% | 105% | 101% |
| 14.1.c * | -0.42% | 0.14% | 0.18% | 105% | 104% | -0.71% | -0.05% | -0.11% | 104% | 103% | -0.75% | 0.30% | 0.38% | 103% | 104% |
| 14.2.a | -0.43% | 0.20% | 0.18% | 113% | 110% | -0.60% | -0.12% | -0.07% | 87%** | 83%** | -0.75% | 0.2% | 0.03% | 103% | 101% |
| 14.2.b | -0.42% | 0.13% | 0.16% | 114% | 108% | -0.59% | -0.13% | -0.11% | 93%** | 103% | -0.75% | 0.37% | 0.03% | 104% | 102% |
| 14.2.c | -0.42% | 0.13% | 0.16% | 108% | 109%* | -0.71% | -0.21% | -0.17% | 106%* | 110%* | -0.71% | 0.48% | 0.38% | 92%** | 103% |
| 14.3.a | -0.48% | 0.28% | 0.31% | 109% | 110% | -0.70% | -0.14% | -0.19% | 105% | 104% | -0.68% | 0.40% | 0.67% | 104% | 104% |
| 14.3.b | -0.47% | 0.28% | 0.32% | 109% | 110% | -0.70% | -0.17% | -0.06% | 105% | 104% | -0.66% | 0.14% | 0.31% | 103% | 104% |

### *) Test CE14.1c uses a model implementation of the LUT (the LUT is expanded at initialization). Simulation results are given for reference purpose only.

### Crosscheck report of mandatory test results

| Test# | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW implemented by cc? (Y/N) | SW studied? (Y/N) | significant inconsistencies between the description and the provided software? (Y/N) | Other comments? | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1.a | J. Rasch | Y | Y | N | Y | N | - | N |
| 14.1.b |  | Y | Y | N | Y | N | - | N |
| 14.1.c |  | Y | Y | N | Y | N | - | N |
| 14.1.a | S. Ikonin | Y | Y | N | Y | N | See Comment #1 | N |
| 14.1.b |  | Y | Y | N | Y | N | Same as above | N |
| 14.1.c |  | Y | Y | N | Y | N | See Comment #2 | N |
| 14.1.a | D. Rusanovskyy | Y |  | N | Y | N |  | N |
| 14.1.b |  |  |  |  |  |  |  |  |
| 14.1.c |  |  |  |  |  |  |  |  |
| 14.2.a | E. Francois | Y | Y | N | Y | N |  | N |
| 14.2.b |  | Y | Y | N | Y | N |  | N |
| 14.2a | S. Ikonin | Y | N | N | Y | Y | See Comment #3 | N |
| 14.2b |  | Y | N | N | Y | Y | Same as above | N |
| 14.3.a | J. Ström | Y |  | partly | Y |  | See Comment #4 |  |
| 14.3.b |  | Y |  | partly | Y |  | Same as above |  |
| 14.3.a | D. Rusanovskyy |  |  |  |  |  |  |  |
| 14.3.b |  | Y |  | N | Y | N |  | N |

## Comment #1:

On 14.1a/b
Total amount of operations per sample matches to numbers provided by proponent.
Operation cascade for one sample filtering to crosschecker’s opinion:
Intra: 2 mult, 6 add, 2 check
Inter: 2 mult, 12 add, 2 check

## Comment #2:

On 14.1c.
LUT expanded into RAM with total size of 180 kBytes. ROM size 992 bytes.
Amount of operations: 
Intra: 3 mult, 7 add, 4 check
Inter: 3 mult, 21 add, 8 check 
Operation cascade for one sample filtering to crosschecker’s opinion:
Intra: 2 mult, 5 add, 2 checks
Inter: 2 mult, 10 add, 2 check

## Comment #3:

On 14.2a/b

SW uses LUT expanded to the RAM of total size 540kBytes (6x1024 entries of uint16 = 12kBytes per QP).

ROM size: 1226 bytes including source tables for LUT generation (122 bytes) and maxPosList (1104 bytes). Memory requirements does not match to numbers provided by proponent.

For inter 3x4 area is used to calculate filter weights.

In contrast with SW the contribution JVET-L0406 (r1 Oct, 1st) describes rather how filtering potentially can be implemented in on-the-fly LUT-free way. SW has LUT-based only implementation, there is no option to run the SW in LUT-free way. In revision3 (Oct 2) 2 potential implementations and complexity estimations are further provided.

According to cross-checker’s analysis of provided source code the following number of operations is obtained (for LUT based version):

Intra: 2 mult, 7 add, 4 check

Inter: 4 mult, 43 add, 22 check
Does not match to numbers provided by proponent

Operation cascade for one sample filtering according to cross-checker’s opinion:

Intra: 1 mult, 5 add, 2 check

Inter: 2 mult, 10 add, 3 check,

Latency number estimation is not clear.

On 14.2c
Only ‘LUT-based’ version (published by Oct, 3rd) is implemented in SW and has been tested. SW contains two modifications: 16x64 blocks for inter and further tuning of filtering parameters. The ‘LUT-free’ version is not presented in SW and was not tested.

On 14.2a/b/c ‘LUT-free’

SW has LUT only implementation, there is no option to run the SW in LUT-free way

Amount of operations needed for table generation is not clear enough from the source code (runtime conditions, branches etc.), roughly: 3 mult, 2 add, 1 max per one LUT entry in addition to filtering; the declaration of int64 and double variables observed in SW for table generation. However, the cross-checker doesn’t state that this accuracy is required for computation, further analysis and source code clean is required for firm understanding of required bit-depth. Total amount of operations and complexity estimations for filtering cannot not be done clearly based on SW provided.

## Comment #4:

On 14.3a/b
Subsampled LUT can lead to quantization effects in brightness ramps (see details in JVET-L0636)

## Cross-check reports

| JVET number | Title |
| --- | --- |
| JVET-L0636 | CE14: Crosscheck of CE14.3 (JVET-L326) |

## Test results and crosscheck reports for optional tests

### BMS_tool_test

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 14.1.a | -0.39% | 0.08% | 0.11% |  |  | -0.74% | 0.01% | -0.02% |  |  | -0.69% | 0.64% | 0.46% |  |  |
| 14.1.b | -0.31% | 0.06% | 0.06% |  |  | -0.67% | -0.09% | -0.06% |  |  | -0.63% | 0.53% | 0.57% |  |  |
| 14.1.c | -0.43% | 0.11% | 0.10% |  |  | -0.79% | -0.14% | -0.06% |  |  | -0.76% | 0.49% | 0.58% |  |  |
| 14.2.a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.2.b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.3a | -0.51% | 0.26% | 0.22% | 109% | 109% | -0.80% | -0.12% | -0.12% | 107% | 102% | -0.68% | 0.56% | 0.81% | 105% | 104% |
| 14.3b | -0.49% | 0.22% | 0.20% | 109% | 109% | -0.77% | -0.12% | -0.06% | 107% | 102% | -0.67% | 0.53% | 0.34% | 105% | 104% |

### Crosscheck report

| Test# | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW implemented by cc? (Y/N) | SW studied? (Y/N) | significant inconsistencies between the description and the provided software? (Y/N) | Other comments? | Other tests? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1.a | J. Rasch |  |  |  |  |  |  |  |
| 14.1.b |  |  |  |  |  |  |  |  |
| 14.1.c |  |  |  |  |  |  |  |  |
| 14.1.a | S. Ikonin | Y | Y | N | Y | N |  |  |
| 14.1.b |  | Y | Y | N | Y | N |  |  |
| 14.1.c |  | Y | Y | N | Y | N | LUT expanded into RAM | N |
| 14.1.a | D. Rusanovskyy |  |  |  |  |  |  |  |
| 14.1.b |  |  |  |  |  |  |  |  |
| 14.1.c |  |  |  |  |  |  |  |  |
| 14.2.a | E. Francois |  |  |  |  |  |  |  |
| 14.2.b |  |  |  |  |  |  |  |  |
| 14.3.a | J. Ström | Y |  | N | Y | N | see comment #4 |  |
| 14.3.b |  | Y |  | N | Y | N | see comment #4 |  |
| 14.3.a | D. Rusanovskyy |  |  |  |  |  |  |  |
| 14.3.b |  |  |  |  |  |  |  |  |

## Test results for additional tests

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 14.3.b SSE | -0.47% | 0.28% | 0.32% | 107% | 106% | -0.70% | -0.17% | -0.06% | 104% | 102% | -0.66% | 0.14% | 0.31% | 102% | 103% |

- 14.3b SSE – additional test with filter implemented in SSE.

## Detailed description of tests

### CE14.1 Reduced complexity bilateral filter JVET-L0172

[1] Technology to be investigated:

The basis of this experiment will be the bilateral filter presented in JVET-K0274. Three experiments were carried out:

14.1a: LUT-free, replaced by a clipped line, do not filter 4×4 blocks (intra or inter).
14.1b: The same as 14.1a, but also avoid filtering 4x8 and 8x4 blocks

14.1c: This is the same filter as JVET-K0274, LUT based

By comparing 14.1a with 14.1b it is possible to see how much BD-rate is lost when removing also 4×8 and 8×4 blocks from the filtering, and this figure is about 0.1% for AI and RA. By comparing 14.1a with 14.1c it is possible to see how much is lost by approximating the LUT with a clipped linear function. The result is about 0.03% for AI and RA, and about 0.1% for LD. The 14.1c is included for BD-rate reference and is not proposed by the proponent. Hence the table in 1.3 is not filled out for 14.1c.

Information about latency and throughput of filtering in terms of number of clock cycles per sample is also solicited. Here a worst-case type approach was used, similar to having a very high clock frequency. The main idea was to compare 14.1a/14.1b with the contribution from the previous meeting, JVET-K0274. A latency reduction of around 21% (inter) and 9% (intra) was found for 14.1a/14.1b using this clock cycle model compared to JVET-K0274. In a more realistic (non-worst-case) clock frequency the latency is estimated to be around 3-4 clock cycles.

### CE14.2 Bilateral filter JVET-L0406

### [1] Technology to be investigated:

Bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkabs(Pk, 0-P0, 0)×Pk, 0-P0, 0, 			 (1)

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk(∙) are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4), in the shape of a plus sign, is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x) 					(2)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,

Rangekx=e– x28*QP – 17*(QP – 17)

and σd is dependent on the coded mode and coding block sizes. To further improve the coding performance, for inter-coded blocks, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference among two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)              (3)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

This family of weighting functions Wkx, QP, mode produced from Eq. 2 is parameterized by a piece-wise linear model from a basis function W0(x), which is coarsely represented with a small number of pivot points W0 and derivation scaler for QP Sqp and mode Sm. The following data is tabulated and provide to decoder as a side information (ROM):

W0 = {42950, 37903, 26050, 13944, 5813, 1887, 477, 94, 14, 2}

Sm = { 30195, 19316, 26068, 19316, 1470 };

Sqp = { 65535, 43690, 32768, 26214, 21845, 18724, 16384, 14563, 13107, 11915, 10922, 10082, 9362, 8738, 8192, 7710, 7281, 6898, 6553, 6241, 5957, 5698, 5461, 5242, 5041, 4854, 4681, 4519, 4369, 4228, 4096, 3971, 3855, 3744, 3640, 3542, 3449, 3360, 3276, 3196, 3120, 3048, 2978, 2912, 2849};

It is proposed that weight for bilateral filter can be derived through a process of piece-wise linear interpolation either prior to CU decoding and provided as LUT, or on-fly with a LUT-free approach depending on application requirements. Complexity and memory estimates are reported for both, the LUT-based and LUT-free approaches. LUT based approach was selected for implementation in simulation software.

Following experiments were carried out:

- E14.2.a tests BIF with 10-ranges piece-wise linear model;

- CE14.2.b tests the reduced complexity of the piece-wise linear model to 2 non-zero ranges,

- CE14.2.c test extends applicability of CE14.2.b to maximum filtered block size 16x64 and 64x16

### CE14.3 Hadamard Transform Domain Filter JVET-L0326

Hadamard transform domain filter is always applied to luma reconstructed blocks with non-zero transform coefficients, excluding 4x4 blocks and if slice quantization parameter is larger than 17. The filter parameters are explicitly derived from the coded information. Proposed filter, if applied, is performed on decoded samples right after inverse transform. The filter has same implementation both for intra and inter CU.

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+m*σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, m=4 is normalization constant equal to number of spectrum components, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2.64*2(0.1296*QP-11)

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of all pixels in CU the accumulated values are normalized by number of processing groups used for each pixel filtering.

Two experiments were carried out:

- CE14-3a with LUT size 140 bytes: 32 entries of 7-bit values per 5 qp group;

- CE14-3b with LUT size 70 bytes: 16 entries of 7-bit values per 5 qp group.

## Related contributions

| JVET number | Title |
| --- | --- |
| JVET-L0357 | CE14 related: Adaptive colour space clipping filter |
| JVET-L0465 | Cross-check of JVET_L0357 - CE14 related: Adaptive colour space clipping filter |
| JVET-L0584 | CE14.2-related: Extended applicability of bilateral filter for CE14.2 |
| JVET-L0465 | Cross-check of JVET_L0357 - CE14 related: Adaptive colour space clipping filter |
| JVET-L0583 | Crosscheck of JVET-L0357 (CE14 related: Adaptive colour space clipping filter) |
| JVET-L0615 | CE14-related: Inter-only bilateral filtering |
| JVET-L0656 | CE14.3-related: Hadamard transform domain filter with modified LUT |

# Appendix

| Test | Proponent(s) and proposal studied | Cross-checker(s) |
| --- | --- | --- |
| CE14.1 | Jacob Ström jacob.strom@ericsson.com JVET-L0172 | Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de Sergey Ikonin sergey.ikonin@huawei.com Dmytro Rusanovskyy Dmytror@qti.qualcomm.com |
| CE14.2 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-L0406 | Francois Edouard Edouard.Francois@technicolor.com |
| CE14.3 | Sergey Ikonin sergey.ikonin@huawei.com JVET-L0326 | Jacob Ström jacob.strom@ericsson.com Dmytro Rusanovskyy Dmytror@qti.qualcomm.com |