| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0046-v2 |
| --- | --- |

| Title: | CE4-related: On line buffer reduction for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes for the affine motion data inheritance from a neighboring PU located at the top CTU boundary, i.e.

- If the neighboring PU uses the 4-parameter model, or the neighboring PU has PU width 8, uses the 6-parameter model and bi-directional prediction, the 4-parameter model using the bottom-left and bottom-right control point motion vectors (CPMVs) is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and unidirectional prediction, the 6-parameter model using the bottom-left, bottom-right and top-left CPMVs is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and bidirectional prediction but its PU width is larger than 8, the 6-parameter model using the bottom-left,  bottom-right CPMVs and bottom-middle two sub-block vectors is applied to the inheritance.

For 4K video for example, the proposed changes reduce the line buffer requirement for the affine mode from 9,216 bytes to 256 bytes without comprising coding efficiency. Compared to the VTM2.0.1 anchor, the average BD-rate differences are 0.02% in RA, 0.01% in LD-B and 0.03% in LD-P, respectively.

# Introduction

In CE tests defined in [5], the 4-paramaeter affine motion model is always applied to the affine motion data inheritance from a neighboring PU located at the top CTU boundary, regardless whether the neighboring PU uses 4- or 6-parameter affine motion model. This could cause a relatively large quality loss (e.g. in Class A2 sequences) if the neighboring PU uses the 6-parameter model while the inheritance uses the 4-parameter model. The algorithm presented in this contribution is aiming for addressing this issue.

No change is introduced to the affine motion data inheritance if the source neighboring PU is not located along the top CTU boundary. The proposed changes only apply to the affine motion data inheritance from a neighboring PU located at the top CTU boundary.

The proposed changes are as follows:

- If the neighboring PU uses the 4-parameter model, or the neighboring PU has PU width 8, uses the 6-parameter model and bi-directional prediction, the 4-parameter model using the bottom-left and bottom-right control point motion vectors (CPMVs) is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and unidirectional prediction, the 6-parameter model using the bottom-left, bottom-right and top-left CPMVs is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and bidirectional prediction, and PU width larger than 8, the 6-parameter model using the bottom-left,  bottom-right CPMVs and bottom-middle two sub-block vectors is applied to the inheritance.

The proposed algorithm provides better results as the mathematically equivalent inheritance (to the current VTM2.0.1 method) is applied to all the cases, except for the case in which the source neighboring PU located along the top CTU boundary has PU width 8, uses the 6-parameter model and bi-directional prediction.

# Algorithm description

The proposed algorithm is described in detail in this section.

Figure 1: Neighboring PU E uses the 4-parameter model, or uses 6-parameter model, bi-directional prediction and PU width equal to 8 (case 1)

As shown in Figure 1, if the current PU inherits the affine motion data from the neighboring PU E which is located along the top CTU boundary, and the neighboring PU E uses the 4-parameter model, or it uses the 6-parameter model, bi-directional prediction and has the PU width 8, the control point vectors v0 and v1 of the current PU are derived by using the 4-parameter model, and by

v0x=vLE1x-vLE0x(xLE1-xLE0)(x0-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0y(xLE1-xLE0)(x0-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y0-yLE0)+vLE0y               Equation 1

v1x=vLE1x-vLE0x(xLE1-xLE0)(x1-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0y(xLE1-xLE0)(x1-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y1-yLE0)+vLE0y               Equation 2

And if the current PU uses the 6-parameter affine motion model, the control point vectors v2 is derived by

v2x=vLE1x-vLE0x(xLE1-xLE0)(x2-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y2-yLE0)+vLE0xv2y=vLE1y-vLE0y(xLE1-xLE0)(x2-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y2-yLE0)+vLE0y               Equation 3

Figure 2: Neighboring PU E uses the 6-parameter model and unidirectional prediction (case 2)

Otherwise, if the neighboring PU E uses the 6-paramter model and unidirectional prediction, the bottom-left, bottom-right and top-left CPMVs (either list 0 or list 1) can be buffered in the regular motion data line buffer. The following mathematically equivalent inheritance method (to the current VTM2.0.1 method) can be employed. In this case, the control point vectors v0 and v1 of the current PU are derived by using the 6-parameter model with the bottom-left, bottom-right and top-left CPMVs, and by

v0x=vLE1x-vLE0xxLE1-xLE0x0-xLE0+vLE0x- vE0x(yE0-yLE0)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0yxLE1-xLE0(x0-xLE0)+vLE0y- vE0y(yE0-yLE0)(y0-yLE0)+vLE0y               Equation 4

v1x=vLE1x-vLE0xxLE1-xLE0x1-xLE0+vLE0x- vE0x(yE0-yLE0)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0yxLE1-xLE0(x1-xLE0)+vLE0y- vE0y(yE0-yLE0)(y1-yLE0)+vLE0y               Equation 5

And if the current PU uses the 6-parameter affine motion model, the control point vectors v2 is derived by

v2x=vLE1x-vLE0xxLE1-xLE0x2-xLE0+vLE0x- vE0x(yE0-yLE0)(y2-yLE0)+vLE0xv2y=vLE1y-vLE0yxLE1-xLE0(x2-xLE0)+vLE0y- vE0y(yE0-yLE0)(y2-yLE0)+vLE0y               Equation 6

Figure 3: Neighboring PU E uses the 6-parameter model and bi-directional prediction but PU width larger tan 8 (case 3)

Otherwise, if the neighboring PU E uses the 6-paramter model and bi-directional prediction but the PU size is larger than 8, the bottom-left, bottom-right CPMVs (both list 0 and list 1) can be buffered in the regular motion data line buffer. The following mathematically equivalent inheritance method (to the current VTM2.0.1 method) can be applied by using additional motion data information carried in the two bottom-middle sub-block vectors stored in the line buffer. In this case, the control point vectors v0 and v1 of the current PU are derived by using the 6-parameter model with the bottom-left, bottom-right CPMVs and bottom-middle two sub-block motion vectors, and by

v0x=vLE1x-vLE0xxLE1-xLE0x0-xLE0+vLE1x+vLE0x-vLEc1x-vLEc0x2(yLE1-yLEc1)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0y(xLE1-xLE0)(x0-xLE0)+vLE1y+vLE0y-vLEc1y-vLEc0y2(yLE1-yLEc1)(y0-yLE0)+vLE0y  Equation 7

v1x=vLE1x-vLE0xxLE1-xLE0x1-xLE0+vLE1x+vLE0x-vLEc1x-vLEc0x2(yLE1-yLEc1)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0y(xLE1-xLE0)(x1-xLE0)+vLE1y+vLE0y-vLEc1y-vLEc0y2(yLE1-yLEc1)(y1-yLE0)+vLE0y  Equation 8

And if the current PU uses the 6-parameter affine motion model, the control point vectors v2 is derived by

v2x=vLE1x-vLE0xxLE1-xLE0x2-xLE0+vLE1x+vLE0x-vLEc1x-vLEc0x2(yLE1-yLEc1)(y2-yLE0)+vLE0xv2y=vLE1y-vLE0y(xLE1-xLE0)(x2-xLE0)+vLE1y+vLE0y-vLEc1y-vLEc0y2(yLE1-yLEc1)(y2-yLE0)+vLE0y    Equation 9

In all three cases, the location selection of the bottom-left and bottom-right CPMVs satisfy the following conditions:

xLE0=xE0xLE1=xE1yLE0=y0yLE1=y1             Equation 10

In case 3, the location selection of the bottom-middle sub-block vectors satisfy the following additional conditions:

yLEc1=yLEc0yLE1=yLEc1+2xLE0+xLE1=xLEc0+xLEc1                 Equation 11

Note that Equations 1 to 9 are for algorithm description purpose. In actual implementation, the divisions are replaced with multiplies and shifts.

# Experimental results

Tests were carried out by using the VTM2.0.1 software, and the common test conditions defined in [1] were followed.

The summary results are provided in Table 1.  On average, the BD-rate differences are 0.02% in RA, 0.01% in LD-B and 0.03% in LD-P, respectively.

Table 1: Summary results of the proposed algorithm

For comparsion, an additional test is carried out in which the mathematically equivalent inheritance method (to the current VTM2.0.1 method) is applied to all the cases, i.e.

- If the neighboring PU uses the 4-parameter model, the 4-parameter model using the bottom-left and bottom-right CPMVs is applied to the inheritance (see Equations 1, 2 and 3).

- Otherwise, if the neighboring PU uses the 6-parameter model, the 6-parameter model using the bottom-left, bottom-right and top-left CPMVs is applied to the inheritance (see Equations 4, 5, and 6).

Unlike the proposed algorithm, this reference method cuts the line buffer of affine mode only by half, as the top-left CPMV still needs to be stored in a separate line buffer. As shown in Table 2, the average BD-rate differences are 0.01% in RA, 0.01% in LD_B and 0.03% in LD_P, respectively.

Table 2: Summary results of the reference method

# Line buffer size analysis

The line buffer size reduction was analyzed for 4K video as example. As shown in Table 3, the line buffer size for the affine mode can be reduced from 9216 bytes to 256 bytes for 4K video by using the proposed algorithm.

Table 3: Line buffer size for 4K video

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, the 11th JVET meeting, July 2018, Ljubljana, SI.

[2] H. Yang, S. Liu and K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

[3] M. Zhou and B. Heng, “Non-CE4: A study on the affine merge mode”, JVET-K0052, the 11th JVET meeting, July 2018, Ljubljana, SI.

[4] Y. Han, H. Huang, Y. Zhang, C. Hung, C. Chen, W. Chien and M. Karczewicz, “CE4.1.3: Affine motion compensation prediction”, JVET-K0337,  the 11th JVET meeting, July 2018, Ljubljana, SI.

[5] M. Zhou, “CE4: Test results of CE4.1.11 on line buffer reduction for affine mode”, JVET-L0045, the 12th JVET meeting, Oct. 2018, Macao, CN.

[6] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001-v6, the 11th JVET meeting, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-K1001-v6

#### Section 8.3.3.3 Derivation process for luma affine control point motion vectors from a neighbouring block

(Note to Ed: changes are also need for sections where 8.3.3.3 is invoked, i.e. adding numNCpMv,   cpNMvLX[ cpIdx ], and mvLX[ xSbIdx ][ ySbIdx ] to the input list)

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- a luma location ( xNb, yNb ) specifying the top-left sample of the neighbouring luma coding block relative to the top-left luma sample of the current picture,

- two variables nNbW and nNbH specifying the width and the height of the neighbouring luma coding block,

- the number of control point motion vectors numNCpMv of the neighbouring luma coding block,

- the luma affine control point vectors of the neighbouring luma coding block cpNMvLX[ cpIdx ] with cpIdx = 0 .. numNCpMv − 1 and X being 0 or 1,

- the luma motion vectors of the neighbouring luma coding block mvLX[ xSbIdx ][ ySbIdx ], with xSbIdx = 0..( nNbW >> 2 ) – 1, ySbIdx = 0..( nNbH >> 2 ) – 1 and X being 0 or 1,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variables log2NbW and log2NbH are derived as follows:

log2NbW = Log2( nNbW )		(8-265)

log2NbH = Log2( nNbH ) 		(8-266)

The variable isCTUboundaryPU set equal to 1 if all the following consitions are true. Otherwise, it is set to 0.

- ((yNb + nNbH) % CtbSizeY) is equal to 0

- And yNb + nNbH is equal to yCb

The variables mvScaleHor and mvScaleVer are derived as follows:

- If  isCTUboundaryPU is equal to 1, the following applies:

mvScaleHor = mvLX[ 0 ][ (nNbH>>2) -1] [ 0 ] << 7	(8-267)

mvScaleVer = mvLX[ 0 ][ (nNbH>>2) -1 ] [ 1 ] << 7 	(8-268)

- Otherwise, the following applies:

mvScaleHor = cpNMvLX[ 0 ][ 0 ] << 7		(8-267)

mvScaleVer = cpNMvLX[ 0 ] [ 1 ] << 7 		(8-268)

The variables dHorX and dVerX are derived as follows:

- If  isCTUboundaryPU is equal to 1, the following applies:

dHorX =  mvLX[ (nNbW>>2)-1 ][ (nNbH>>2)-1 ][ 0 ] − mvLX[ 0 ][ (nNbH>>2)-1  ][ 0 ]  	(8-269)

dVerX =  mvLX[ (nNbW>>2)-1 ][ (nNbH>>2)-1 ][ 1 ] − mvLX[ 0 ][ (nNbH>>2)-1  ][ 1 ]  	(8-270)

dHorX =  dHorX <<  ( 7 − log2NbW )	(8-269)

dVerX =  dVerX <<  ( 7 − log2NbW )	(8-269)

- Otherwise,  the following applies:

dHorX = ( cpNMvLX[ 1 ][ 0 ] − cpNMvLX[ 0 ][ 0 ] ) << ( 7 − log2NbW )	(8-269)

dVerX = ( cpNMvLX[ 1 ][ 1 ] − cpNMvLX[ 0 ][ 1 ] )  << ( 7 − log2NbW )	(8-270)

The variables dHorY and dVerY are derived as follows:

- If isCTUboundaryPU is equal to 1, numNCpMv is equal to 3 and inter_pred_idc[ xNb ][ yNb ] is equal to PRED_L0 or PRED_B0, the following applies:

dHorY = ( mvLX[ 0 ][ (nNbH>>2)-1][ 0 ] − cpNMvLX[ 0 ][ 0 ]) << ( 7 − log2NbH ) 	(8-271)

dVerY = ( mvLX[ 0 ][ (nNbH>>2)-1][ 1 ] − cpNMvLX[ 0 ][ 1 ] ) << ( 7 − log2NbH ) 	(8-272)

- Otherwise, if isCTUboundaryPU is equal to 1, numNCpMv is equal to 3 and nNbW is greater than 8, the following applies:

dHorY =  mvLX[ 0 ][ (nNbH>>2)-1][ 0 ] + mvLX[ (nNbW>>2)-1 ][ (nNbH>>2)-1 ][ 0 ] 	(8-271)

dVerY =  mvLX[ 0 ][ (nNbH>>2)-1][ 1 ] + mvLX[ (nNbW>>2)-1 ][ (nNbH>>2)-1 ][ 1 ] 	(8-272)

dHorY = dHorY - mvLX[(nNbW>>3)-1][(nNbH>>2)-1][ 0 ] - mvLX[(nNbW>>3)][(nNbH>>2)-1][ 0 ] 	(8-271)

dVerY =  dVerY - mvLX[(nNbW>>3)-1][(nNbH>>2)-1][ 1 ] - mvLX[(nNbW>>3)][(nNbH>>2)-1][ 1 ] 	(8-272)

dHorY =  dHorX <<  5	(8-269)

dVerY =  dVerX <<  5	(8-269)

- Otherwise, if isCTUboundaryPU is equal to 0 and numNCpMv is equal to 3, the following applies:

dHorY = ( cpNMvLX[ 2 ][ 0 ]− cpNMvLX[ 0 ][ 0 ]) << ( 7 − log2NbH ) 	(8-271)

dVerY = ( cpNMvLX[ 2 ][ 0 ]− cpNMvLX[ 0 ][ 1 ] ) << ( 7 − log2NbH ) 	(8-272)

- Ohterwise (numNCpMv is equal to 2, or isCTUboundaryPU is equal to 1, numNCpMv is equal to 3, inter_pred_idc[ xNb ][ yNb ] is equal to PRED_BI and  nNbW is equal to 8), the following applies:

dHorY = − dVerX		(8-273)

dVerY =   dHorX		(8-274)

If  isCTUboundaryPU is equal to 1, yNb is set equal to yCb.

The luma affine control point motion vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1 are derived as follows:

- The first two control point motion vectors cpMvLX[ 0 ] and cpMvLX[ 1 ] are derived as follows:

cpMvLX[ 0 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb − yNb ) )	(8-275)

cpMvLX[ 0 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb − yNb ) )	8-276)

cpMvLX[ 1 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb + cbWidth − xNb ) + dHorY * ( yCb − yNb ) )	(8-277)

cpMvLX[ 1 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb + cbWidth − xNb ) + dVerY * ( yCb − yNb ) )	(8-278)

- If numCpMv is equal to 3, the third control point vector cpMvLX[ 2 ] is derived as follows:

cpMvLX[ 2 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb + cbHeight − yNb ) )	(8-279)

cpMvLX[ 2 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb + cbHeight − yNb ) )	(8-280)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to cpMvLX[ cpIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded cpMvLX[ cpIdx ] as output, with X being 0 or 1 and cpIdx = 0 .. numCpMv − 1.