| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0047-v1 |
| --- | --- |

| Title: | CE4-related: A clean up for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou and Brian Heng 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com brian.heng@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

As a clean-up of the affine mode design, it is proposed to use the same sub-block motion vector field for the motion compensation, merge/skip/AMVP list derivation, and storage of the temporal motion vector predictors (TMVPs), and to use the control point motion vectors (CPMVs) only for the affine motion data inheritance (i.e. for merge mode and affine AMVP mode).  It is asserted that the proposed changes lead to a more consistent and implementation friendly design of the affine mode, and simplify the specification text. Compared to the VTM2.0.1, the BD-rate differences are 0.01% in RA, -0.03% in LD_B and 0.02% in LD_P, respectively.

# Introduction

Figure 1: Illustration of motion vector field handling in the current design of affine mode

In the current design of the affine mode [2] implemented in the VTM2.0.1, a mixed motion vector field of the affine control point vectors (CPMVs) and sub-block motion vectors is used for the affine motion data inheritance (i.e. for affine merge and affine AMVP list derivation), for the merge/skip and AMVP list derivation (i.e. serve as spatial neighboring candidates), and for storage of temporal motion vectors (TMVPs) for use in future pictures. A separate sub-block motion vector field (computed on-the-fly) is used for the motion compensation of PUs coded in affine mode.

Figure 1 shows an example, in which a CTU is partitioned in two PUs, and both of them are coded in affine mode. As shown in Figure 1 (a), in the current design the CPMVs are stored in the top-left, top-right and bottom-left (if a PU uses 6-parameter affine motion models) sub-block locations of the sub-block motion vector field (overwriting the computed sub-block vectors for those locations).  This creates a mixed set of CPMVs and sub-block motion vectors that is used for the merge/skip/AMVP/TMVPs and affine inheritance. For motion compensation, a separate sub-block motion vector field (shown in Figure 1 (b)) is generated again as only the sub-block vectors (not control point vectors) are used in the motion compensation of affine mode.

For actual decoder implementations, the sub-block motion vector field (shown in Figure 1 (b) for example) needs to be generated in advance, and pushed to the motion compensation engine to set up pre-fetch of reference blocks for motion compensation. Before the sub-block motion vector field is consumed by the motion compensation, it cannot be overwritten by e.g. CPMVs.

To minimize the memory buffer size, one possible solution to store the CPMVs and the sub-block vectors separately, in which the CPMVs are stored in one buffer, and the sub-block vectors for the MC are stored in another, so that the sub-block vector field used for the MC won’t get overwritten before it is consumed by the MC.  For the merge/skip and AMVP list derivation and the storage of the TMVPs, the decoder would need to switch back and forth between those two motion vector buffers to fetch either the CPMVs or sub-block vectors from neighboring PUs as the spatial MV candidates or candidate vectors for TMVP storage.  Also, more storage would be needed for the CPMVs, because the non-adjacent CPMVs, which are no longer required by the merge/skip/AMVP list derivation of the current PU, cannot be disposed before they are compressed together with other sub-block motion vectors in the CTU and written out as temporal motion vectors for use in future pictures.

# Proposed clean-up

Figure 2: Illustration of motion vector field handling in the proposed clean up (clean-up method 1)

As discussed above, the CPMVs would need to be stored separately anyway in actual implementations for minimizing the memory buffer size, it makes less sense to mix the CPMVs and sub-block vectors in the merge/skip and AMVP list derivation process and in the storage of TMVPs, as doing so won’t reduce the memory footprint. It would be more straightforward and consistent to use the sub-block vectors as spatial candidates for merge/skip and AMVP list derivation and for TMVPs, and leave the CPMVs for the use of the affine motion data inheritance only.

Figure 2 shows an example for the proposed clean-up (i.e. clean-up method 1), in which the CPMVs (i.e. Figure 2 (a)) are stored separately and used for the affine inheritance only. The sub-block vector field (i.e. Figure 2 (b)) is used not only for the motion compensation, but also for the merge/skip/AMVP list derivation and for the storage of TMVPs.

Figure 3: Top-row / left-column motion vector context storage

For 128x128 CTU size, the buffers for storing all CPMVs and 4x4 based sub-block motion vectors inside the CTU are about 6,144 bytes and 8,192 bytes (just counting motion vectors, not other parameters such as PU sizes and reference indices for the sake of explanation), respectively.

Because the CPMVs are only used for the affine motion data in heritance and the non-adjacent CPMVs can be disposed, in actual implementations the CPMVs do not need to be stored for the entire CTU. In order to minimize the storage requirements, a decoder can store only a top-row and left-column of vector context, rather than a full-CTU, or a full-picture worth of vector context, as is done in VTM2.0.1.  This concept is shown in Figure 3.  The figure illustrates the vector context state after decoding CU #10.  The CPMV context is shown in green (8 pixel granularity), and the standard sub-block MV context is shown in blue (4 pixel granularity). As each CU is decoded, the corresponding left and top context are updated, shifting the blue and green lines further down and to the right, working the way across the CTU and eventually across the picture.

In this type of memory-optimized implementation, the CPMVs would be stored for each 8-pixel segment.  There would be 16 such segments for the top-row, 16 for the left-column, and 16 segments for storing top-left context. This CPMV storage would require roughly 24 bytes per segment (3 CPMVs – 8 bytes each), so 1,152 bytes (24 bytes * 48 segments) for the top/left CPMV context within the CTU.  Additional such storage per 8-pixel segment would be required across the top of the Figure (top line buffer) if affine inheritance was allowed to extend outside the current CTU, as it does in VTM 2.0.1.

The sub-block MVs would be stored for each 4-pixel segment.  There would be 32 such segments for the top-row, 32 for the left-column, and 32 segments for storing top-left context. This MV storage would require roughly 8 bytes per segment (1 bi-directional MV – 8 bytes each), so 768 bytes (8 bytes * 96 segments) for the top/left sub-block MV context within the CTU.

This kind of memory-optimized implementation effectively cuts the memory footprint for the CPMVs from 6,144 bytes to 1,152 bytes for a 128x128 CTU.  For the 64x64 block based decoder pipeline, the proposed cleanup avoids the need of buffering the CPMVs for the 64x64 blocks, which saves about 1,536 bytes memory (i.e. 6144/4 bytes) when compared to the current design in VTM 2.0.1.  In addition, this implementation supports the motion vector reconstruction on a small block basis (e.g. 64x64 instead of 128x128), which further reduces the memory footprint for storage of the sub-block motion vectors from e.g. 8,192 bytes to 2,816 bytes (i.e. 8192/4 + 768).

Figure 4: Illustration of motion vector field handling in an alternative clean up (clean-up method 2)

In an alternative clean-up illustrated in Figure 4 (i.e. clean-up method 2),  the same mixed MV field of CPMVs and sub-block vectors are used for motion compensation, in addition to the use in the merge/skip/AMVP and affine inheritance as implemented in the VTM2.0.1.  In this scenarios, two-third of the CPMV context storage (see Figure 3 (a), green buffer) can overlay on the sub-block motion vector buffer (see Figure 3 (b), blue buffer). This is the consequence of the fact, for each 8-pixel segment the CPMVs can have up to three bi-directional vectors (for 6-paramrater model), but the sub-block vectors can only have up to two bi-directional vectors. Therefore, the memory footprint saving for storing the CPMVs is 768 bytes when compared to the proposed clean-up (i.e. clean-up method 1).

Table 1 summarizes the motion vector memory buffer usage for different methods discussed above.

| Method | Local sub-block MV buffer (bytes) | Local CPMV buffer (bytes) | Local buffers for left/top-left/top neighboring MV contexts (bytes) | Local buffers for left/top-left/top neighboring CPMV contexts (bytes) | Total local memory size (bytes) |
| --- | --- | --- | --- | --- | --- |
| 128x128 CTU based VTM2.0.1 | 8,192 | 6,144 | 256 (left only) | 384 (left only) | 14,976 |
| 64x64 based VTM2.0.1 | 2,048 | 1,536 | 768 | 1,152 | 5,504 |
| 64x64 based clean-up method 1 | 2,048 |  | 768 | 1,152 | 3,968 |
| 64x64 based clean-up method 2 | 2,048 |  | 768 | 384 | 3,200 |

Table 1: Summary of motion vector memory buffer usage for different methods

# Experimental results

Tests were carried out by using the VTM2.0.1 software, and the common test conditions defined in [1] were followed.

As shown in Table 2, the proposed clean-up (clean-up method 1) leads to the BD-rate differences are 0.01% in RA, -0.03% in LD_B and 0.02% in LD_P, respectively. Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

Table 2: Summary results of clean-up method 1

The test results for clean-up method 2 is provided in Table 3. Compared to the VTM2.0.1 anchor, the average BD-rate differences are 0.06% in RA, 0.01% in LD-B and 0.03% in LD-P, respectively. Non-negligible loss was observed in some sequences. For example, 0.23% loss in sequence Catus and 0.32% loss in sequence BQSquare of RA configuration.

Table 3: Summary results of clean-up method 2

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, the 11th JVET meeting, July 2018, Ljubljana, SI.

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001, the 11th JVET meeting, July 2018, Ljubljana, SI.

[3] M. Zhou and B. Heng, “Non-CE4: A study on the affine merge mode”, JVET-K0052, the 11th JVET meeting, July 2018, Ljubljana, SI.

[4] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001-v6, the 11th JVET meeting, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

Broadcom Inc. does not have patent rights relating to the technology described in this contribution.

# Spec Text Changes relative to JVET-K1001-v6

## Text changes for clean-up method 1 & 3 (Changes highlighted in yellow)

Section 8.3.1.1 General

- For xSbIdx = 0..( cbWidth >> 2 ) − 1 and ySbIdx = 0..( cbHeight >> 2 ) − 1, the motion vectors MvLX with X being 0 and 1 are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * 4, yCb + ySbIdx * 4 )	(8-252)

- If one or more of the following conditions are true,

- xSbIdx and ySbIdx are both equal to 0 (top-left subblock)

- xSbIdx is equal to ( cbWidth >> 2 ) − 1 and ySbIdx is equal to 0 (top-right subblock)

- numCpMv is equal to 3 and xSbIdx is equal to 0 and ySbIdx is equal to ( cbHeight >> 2 ) − 1 (top-right subblock)

the following applies for x = 0..3 and y = 0..3:

MvL0[ xSb + x ][ ySb + y ] = cpMvL0[ xSbIdx ][ ySbIdx ]	(8-253)

MvL1[ xSb + x ][ ySb + y ] = cpMvL1[ xSbIdx ][ ySbIdx ]	(8-254)

- Otherwise, the The following applies for x = 0..3 and y = 0..3:

MvL0[ xSb + x ][ ySb + y ] = mvL0[ xSbIdx ][ ySbIdx ]	(8-255)

MvL1[ xSb + x ][ ySb + y ] = mvL1[ xSbIdx ][ ySbIdx ]	(8-256)

#### Replace

#### Section 8.3.3.3 Derivation process for luma affine control point motion vectors from a neighbouring block

(Note to Ed: this replacement is needed because  the current text uses TMVPs to do affine inheritance, changes are also need for sections where 8.3.3.3 is invoked, i.e. adding numNCpMv and  cpNMvLX[ cpIdx ] to the input list)

with

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- a luma location ( xNb, yNb ) specifying the top-left sample of the neighbouring luma coding block relative to the top-left luma sample of the current picture,

- two variables nNbW and nNbH specifying the width and the height of the neighbouring luma coding block,

- the number of control point motion vectors numNCpMv of the neighbouring luma coding block,

- the luma affine control point vectors of the neighbouring luma coding block cpNMvLX[ cpIdx ] with cpIdx = 0 .. numNCpMv − 1 and X being 0 or 1,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variables log2NbW and log2NbH are derived as follows:

log2NbW = Log2( nNbW )		(8-265)

log2NbH = Log2( nNbH ) 		(8-266)

The variables mvScaleHor and mvScaleVer are derived as follows:

mvScaleHor = cpNMvLX[ 0 ][ 0 ] << 7		(8-267)

mvScaleVer = cpNMvLX[ 0 ] [ 1 ] << 7 		(8-268)

The variables dHorX and dVerX are derived as follows:

dHorX = ( cpNMvLX[ 1 ][ 0 ] − cpNMvLX[ 0 ][ 0 ] ) << ( 7 − log2NbW )	(8-269)

dVerX = ( cpNMvLX[ 1 ][ 1 ] − cpNMvLX[ 0 ][ 1 ] )  << ( 7 − log2NbW )	(8-270)

The variables dHorY and dVerY are derived as follows:

- if numNCpMv is equal to 3, the following applies:

dHorY = ( cpNMvLX[ 2 ][ 0 ]− cpNMvLX[ 0 ][ 0 ]) << ( 7 − log2NbH ) 	(8-271)

dVerY = ( cpNMvLX[ 2 ][ 0 ]− cpNMvLX[ 0 ][ 1 ] ) << ( 7 − log2NbH ) 	(8-272)

- Ohterwise (numNCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-273)

dVerY =   dHorX		(8-274)

The luma affine control point motion vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1 are derived as follows:

- The first two control point motion vectors cpMvLX[ 0 ] and cpMvLX[ 1 ] are derived as follows:

cpMvLX[ 0 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb − yNb ) )	(8-275)

cpMvLX[ 0 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb − yNb ) )	8-276)

cpMvLX[ 1 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb + cbWidth − xNb ) + dHorY * ( yCb − yNb ) )	(8-277)

cpMvLX[ 1 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb + cbWidth − xNb ) + dVerY * ( yCb − yNb ) )	(8-278)

- If numCpMv is equal to 3, the third control point vector cpMvLX[ 2 ] is derived as follows:

cpMvLX[ 2 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb + cbHeight − yNb ) )	(8-279)

cpMvLX[ 2 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb + cbHeight − yNb ) )	(8-280)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to cpMvLX[ cpIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded cpMvLX[ cpIdx ] as output, with X being 0 or 1 and cpIdx = 0 .. numCpMv − 1.

## Additional spec text changes for clean-up method 2 (Changes highlighted in yellow)

#### Section 8.3.3.7 Derivation process for motion vector arrays from affine control point motion vectors

- For xSbIdx = 0..( cbWidth >> 2 ) − 1 and ySbIdx = 0..( cbHeight >> 2 ) − 1, the following applies:

  - If xSbIdx and ySbIdx are both equal to 0 (top-left subblock), the following applies:

mvLX[ xSbIdx ][ ySbIdx ]= cpMvLX[ 0 ]                                                            (8-33x)

  - Otherwise, if xSbIdx is equal to ( cbWidth >> 2 ) − 1 and ySbIdx is equal to 0 (top-right subblock), the following applies:

mvLX[ xSbIdx ][ ySbIdx ]= cpMvLX[ 1 ]                                                            (8-33x)

–        Otherwise, if numCpMv is equal to 3 and xSbIdx is equal to 0 and ySbIdx is equal to ( cbHeight >> 2 ) − 1 (below-left subblock), the following applies:

mvLX[ xSbIdx ][ ySbIdx ]= cpMvLX[ 2 ]                                                            (8-33x)

  - Otherwise, the luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-334)

yPosCb = 2 + ( ySbIdx << 2 )		(8-335)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-336)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-337)

The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set  equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

  - The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.