| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0053-v2 |
| --- | --- |

| Title: | CE3-related: Chroma DM modification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Min Woo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | narae1.choi@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

Chroma DM is a mode to directly use the intra prediction mode of the corresponding luma block. If the separate tree for luma and chroma is used, there might be multiple luma blocks corresponding to the current chroma block. In this case, the intra prediction mode of the corresponding luma block covering the top-left pixel position of the current chroma block is directly inherited. It is important to choose an appropriate luma block among them to improve coding efficiency. Therefore, this contribution proposes to change the position from top-left to the center for finding a corresponding luma block. The simulation results reportedly show –0.1% BD-rate on AI for both VTM and BMS. In addition to this, this contribution proposes to use a single DM for MDMS instead of multiple DMs. The simulation results reportedly show –0.2% BD-rate on AI for both VTM and BMS.

Proposed Methods

Chroma DM (direct mode) is a mode to directly use the intra prediction mode of the corresponding luma block. However, if the separate tree for luma and chroma is used, there might be multiple luma blocks corresponding to the current chroma block. Currently in VVC [1], the corresponding luma block is determined to cover the top-left position of the current chroma block. In this context, it is important to choose an appropriate block among corresponding luma blocks in terms of coding efficiency. Therefore, this contribution proposes to change the position for finding a corresponding luma block in DM from top-left to center as shown in Figure 1.

Figure 1. Proposed DM position

The proposed DM position can be represented as following working draft changes. Changes are highlighted in green.

- 8.2.1 General decoding process for coding units coded in intra prediction mode … If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for chroma samples is specified as follows: - The derivation process for the chroma intra prediction mode as specified in clause 8.2.3 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input. … ### 8.2.3 Derivation process for chroma intra prediction mode Input to this process are: - a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived. The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb + ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] as specified in Table 8-2 – and Table 8-3. Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 0 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 0; 50; 18; 1; X. Table 8-3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1 intra_chroma_pred_mode[ xCb ][ yCb ]; IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ]. 0; 50; 18; 1; X ( 0 <= X <= 66 ). 0; 66; 0; 0; 0; 0. 1; 50; 66; 50; 50; 50. 2; 18; 18; 66; 18; 18. 3; 1; 1; 1; 66; 1. 4; 77; 77; 77; 77; 77. 5; 0; 50; 18; 1; X.

In addition to this, this contribution also proposes to use only one DM proposed in this proposal instead of multiple DMs (5 candidates) for MDMS (multiple DM signaling) method in JEM. From the simulation results, the single DM and multiple DMs for MDMS have similar coding performance with reduction of pruning process.

Experimental Results

The proposed scheme is implemented on top of the BMS-2.0.1 software. The first anchor is the BMS with ‘vtm’ configuration files and the second anchor is the BMS with ‘bms’ configuration files. Both the ‘All Intra’ and ‘Random Access’ test configurations were used. The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions.

Table 2-1. Results for changing DM position from top-left to center (VTM)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | -0.06% | 95% | 100% |
| Class A2 | -0.10% | 0.06% | -0.03% | 100% | 101% |
| Class B | -0.07% | 0.07% | 0.11% | 99% | 100% |
| Class C | -0.05% | -0.01% | 0.24% | 95% | 100% |
| Class E | -0.24% | 0.06% | -0.25% | 100% | 100% |
| Overall | -0.10% | 0.04% | 0.03% | 98% | 100% |
| Class D | -0.06% | 0.15% | -0.10% | 99% | 97% |
| Class F (optional) | -0.02% | -0.11% | 0.03% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.01% | 99% | 102% |
| Class A2 | -0.02% | -0.03% | -0.13% | 108% | 100% |
| Class B | 0.00% | 0.01% | -0.09% | 98% | 100% |
| Class C | 0.01% | 0.08% | 0.15% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.02% | 101% | 100% |
| Class D | 0.00% | -0.22% | 0.43% | 99% | 99% |
| Class F (optional) | -0.02% | -0.04% | -0.15% | 96% | 99% |

Table 2-2. Results for changing DM position from top-left to center (BMS)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | -0.09% | 0.10% | -0.04% | 100% | 100% |
| Class B | -0.08% | 0.01% | 0.07% | 100% | 101% |
| Class C | -0.04% | 0.04% | 0.22% | 100% | 99% |
| Class E | -0.19% | 0.03% | -0.03% | 100% | 101% |
| Overall | -0.09% | 0.03% | 0.06% | 100% | 100% |
| Class D | -0.04% | 0.01% | -0.23% | 100% | 98% |
| Class F (optional) | -0.02% | 0.02% | 0.24% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.09% | 101% | 101% |
| Class A2 | -0.03% | -0.09% | -0.10% | 100% | 101% |
| Class B | 0.00% | 0.11% | 0.02% | 99% | 100% |
| Class C | 0.00% | -0.02% | 0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.03% | 100% | 100% |
| Class D | -0.01% | 0.15% | 0.24% | 99% | 98% |
| Class F (optional) | -0.06% | -0.11% | 0.09% | 99% | 100% |

Table 2-3. Results for using one DM instead of multiple DMs for MDMS (VTM)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 w/ VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.42% | -0.52% | 95% | 108% |
| Class A2 | -0.19% | -0.72% | -0.75% | 100% | 105% |
| Class B | -0.12% | -0.85% | -0.91% | 100% | 105% |
| Class C | -0.14% | -0.70% | -0.46% | 95% | 101% |
| Class E | -0.32% | -1.53% | -1.51% | 100% | 103% |
| Overall | -0.17% | -0.84% | -0.82% | 98% | 104% |
| Class D | -0.13% | -0.39% | -0.73% | 100% | 99% |
| Class F (optional) | -0.06% | -0.63% | -0.55% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 w/ VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.27% | -0.34% | 99% | 103% |
| Class A2 | -0.04% | -0.59% | -0.48% | 99% | 101% |
| Class B | 0.03% | -0.98% | -1.17% | 98% | 102% |
| Class C | 0.04% | -0.65% | -0.83% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.67% | -0.78% | 99% | 101% |
| Class D | 0.02% | -0.76% | -0.20% | 99% | 99% |
| Class F (optional) | -0.02% | -0.34% | -0.12% | 96% | 99% |

Table2-4. Results for using one DM instead of multiple DMs for MDMS (BMS)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.56% | -0.47% | 99% | 107% |
| Class A2 | -0.20% | -0.84% | -0.76% | 100% | 104% |
| Class B | -0.14% | -0.96% | -1.06% | 100% | 104% |
| Class C | -0.12% | -0.81% | -0.74% | 104% | 101% |
| Class E | -0.32% | -1.52% | -1.24% | 100% | 103% |
| Overall | -0.17% | -0.93% | -0.87% | 101% | 104% |
| Class D | -0.10% | -0.55% | -0.75% | 100% | 100% |
| Class F (optional) | -0.05% | -0.59% | -0.39% | 100% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 with BMS cfgs |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.28% | -0.36% | 99% | 101% |
| Class A2 | -0.07% | -0.61% | -0.56% | 103% | 102% |
| Class B | 0.00% | -1.07% | -1.13% | 100% | 101% |
| Class C | -0.01% | -0.88% | -0.82% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.77% | -0.78% | 100% | 101% |
| Class D | 0.05% | -0.80% | -0.79% | 99% | 98% |
| Class F (optional) | 0.00% | -0.65% | -0.32% | 99% | 101% |

Conclusion

In this contribution, it is proposed to change the position for DM derivation process from top-left to center. The proposed method provides 0.1% bit-saving on AI for both VTM and BMS with simple change. The simulation results prove that the center position is a proper position for chroma DM derivation. Therefore, it is suggested to adopt the proposed method into next working draft of VVC.

References

[1] “Versatile Video Coding (Draft 2),” JVET-K1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).