| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0072 |
| --- | --- |

| Title: | CE11: Long deblocking filters for luma (CE11.1.1) and for both luma and chroma (CE11.1.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, Z. Zhang, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes long filters and decisions for luma in CE11.1.1 and for both luma and chroma using HEVC luma deblocking for chroma deblocking in CE11.1.9. Deblocking is applied on an 8x8 grid. Long filters for luma can be selected for boundary segments of 16 samples, symmetrically deblocking 7 or 5 samples on respective side of the block boundary for block sizes equal to or larger than 32x32 respectively 16x16. The long filters are designed to not modify a plane/ramp and to be computationally efficient to implement.

The proposed solution in CE11.1.1 is claimed to improve subjective quality at low bitrates. It also provides a luma BD rate impact of -0.01%/-0.01%/0.02%/0.02% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.03%/-0.05%/0.06%/-0.07% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 99%/100%/100%/101% for AI/RA/LD/LDP compared to VTM-2.0.1 and 99%/101%/101%/101% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

The proposed solution in CE11.1.9 is claimed to improve subjective quality at low bitrates. It also provides a luma BD rate impact of -0.01%/-0.03%/-0.05%/-0.09% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.03%/-0.07%/-0.04%/-0.15% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 103%/104%/105%/104% for AI/RA/LD/LDP compared to VTM-2.0.1 and 103%/106%/107%/105% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

# Introduction

The deblocking in the current test model is based on HEVC deblocking and is applied on an 8x8 grid. The proposed long filters and decisions for luma in CE11.1.1 and CE11.1.9 [1] were originally proposed in CfP response JVET-J0012 [2], JVET-K0307 [3], JVET-K0318 [4]. The long filters and decisions on top of the HEVC filters and decisions are described in section 2. In section 3 the use of the HEVC luma filters and decisions for chroma deblocking is described, originally proposed in JVET-K0318 [4]. In section 4 objective results in comparison to VTM-2.0.1 and VTM-2.0.1 without ALF are shown. Finally some conclusions in section 5.

# Long filters and decisions for luma

The approach is to apply a long filter from a set of long filters to symmetrically filter 5 or 7 samples on respective side of a block boundary for larger blocks and smooth regions. The filters are designed to not modify a plane/ramp and to be computationally efficient to implement.

The filtering of N samples on each side of a line of samples across the block boundary is performed as follows. First pre-filter to obtain three virtual samples, one centered at a midpoint refMiddle between current block Q and neighbouring block P, one refP centered between two samples in block P and one refQ centered between two samples in block Q. Then two tap linear interpolation between refMiddle and refP to determined filtered samples in block P, and between refMiddle and refQ to determine filtered samples in block Q. This procedure facilitates that a plan/ramp not is modified but that discontinuities between block P and Q can be reduced.

One of the longer filters is used when the strong weak filter decision (identical with HEVC/VTM/BMS) indicates strong filter for 8 lines of a 16 sample segment of a boundary to be filtered and that samples near the boundary are smooth. This gives less frequent use of long filters and also less frequent switching between long filters and weaker filters which are determined for segments of four samples. Filter decisions are shown in the Table 1. The 7 sample filter is used if the current and the neighbouring CUs have a width and height equal or greater than 32 and when 8 samples in current and neighbouring block are smooth. If that is not the case the 5 sample filter is used if the current and neighbouring CUs have a width and height equal or greater than 16 and when 6 samples in current and neighbouring block are smooth. If that is not the case filters in VTM/BMS are used.

Table 1: Filter decisions

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
| 0-3 | HEVC filter and decisions apply |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

Table 2: Filter kernels

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |

## Complexity analysis

Filtering is performed on an 8x8 grid as for the anchor. All filtering can be performed in parallel. Two cases of long filters are supported, either 7 or 5 samples are modified on each of a block boundary. At most 8 samples are read from both sides of a block boundary when 7 samples of filtering is selected which gives a line buffer requirement of 8 lines. The size of the blocks adjacent to the block boundary is at least 32x32 in the case of 7+7 sample filtering and at least 16x16 for the case of 5+5 sample filtering. Long filtering is performed on segments of 16 samples.

### Filtering

First two tap filterings and one 14 or 10 tap filtering. Then bi-linear filtering on respective side of the boundary.

7+7 sample filtering of one line requires in total 120 operations including rounding (24 multiplications, 46 additions, 22 shifts, 28 cmp).

5+5 sample filtering of one line requires in total 88 operations including rounding (16 multiplications, 34 additions, 18 shifts, 20 cmp).

Worst case: 120 operations for block size P and Q of 32x32.

### Decision

In case HEVC strong filter decision is true, long filter decisions are checked.

7+7 sample filtering decision for a line (8 of 16 lines are checked):

- Compare width and height for both current and neighbouring block with a threshold.

- Smoothness: 20 operations (12 cmp, 24 additions, 4 shifts)/2

- Total operations per line: 24

5+5 sample filtering decisions for a line (8 of 16 lines are checked):

- Compare width and height for both current and neighbouring block with a threshold.

- Smoothness: 6 operations (6 cmp, 6 additions)/2

- Total operations per line: 10

Worst case: 30 operations per line for block size P and Q of 32x32.

# Deblocking for chroma

The HEVC luma filters and decisions are used to deblock chroma with some adaptations but with same  8x8 granularity as before.

First adaptation is to filter chroma also for inter blocks when bs=1, re-using the luma decision for motion check but adding specific cbf check for respective chroma component.

Second adaptation is to limit the maximum deblocking filter length on P side (neighbouring CU) with the length of neighbouring block orthogonal to the boundary divided by 4 and for Q side (current CU) the length of the current block orthogonal to the boundary divided by 4.

lengthP = edge==EDGE_HOR ? (CUP_height_chroma >> 2) : (CUP_width_chroma >>2)

lengthQ = edge==EDGE_HOR ? (CUQ_height_chroma >> 2) : (CUQ_width_chroma >>2)

Filtering of the boundary is only applied if both sides have a maximum deblocking filter length larger than 0. The HEVC strong filter (reading 4 samples and modifying 3 samples on respective side) is only applied if both sides have a maximum deblocking filter length larger than 2.

Thirdly the decision for strong/weak filter and number of samples to filter is determined for each line instead of for a group of 4 sample segment as in HEVC. Inter blocks are only filtered if Laplacian gradients near the boundary on respective side is below the structure threshold “beta” as in HEVC but for intra blocks that check is by passed.

Fourthly is “beta” parameter divided by 8 compared to what is used for luma. This makes the use of more than 1 pixel to filter only happening when there is very little structure near the boundary.

## Complexity analysis

Filtering is performed on an 8x8 grid as for the anchor. All filtering can be performed in parallel. Same filter combinations (HEVC strong and weak filter) as for the anchor but for luma. At most 4 samples is read from both sides of a block boundary when 3 samples of filtering is selected (HEVC strong filter) which gives a line buffer requirement of 4 lines. The size of the blocks adjacent to the block boundary is at least 4xN in the case of 1 sample weak filtering, 8xN in the case of 2 sample weak filtering and at least 16xN for the case of 3 sample strong filtering. Filtering is performed for each line of chroma samples.

### Filtering

3 sample filtering: HEVC strong filter: 52 operations (2 multiplications, 28 additions, 10 shifts, 12 cmp)

1 or 2 simple filtering: HEVC weak filter: 33 operations (2 multiplications, 16 additions, 5 shifts, 8 cmp)

Worst case: 52 operations per line for block size P 16xN and Q 16xN

### Decisions

HEVC d decision: 4 operations (5 additions, 3 cmp)/2

HEVC strong weak decision: 10 operations (4 additions, 6 cmp)

HEVC weak decision: 4 operations (4 cmp)

Worst case: 18 operations per line for block size P 16xN and Q 16xN

# Results

The simulations were performed using the CTC with the inclusion of LDP. We also tested the performance with ALF disabled. The decoding times for the anchor and test configurations were measured using the same machines, but encoding times were measured on different machines which make the encoding times unreliable. However, since the decoding times are magnitudes shorter and the deblocking filter is done once per picture for both encoder and decoder, we conclude the encoding time difference is negligible.

The results indicate that that the objective performance of the proposed long filters for luma as in CE11.1.1 is similar to the anchor. It has slightly better objective performance when ALF is disabled than when it is enabled.  Encoding time is about 100% and decoding time is about 101% compared to the anchor.

For the case of both long luma filters and longer chroma filters as in CE11.1.9 the objective performance is slightly better than the anchor. Also here the objective performance is slightly better when ALF is disabled than when it is enabled. Encoding time is about 100% and decoding time 106% compared to the anchor.

## Objective performance for CE11.1.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.00% | 106% | 98% | -0.09% | -0.02% | 0.00% | 103% | 98% |
| Class A2 | -0.01% | 0.00% | 0.00% | 108% | 99% | -0.01% | -0.01% | 0.00% | 94% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 108% | 98% | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 107% | 101% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | -0.06% | -0.01% | 0.00% | 98% | 100% | -0.06% | -0.01% | 0.00% | 98% | 100% |
| Overall | -0.01% | 0.00% | 0.00% | 106% | 99% | -0.03% | -0.01% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 101% | 0.00% | 0.00% | 0.00% | 104% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.01% | 0.03% | 99% | 100% | -0.24% | -0.01% | 0.02% | 99% | 104% |
| Class A2 | 0.00% | -0.10% | -0.08% | 99% | 101% | -0.04% | 0.05% | 0.04% | 101% | 98% |
| Class B | 0.03% | -0.01% | -0.09% | 97% | 100% | -0.01% | 0.04% | 0.00% | 100% | 102% |
| Class C | 0.02% | 0.06% | -0.03% | 100% | 102% | 0.02% | 0.02% | -0.07% | 101% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.05% | 99% | 100% | -0.05% | 0.03% | -0.01% | 100% | 101% |
| Class D | 0.03% | -0.04% | 0.01% | 97% | 100% | -0.01% | 0.00% | 0.08% | 99% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% | -0.17% | 0.00% | 103% | 100% | 0.06% | 0.09% | -0.10% | 102% | 101% |
| Class C | 0.03% | -0.13% | -0.10% | 102% | 100% | 0.01% | -0.15% | -0.13% | 100% | 102% |
| Class E | -0.07% | 0.37% | -0.07% | 95% | 102% | 0.14% | -0.09% | 0.29% | 96% | 102% |
| Overall | 0.02% | -0.02% | -0.05% | 101% | 100% | 0.06% | -0.04% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.03% | -0.49% | 104% | 100% | -0.03% | -0.48% | -0.06% | 103% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.17% | 0.18% | 102% | 100% | -0.06% | -0.03% | -0.09% | 101% | 100% |
| Class C | 0.02% | 0.02% | 0.03% | 100% | 101% | 0.03% | 0.12% | 0.00% | 101% | 100% |
| Class E | 0.03% | 0.00% | 0.60% | 107% | 101% | -0.22% | 0.23% | -0.20% | 100% | 102% |
| Overall | 0.02% | 0.08% | 0.24% | 102% | 101% | -0.07% | 0.08% | -0.09% | 101% | 101% |
| Class D | 0.04% | 0.05% | -0.32% | 103% | 101% | -0.01% | -0.40% | -0.18% | 102% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Objective performance for CE11.1.9

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.90% | -0.93% | 101% | 105% | -0.09% | -1.36% | -1.24% | 104% | 103% |
| Class A2 | 0.00% | -0.41% | -0.32% | 104% | 103% | 0.00% | -0.32% | -0.18% | 97% | 103% |
| Class B | 0.00% | -0.72% | -0.80% | 105% | 103% | -0.01% | -0.66% | -0.69% | 95% | 103% |
| Class C | 0.00% | -0.45% | -0.46% | 102% | 102% | 0.00% | -0.20% | -0.18% | 102% | 102% |
| Class E | -0.05% | -0.68% | -0.53% | 100% | 104% | -0.06% | -0.78% | -0.65% | 97% | 104% |
| Overall | -0.01% | -0.63% | -0.62% | 103% | 103% | -0.03% | -0.64% | -0.58% | 99% | 103% |
| Class D | 0.00% | -0.42% | -0.50% | 106% | 101% | 0.00% | -0.10% | -0.09% | 97% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -2.41% | -3.28% | 98% | 103% | -0.27% | -3.46% | -4.09% | 99% | 108% |
| Class A2 | -0.03% | -1.62% | -1.47% | 99% | 105% | -0.06% | -1.63% | -1.35% | 101% | 105% |
| Class B | 0.01% | -1.97% | -2.18% | 97% | 104% | -0.02% | -2.10% | -2.32% | 99% | 106% |
| Class C | -0.01% | -1.38% | -1.55% | 99% | 105% | 0.01% | -1.79% | -1.96% | 99% | 105% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% | -1.83% | -2.09% | 98% | 104% | -0.07% | -2.20% | -2.39% | 99% | 106% |
| Class D | 0.00% | -1.21% | -1.25% | 99% | 105% | 0.02% | -1.63% | -1.69% | 95% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -1.72% | -1.98% | 101% | 104% | -0.01% | -1.66% | -2.24% | 103% | 107% |
| Class C | -0.06% | -1.46% | -1.57% | 105% | 105% | -0.07% | -1.29% | -1.44% | 100% | 107% |
| Class E | -0.13% | -3.09% | -3.03% | 95% | 107% | -0.04% | -2.84% | -2.45% | 95% | 108% |
| Overall | -0.05% | -1.98% | -2.10% | 101% | 105% | -0.04% | -1.83% | -2.03% | 100% | 107% |
| Class D | -0.01% | -0.34% | -0.82% | 100% | 104% | -0.06% | -0.80% | -0.84% | 102% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | -2.22% | -2.16% | 101% | 103% | -0.15% | -2.65% | -2.98% | 103% | 106% |
| Class C | -0.07% | -1.34% | -1.38% | 99% | 105% | -0.06% | -1.17% | -1.49% | 100% | 105% |
| Class E | -0.09% | -3.15% | -2.47% | 101% | 104% | -0.26% | -3.28% | -3.57% | 96% | 103% |
| Overall | -0.09% | -2.16% | -1.98% | 100% | 104% | -0.15% | -2.32% | -2.63% | 100% | 105% |
| Class D | -0.01% | -1.03% | -1.26% | 101% | 104% | -0.08% | -1.71% | -1.32% | 102% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

We propose to adopt the proposed long filters for luma to combat very visual artifacts for large blocks. We also propose to use HEVC luma deblocking for chroma deblocking but that mainly improves subjective performance on the Campfire sequence.

# References

[1] A. Norkin, A. Meher Kotra, “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, Ljubljana, July 2018

[2] R. Sjöberg, K. Andersson, R. Yu, Z. Zhang, P. Wennersten, J. Lainema, A. Hallapuro, A. Aminlou, M. Hannuksela, R. Ghaznavi-Youvalari, J. Ridge, “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia”, JVET-J0012, San Diego, April 2018

[3] K. Andersson, Z. Zhang, R. Sjöberg, “CE2-2.1.1: Long deblocking filters and fixes”, JVET-K0307, Ljubljana, July 2018

[4] K. Andersson, Z. Zhang, R. Sjöberg, “CE2-2.1.1 related: HEVC luma filters and decisions for chroma deblocking”, JVET-K0318, Ljubljana, July 2018

[5] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, July 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).