| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0074 |
| --- | --- |

| Title: | CE11: Deblocking of sub-block boundaries for luma (CE11.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, Z. Zhang, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

ATMVP in VVC can result in sub-blocks that use different motion vectors and different reference pictures for motion compensation. Currently, no deblocking filter is applied on ATMVP sub-block boundaries. This contribution proposes to apply the current VVC deblocking filters and filter decisions as-is on ATMVP sub-block boundaries, including only applying deblocking on an 8x8 grid.

The proposed solution is claimed to improve subjective quality especially noticeable in large CUs that uses ATMVP. It also provides a luma BD rate impact of 0.00%/-0.04%/-0.07%/-0.02% for AI/RA/LD/LDP compared to VTM-2.0.1 and 0.00%/-0.07%/-0.07%/-0.10% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 100%/100%/100%/100% for AI/RA/LD/LDP compared to VTM-2.0.1 and 100%/99%/100%/100% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

# Introduction

The deblocking filter in the current test model is based on HEVC deblocking and is applied on an 8x8 grid. The proposed solution also applies deblocking of prediction boundaries from ATMVP on an 8x8 grid as described in section 2. In section 3, objective results in comparison to VTM-2.0.1 and VTM-2.0.1 without ALF are shown. Finally, some conclusions in section 4.

# Deblocking of luma sub-block boundaries

ATMVP can determine individual motion vectors and reference pictures on an 4x4 and 8x8 grid. This can cause boundary artifacts inside CUs that uses ATMVP. Since VTM deblock prediction boundaries that are aligned with CU boundaries on an 8x8 grid, it makes sense to do the same for sub-block prediction boundaries from ATMVP. Therefore, it is proposed to deblock sub-block boundaries for CUs that use ATMVP and do that on an 8x8 grid.

## Complexity analysis

The worst case complexity is the same as in the current VTM since any ATMVP sub-block partition can be expressed directly. As said, the 8x8 grid is kept as-is in this contribution.

### Filtering

We propose to use the same deblocking filter as in VTM for ATMVP sub-block boundaries.

### Decision

We propose to use the same deblocking filter decisions as in VTM for ATMVP sub-block boundaries.

# Results

## Visual example of artifacts from not deblocking sub-blocks

## Objective results

The simulations were performed using the CTC with the inclusion of LDP. We also tested the performance with ALF disabled. The decoding times for the anchor and test configurations were measured using the same machines, but encoding times were measured on different machines which make the encoding times unreliable. However, since the decoding times are magnitudes shorter and the deblocking filter is done once per picture for both encoder and decoder, we conclude the encoding time difference is negligible.

The results indicate that that the objective performance of the proposed sub-block filtering is slightly better than the anchor. The proposal has slightly better objective performance when ALF is disabled than when ALF is enabled.  The encoding time is about 100% and decoding time is about 100% compared to the anchor.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 100% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 106% | 99% | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 99% | 0.00% | 0.00% | 0.00% | 96% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.02% | -0.01% | 97% | 101% | -0.09% | 0.02% | 0.02% | 99% | 101% |
| Class A2 | -0.04% | -0.03% | 0.00% | 98% | 96% | -0.07% | 0.03% | -0.01% | 101% | 97% |
| Class B | -0.03% | 0.00% | 0.03% | 97% | 101% | -0.06% | -0.01% | 0.02% | 100% | 99% |
| Class C | -0.04% | 0.05% | -0.02% | 100% | 101% | -0.04% | -0.01% | 0.01% | 97% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | 0.01% | 0.00% | 98% | 100% | -0.07% | 0.00% | 0.01% | 99% | 99% |
| Class D | -0.01% | 0.04% | 0.02% | 100% | 101% | -0.01% | 0.02% | 0.01% | 98% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.14% | 100% | 100% | -0.05% | 0.12% | -0.07% | 100% | 99% |
| Class C | -0.01% | -0.09% | -0.18% | 101% | 99% | -0.06% | -0.24% | -0.21% | 105% | 101% |
| Class E | -0.24% | -0.17% | 0.22% | 92% | 101% | -0.12% | 0.03% | 0.17% | 94% | 100% |
| Overall | -0.07% | -0.07% | 0.05% | 98% | 100% | -0.07% | -0.02% | -0.06% | 100% | 100% |
| Class D | 0.00% | 0.31% | 0.06% | 99% | 102% | 0.02% | 0.01% | 0.44% | 103% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.11% | 0.18% | 97% | 99% | -0.05% | -0.14% | -0.19% | 101% | 99% |
| Class C | -0.03% | -0.10% | -0.04% | 99% | 100% | -0.07% | 0.15% | -0.10% | 100% | 99% |
| Class E | 0.01% | 0.05% | 0.01% | 99% | 101% | -0.22% | 0.20% | 0.36% | 96% | 100% |
| Overall | -0.02% | 0.03% | 0.07% | 98% | 100% | -0.10% | 0.04% | -0.02% | 99% | 100% |
| Class D | 0.08% | 0.16% | -0.49% | 99% | 99% | -0.05% | -0.28% | -0.42% | 101% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

We propose to adopt deblocking of ATMVP sub-block boundaries using the current VVC deblocking filter.

# References

[1] A. Norkin, A. Meher Kotra, “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, Ljubljana, July 2018

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, July 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).