| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0075_v2 |
| --- | --- |

| Title: | CE13: Hybrid Equiangular Cubemap with Pre-rotation (Test 6.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akhil Konda Chirag Pujara Anubhav Singh | Email: | akhil.konda@samsung.com chirag.p@samsung.com anubhav.23@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 6.2 Hybrid Equiangular Cubemap (HEC) implementation over 360Lib7.0 merged with BMS2.0.1 with VTM configuration has been tested. The results are reported for base HEC code (CE 13.1.1a) and base with pre-rotation.  Results show that HEC’s Luma component BD-Rate gains approximately -0.72% for 8k sequences  and looses around 0.5 % for 6k sequences over base HEC, under 360 Common Test Condition (CTC) for E2EWSPSNR. Loss in 6k sequences are majorly due to Landing2 and BranCastle2 sequences because of the high camera motion in the sequence. Maximum gain is observed for Gaslamp sequence which is around -2.9%.

# Introduction

The simulations were performed under 360 video CTC [JVET-K1012] using the code uploaded at https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS/tree/CE13.6.2. The simulations were run with HEC implementation over 360Lib7.0 merged with BMS2.0.1 software with VTM configuration. VTM results are generated using encoder_randomaccess_vtm.cfg file.

The tests performed are as follows:

- 6.2 HEC with pre-rotation: Rotation is implemented over base HEC (13.1.1a). An algorithm to compute best rotation angle is implemented as described in CE13 document [JVET-K1033] and also in the CfP document [JVET-J0024/0025] and the results are compiled for VTM.

# Analysis

(a)                                                                             (b)

(c)                                                                                           (d)

Figure 1. Frame from Gaslamp sequence and its edge image. (a) image with no rotation, (b) its edge image, (c) image with optimal rotation (140 degrees), (d) its edge image

- (b)

(c)                                                                              (d)

Figure 2. Frame from Landing sequence and its edge image. (a) Image with no rotation, (b) its edge image, (c) image with optimal rotation (170 degrees), (d) its edge image

From the image 1, 2 it can be seen that the edges in the image are aligned to horizontal and vertical directions in case of optimal rotation angle which improves the coding gain.

- (b)

(c)                                                                                        (d)

Figure 3. BD-rate and Horizontal + vertical edge percentage w.r.t. rotation from 0 to 360 degrees, (a) averaged over full  Landing2 sequence, (b) averaged over first RAP of Landing2 sequence, (c) averaged over full sequence of Broadway and 9(d) averaged over first RAP of Broadway

As per our analysis for the loss in case of BD-rate gain for the Landing2 sequence is due to very high camera motion with in the sequence. It can be seen from image 2 that the BD-rate curve is not symmetric over rotation (which is the case for other sequences like Balboa, Broadway in 6k category). This makes the algorithm less appropriate for this kind of sequence. For Landing2 kind of sequences we may have to run this algorithm for each GoP or can arrive at the decision of not to apply rotation based on the motion analysis.

# Experimental Results

Snapshot of experimental results is as follows,

- 6.2 HEC + Pre-rotation:

# Observations

Rotation provides gain of around 0.7% for 8k sequences and we observed a loss of 0.5% for 6k sequences. 6k sequences are found to be having more abrupt motion specifically landing2 and brancastle2 sequences. Highest gain is observed for Gaslamp sequence around 2.9% for the Luma component.

# Cross check

The cross-check for CE13-6.2 has been performed by Philippe Hanhart from InterDigital.

# References

[1] Philippe Hanhart, Jian-Liang Lin, Chirag Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” JVET-K1033, July 2018.

[2] P. Hanhart, J. Boyce, K. Choi, “Common test conditions and evaluation procedures for 360° video,” JVET-K1012, July 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).