| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0078 |
| --- | --- |

| Title: | AHG11: Block DPCM for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli Gordon Clare Félix Henry Pierrick Philippe | Tel: Email: | +33299124558 mohsen.abdoli @orange.com gordon.clare @orange.com felix.henry@orange.com pierrick.philippe@orange.com |
| Source: | Orange |  |  |

_____________________________

# Abstract

This contribution proposes to use DPCM at the block level to encode Screen Content. The proposed Block-DPCM uses the MED predictor of the JPEG-LS (LOCO-I) algorithm. The prediction error is quantized in the spatial domain and encoded using unary binarization of amplitude and bypass coding of signs. This contribution reports the following compression performance changes for the different tested categories (a negative value indicating an overall gain for Block-DPCM):

Class A to E, All Intra: -0.1% (EncT 103%, DecT 99%)

Class F, All Intra: -7.0% (EncT 103%, DecT 89%)

TGM, All Intra: -21.0% (EncT 105%, DecT 73%)

Class A to E, Random Access: 0.0% (EncT 100%, DecT 100%)

Class F, Random Access: -5.1% (EncT 102%, DecT 96%)

TGM, Random Access: -9.8% (EncT 102%, DecT 92%)

# Introduction

As reported in the meeting notes of the JVET K meeting [1], participants expressed the need for coding tools that can compress Screen Content while avoiding the complexity of Current Picture Referencing approaches. This is what the current proposal is attempting to address.

The current contribution proposes to use a classical DPCM approach at the block level [3].

# Signalization

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller or equal to 32. This flag indicates whether regular intra coding or DPCM is used. This flag is encoded using a single CABAC context.

# Prediction

The proposed Block DPCM uses the Median Edge Detector of LOCO-I [2]. For a current pixel X having pixel A as left neighbor, pixel B as top neighbor and C as top-left neighbor, the prediction P(X) is determined by

P(X)=	min(A,B) if C≥max(A,B)

max(A,B) if C≤min(A,B)

A+B-C otherwise

The predictor uses unfiltered reference pixels when predicting the top row and left column of the CU. The predictor then uses reconstructed pixels for the rest of the CU. Pixels are processed in raster-scan order inside the CU.

# Quantization

The prediction error is quantized in the spatial domain, after rescaling, in a way identical to the Transform Skip quantizer.

# Pixel reconstruction

Each pixel is reconstructed by adding the dequantized prediction error to the prediction. Thus, the reconstructed pixels are used to predict the next pixels in raster-scan order.

# Coefficient coding

Amplitude and signs of the quantized prediction error are encoded separately.

## Amplitude coding

A cbf_bdpcm_flag is coded. If it is equal to 0, it indicates that all amplitudes of the block are to be decoded as zero. If the flag is equal to 1, all amplitudes of the block are encoded individually in raster-scan order. In order to keep complexity low, the amplitude is limited to at most 31 (inclusive). The amplitude is encoded using unary binarization, with three contexts for the first bit, then one context for each additional bin until the 12th bin, and one context for all remaining bins.

## Sign coding

A sign is encoded in bypass mode for each zero residue.

# Other aspects

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

Block-DPCM does not use any other step than the ones described here. In particular it does not use any transform.

# Performance

In the encoder implementation of this contribution, no RDOQ-type search is performed when encoding a Block-DPCM CU. Only nearest-neighbor quantization is applied.

This contribution reports the compression performance in the tables below (table 1 and table 2). Since Block-DPCM is essentially a Screen Content encoding tool, we report the performance for AI and RA only (using CTC of Screen Content Coding related Core Experiments).

Table 1: Performance of Block-DPCM in VTM2, All Intra

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| Class A1 | Tango2 | 0,10% | 103% | 98% |
|  | FoodMarket4 | 0,04% | 104% | 97% |
|  | Campfire | -0,20% | 103% | 99% |
| Class A2 | CatRobot1 | 0,02% | 103% | 99% |
|  | DaylightRoad2 | 0,15% | 103% | 100% |
|  | ParkRunning3 | 0,05% | 103% | 99% |
| Class B | MarketPlace | 0,03% | 103% | 99% |
|  | RitualDance | 0,00% | 103% | 99% |
|  | Cactus | -0,02% | 103% | 100% |
|  | BasketballDrive | -0,07% | 103% | 100% |
|  | BQTerrace | -0,17% | 103% | 101% |
| Class C | BasketballDrill | -0,16% | 103% | 101% |
|  | BQMall | -0,57% | 103% | 100% |
|  | PartyScene | -0,55% | 103% | 100% |
|  | RaceHorses | -0,11% | 103% | 101% |
| Class D | BasketballPass | -0,30% | 102% | 100% |
|  | BQSquare | -1,10% | 102% | 98% |
|  | BlowingBubbles | -0,27% | 103% | 100% |
|  | RaceHorses | -0,17% | 103% | 100% |
| ClassE | FourPeople | -0,11% | 103% | 100% |
|  | Johnny | -0,07% | 103% | 100% |
|  | KristenAndSara | -0,48% | 102% | 99% |
| ClassF | BasketballDrillText | -1,89% | 103% | 98% |
|  | ArenaOfValor | -1,22% | 103% | 100% |
|  | SlideEditing | -15,60% | 107% | 69% |
|  | SlideShow | -9,14% | 100% | 93% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -8,62% | 107% | 86% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | -26,25% | 107% | 63% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -33,37% | 99% | 74% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -15,70% | 109% | 72% |

Table 2: Performance of Block-DPCM in VTM2, Random Access

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| Class A1 | Tango2 | -0,03% | 100% | 100% |
|  | FoodMarket4 | 0,03% | 100% | 100% |
|  | Campfire | -0,02% | 101% | 100% |
| Class A2 | CatRobot1 | 0,01% | 100% | 100% |
|  | DaylightRoad2 | 0,05% | 100% | 100% |
|  | ParkRunning3 | 0,02% | 101% | 100% |
| Class B | MarketPlace | 0,04% | 101% | 100% |
|  | RitualDance | 0,05% | 100% | 100% |
|  | Cactus | 0,03% | 101% | 100% |
|  | BasketballDrive | 0,03% | 101% | 100% |
|  | BQTerrace | -0,12% | 101% | 100% |
| Class C | BasketballDrill | -0,01% | 101% | 100% |
|  | BQMall | -0,14% | 101% | 100% |
|  | PartyScene | -0,17% | 101% | 100% |
|  | RaceHorses | 0,10% | 101% | 100% |
| Class D | BasketballPass | 0,00% | 101% | 100% |
|  | BQSquare | -0,29% | 101% | 100% |
|  | BlowingBubbles | -0,09% | 101% | 100% |
|  | RaceHorses | 0,09% | 101% | 100% |
| ClassE | FourPeople | - | - | - |
|  | Johnny | - | - | - |
|  | KristenAndSara | - | - | - |
| ClassF | BasketballDrillText | -0,61% | 101% | 100% |
|  | ArenaOfValor | -0,36% | 101% | 100% |
|  | SlideEditing | -12,79% | 104% | 89% |
|  | SlideShow | -6,48% | 101% | 97% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -2,37% | 101% | 98% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | -15,56% | 103% | 90% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -10,05% | 101% | 93% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -11,16% | 105% | 89% |

# Additional results: combination with Palette

In this section additional results are provided showing the performance of Block-DPCM on top of Palette (in BMS2.0.1). The results are presented for Class F and TGM only, in All Intra and Random Access configurations.

Table 3 has been updated compared to the initial version of this contribution as it was noticed that a useless palette flag was written when BDPCM was used to encode a block. The fixed source code has been crosschecked in [4].

Table 3: Reference Palette, Test Palette+Block-DPCM, All Intra

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| ClassF | BasketballDrillText | -0,65% | 101% | 101% |
|  | ArenaOfValor | -0,99% | 101% | 100% |
|  | SlideEditing | -1,49% | 98% | 98% |
|  | SlideShow | -1,00% | 99% | 101% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -1,99% | 98% | 101% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | -0,91% | 98% | 100% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -1.08% | 100% | 101% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -1,31% | 99% | 101% |

Table 4: Reference Palette, Test Palette+Block-DPCM, Random Access

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| ClassF | BasketballDrillText | -0,15% | 101% | 100% |
|  | ArenaOfValor | -0,21% | 101% | 100% |
|  | SlideEditing | -2,08% | 100% | 100% |
|  | SlideShow | -1,14% | 100% | 100% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -0,88% | 99% | 100% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | -0,79% | 100% | 100% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -0,94% | 99% | 100% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -1,87% | 99% | 100% |

# Additional results: combination with IBC

In this section additional results are provided showing the performance of Block-DPCM on top of Intra Block Copy (in BMS2.1). The results are presented for Class F and TGM only, in All Intra and Random Access configurations.

Table 5: Reference IBC, Test IBC+Block DPCM, All Intra

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| ClassF | BasketballDrillText | -0,53% | 101% | 99% |
|  | ArenaOfValor | -0,39% | 102% | 100% |
|  | SlideEditing | -2,40% | 99% | 94% |
|  | SlideShow | -1,86% | 99% | 97% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -0,10% | 101% | 98% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | -1,15% | 99% | 93% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -0,55% | 101% | 96% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -1,55% | 98% | 93% |

Table 6: Reference IBC, Test IBC+Block DPCM, Random Access

|  | Sequence | BDR change | Average EncT | Average DecT |
| --- | --- | --- | --- | --- |
| ClassF | BasketballDrillText | -0,09% | 101% | 100% |
|  | ArenaOfValor | -0,10% | 101% | 100% |
|  | SlideEditing | -1,86% | 100% | 99% |
|  | SlideShow | -1,27% | 101% | 100% |
| TGM | ai-sc_flyingGraphics_1920x1080_60_8bit_420 | -0,45% | 100% | 99% |
|  | ai-sc_desktop_1920x1080_60_8bit_420 | 0,28% | 99% | 98% |
|  | ai-sc_console_1920x1080_60_8bit_420 | -0,24% | 99% | 97% |
|  | ai-ChineseEditing_1920x1080_60_8bit_420 | -0,91% | 100% | 96% |

# Conclusion

The proponents suggest that Block-DPCM is adopted in the standard draft text as it provides substantial gains for Screen Content Coding, with a positive impact on decoding runtime and very little impact on encoder runtime.

The proponents would like to thank Mediatek for crosschecking their proposal in document JVET-L0481 [4].

# Draft text

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( pred_mode_flag = = MODE_INTRA && ( cIdx == 0 ) && ( log2CbWidth <= 5 ) && ( log2CbHeight <= 5 )) { |  |
| bdpcm_flag | ae(v) |
| if( bdpcm_flag ) { |  |
| bdpcm_cbf | ae(v) |
| } |  |
| if( bdpcm_cbf ) { |  |
| bdpcm_residual_coding( x0, y0, cbWidth, cbHeight) |  |
| } |  |
| else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] |  |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| [Ed. (BB): Inter prediction yet to be added, pending further specification development.] |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf && !bdpcm_flag) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

| bdpcm_residual_coding ( x0, y0, log2CbWidth, log2CbHeight) { | Descriptor |
| --- | --- |
| for( i = 0; i < ( 1<<log2CbWidth )* ( 1<<log2CbHeight ); i++ ) { |  |
| bdpcm_gt_flag[0][ i ] | ae(v) |
| for( l = 1; l < 32 && bdpcm_gt_flag[l-1][ i ]; l++ ) { |  |
| bdpcm_gt_flag[l][ i ] | ae(v) |
| } |  |
| } |  |
| for( i = 0; i < ( 1<<log2CbWidth )* ( 1<<log2CbHeight ); i++ ) { |  |
| if( bdpcm_gt_flag[0][ i ] ) |  |
| bdpcm_sign_flag[ i ] | u(1) |
| } |  |
| } |  |

# Bibliography

[1] https://www.itu.int/wftp3/av-arch/jvet-site/2018_07_K_Ljubljana/JVET-K_Notes_dC.docx

[2] Marcelo J. Weinberger, Gadiel Seroussi, and Guillermo Sapiro, “The LOCO-I Lossless Image Compression Algorithm: Principles and Standardization into JPEG-LS”, IEEE Transactions on Image Processing, vol. 9, no. 8, august 2000

[3] Mohsen Abdoli, Félix Henry, Patrice Brault, Pierre Duhamel, and Frédéric Dufaux. Short Distance Intra Prediction of Screen Content in Versatile Video Coding (VVC), to appear in IEEE Signal Processing Letters, DOI : 10.1109/LSP.2018.2871872, 2018

[4] JVET-L0481, Crosscheck of JVET-L0078 (AHG11: Block DPCM for Screen Content Coding), Mediatek

# Patent rights declaration(s)

Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).