| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0079_r1 |
| --- | --- |

| Title: | AHG14: Study of methods for progressive intra refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution reports possible methods for progressive (or gradual) intra refresh, according to the mandates of AHG14 “Low-latency random access”. Firstly, essential restrictions on encoding and possible non-normative methods on the VTM2.0 and BMS2.0 tools for perfect reconstruction (i.e. exact match) at a recovery point picture are studied. Then one of possible normative methods to theoretically mitigate the loss of coding efficiency caused by the restrictions on encoding is suggested.

Introduction

Ultra low-latency coding with gradual intra refresh

Some real-time applications such as bi-directional audio-visual communication and remote operation require very low latency video coding since the user experience of those applications is significantly influenced by the amount of delay. For example, ITU-T G.114 specifies that the acceptable upper limit of delay is 150 milliseconds for bi-directional audio communication (and synchronized video communication). For virtual reality applications, ultra low latency less than 20 milliseconds is required in order to avoid motion sickness caused by timing discrepancy between head movement and visual effect caused by the movement.

An encoder-decoder delay time is the summation of the transmission time of the first coded picture and the reordering time of reconstructed pictures at the decoder. Generally, picture reordering is not utilized in low-delay coding since it introduces the delay time of several pictures. Thus, the transmission time of a coded picture mainly contributes to the delay time. The transmission time of the first coded picture is generally equal to the capacity of the coded picture buffer (CPB) divided by bitrate. As of the constraints on HRD operation, the size of any coded picture shall be less than the size of CPB to avoid CPB overflow.

Random access by inserting an I-picture generally increase the encoder-decoder delay time of 200ms to 500ms since the size of an I-picture is generally set larger than that of an B-picture or P-picture to maximize coding efficiency. Thus, the use of an I-picture is not suitable for random access in ultra-low delay coding.

The progressive (or gradual) intra refresh scheme, which is commonly used in (ultra-) low delay coding, refreshes pictures progressively by inserting intra-coded regions in B/P-pictures so to diffuse refreshed regions inside the pictures. By dispersing intra-coded regions in each non-intra picture, the size of each coded picture is unified. Consequently, the size of the CPB can be reduced to the minimal value (i.e. the bitrate divided by the frame rate), and the encoder-decoder delay time can be reduced to one picture interval.

The HEVC standard further introduces the signalling and HRD operation of a decoding unit (DU) which contains one or more slices so to realize interoperable ultra-low delay coding less than one picture interval. For example, when each DU contains one CTU line and the same number of intra CUs, then the size of the CPB for DU-based HDR operation can be reduced to the averaged size of a DU (i.e. the bitrate divided by the number of DU per second) [1][2].

Correct reconstruction of recovery point　picture

Generally, real-time applications require the functionality of random access in which perfect reconstruction (i.e. exact match) of coded pictures is possible even when the decoding process starts from a random access picture.

The AVC and HEVC standards specify a recovery point SEI message for the support of random access in decoding. A picture associated with a recovery point SEI message is called a random access picture. A picture pointed by the syntax element recovery_poc_cnt of the recovery point SEI message is called a recovery point picture. When the syntax element exact_match_flag of the SEI message is equal to 1, the bitstream shall be encoded so that the decoding result of any recovery point picture when the decoding process starts from the previous random access picture is guaranteed to be the same as that from the previous IDR picture. Figure 1 depicts the relationship of a random access picture and a recovery point picture

Figure 1: Definition of random access picture and recovery point picture

In the HEVC encoding with an IRAP picture, perfect reconstruction of all subsequent pictures of the IRAP picture in display order is guaranteed.

On the other hand, in the encoding with gradual intra refresh, several block-level restrictions on encoding are necessary for the perfect reconstruction of the recovery point picture. For example, any inter-coded block in the recovery point picture shall refer to samples in its reference pictures whose values are guaranteed to be correct in random access.

Restrictions on encoding with gradual intra refresh are applied to all coding tools which perform temporal prediction and spatial prediction to a temporally predicting block.

Figure 2 depicts one example of non-perfect reconstruction at the recovery point picture due to incomplete restriction on encode.

Figure 2: Example of non-perfect reconstruction at recovery point picture

Methods for correct reconstruction of recovery point picture in gradual intra refresh

Definition

In the following discussion, the concepts of clean region, dirty region and clean region boundary are introduced, similarly as in [1][2]. Figure 3 depicts the movement of a clean region boundary among consequent pictures from right to left.

Figure 3: Clean region, dirty region and clean region boundary

A region is called a clean region if the decoding result of coding parameters and sample values of any CU in the region are guaranteed to be correct.

A region is called a dirty region if the decoding result of coding parameters and sample values of any CU in the region are not guaranteed to be correct when the decoding process starts from the immediately preceding random access picture.

A clean region boundary is defined as a boundary line which lies between a clean region and a dirty region. Generally, the position of a clean region boundary aligns to the MxN grid, where M and N are the minimum width and height of a CU.

When the gradual intra refresh scheme is applied, the encoding process is controlled so that the area covered by the clean region expands gradually from the random access picture to the following picture, and the whole picture area is filled by the clean region at the recovery point picture. Generally, CUs inside the clean region and adjacent to the clean region boundary are intra-coded.

List of tools subject to restrictions

When the decoding process starts from a random access picture, any CU which temporally refers to a CU inside the picture preceding the random access picture in the decoding order cannot be correctly decoded since it loses its reference in random access. Similarly, any CU which spatially or temporally refers to a CU which is not correctly decoded cannot be correctly decode. Therefore, any CU in the clean region shall be guaranteed not to refer to any CU in the dirty region. On the other hand, there is no need to restrict the reference from a CU in the dirty region. By imposing no restriction to the dirty region, overall coding efficiency becomes better than that with restriction.

Table 1 summarizes the coding tools in which temporal and/or spatial reference from the clean region to the dirty region shall be prohibited to guarantee the correct decoding of a CU in the clean region.

Table 1: List of tools subject to restrictions on temporal/spatial prediction

| Tool | Restriction | Note |
| --- | --- | --- |
| Advanced temporal MV prediction (ATMVP) | Any CU in the clean region shall not temporally refer to the MV of its co-located CU unless the correct decoding of the MV is guaranteed. | Since each entry of the MV candidate list may be affected by the value of the temporal MV predictor, restriction on index selection does not work perfectly. |
| Intra prediction | Any CU in the clean region shall not spatially refer to the sample values of its neighboring CU if it is inside the dirty region. | Any CU in the clean region shall be decoded so that the result of sample reconstruction before in-loop filtering is guaranteed to be correct |
| Intra block copy | Any CU in the clean region shall not spatially refer to the sample values of its surrounding CU if it is inside the dirty region. | Same as above. |
| Inter prediction | Any CU in the clean region shall not temporally refer to the sample values of a CU in the dirty region of its reference picture | Same as above. |
| In-loop filter (SAO, Deblock, ALF) | Any CU in the clean region shall not spatially refer to the sample values of its neighboring CU in the dirty region. | Any CU in the clean region shall be decoded correctly. |

Non-normative methods to satisfy the restrictions

In the following, possible methods to satisfy the restrictions shown in Table 1are enumerated.

Note: Some methods such as disabling its functionality at picture level are omitted from this study since those introduces significant amount of loss in coding efficiency (e.g. 2%–5% in BD-Rate).

Advanced temporal MV prediction (ATMVP)

One possible method to minimize the loss of coding efficiency while guaranteeing the correct MV reconstruction is to disable ATMVP at pictures where error in temporal MV prediction may occur due to missing reference pictures.

Specifically, the syntax element slice_temporal_mvp_enable_flag in a slice header is set equal to 0 at the random access picture. By doing so, the MV of any CU in the random access picture and its following pictures is not predicted from that in the missing pictures. Thus, all MVs in all pictures are guaranteed to be correctly decoded.

Intra prediction

Three possible methods for avoiding error propagation from the dirty region to the clean region in intra prediction are described in the following.

The first method is to set constrained_intra_pred_flag equal to 1 for disabling reference to an inter-coded CU in intra prediction. By doing so, perfect reconstruction of intra-coded CU before in-loop filtering is guaranteed since that of a neighboring intra-coded CU is also guaranteed recursively. The coding efficiency with constrained_intra_pred_flag equal to 1 is obviously less than that with equal to 0.

Note: This method is used in the software created in AHG14.

The second method is to restrict the set of intra prediction modes subject to mode decision to a CU in the clean region. Specifically, the set is limited so that none of modes refers to samples of a CU in the dirty region. One example of such mode restriction is depicted in Figure 4. It is commented that this method shall be also applied to a CU which is in the clean region and is not adjacent to the clean region boundary if the range of reference is across the clean region boundary. It is also commented that the allowed set of intra prediction modes varies according to the position of the clean region boundary and the CU size (i.e. presence of smoothing filter to reference samples). The coding efficiency obviously decreases because of limited number of intra prediction modes used for a CU in the clean region.

Figure 4: Restriction on intra prediction mode

The third method is to insert a slice boundary. It is commented that this method is only effective if the clean region boundary lies on the CTU boundary. By doing so, reference to a CU in the dirty region is normatively disabled. One example of slice boundary insertion is depicted in Figure 5. It is observed non-negligible loss in coding efficiency occurs if the number of slices increases (up to the number of CTU lines per picture) due to vertical insertion of the clean region boundary (i.e. case 3 and 4 in Figure 5).

Figure 5: Slice boundary insertion for intra prediction

Intra block copy

One possible method is to restrict the search range of a displacement vector so as not to refer to the dirty region from a CU in the clean region. The coding efficiency obviously decreases because of limited search range of intra block copy for a CU in the clean region.

Inter prediction

One possible method is to restrict the search range of a motion vector so as not to refer to the dirty region in the reference picture from a CU in the clean region. The coding efficiency obviously decreases because of limited search range of inter prediction for a CU in the clean region.

Adaptive sample offset (SAO)

The source of error propagation is the derivation process of edgeIdx in the edge offset mode. Samples outside the CTU is referred for detecting the presence of an edge for the samples on the boundary of the CTU.

Note: This study is according to the specification of SAO in the HEVC standard.

Two possible methods for avoiding error propagation from the dirty region to the clean region in SAO are described in the following.

The first method is to restrict the set of SAO modes for a CTU in the clean region. If the clean region boundary exists at the CTU boundary or inside the CTU, then the SAO mode applied to the CTU is restricted to either band offset mode or “no SAO”. If the chosen edge offset class is not coincident to the direction of the dirty region from the clean region, the edge offset mode can be used. The coding efficiency decreases due to mode restriction at the CTU.

The second method is to insert a slice boundary. By doing so, reference to a CU in the dirty region is normatively disabled. Similarly as intra prediction, non-negligible loss in coding efficiency occurs if the number of slices increases due to the vertical insertion of the clean region boundary.

Deblocking filter

Samples in the neighboring CU are referred in the deblocking filtering process. If the neighboring CU is in the dirty region, deblocking filtering shall not be applied to a neighboring CU in the clean region. Unlike SAO and ALF, there is no normative way to disable deblocking filter at CTU level.

Note: This study is according to the specification of deblocking filter in the HEVC standard.

Two possible methods for avoiding error propagation from the dirty region to the clean region in deblocking filtering are described in the following.

The first method is to insert a slice boundary. It is commented that this method is only effective if the clean region boundary lies on the CTU boundary. By doing so, reference to a CU in the dirty region is normatively disabled. Similarly as intra prediction, non-negligible loss in coding efficiency occurs if the number of slices increases due to vertical insertion of the clean region boundary.

The second method is to insert a “semi-dirty” region between the dirty region and clean region. The detail is described below.

Figure 6 depicts the concept of a semi-dirty region in the case of the clean region boundary moving from left to right. A semi-dirty region is defined as a region in which sample values before in-loop filtering are guaranteed to be correctly decoded. Such guarantee is realized by intra-coding all CUs in the semi-dirty region with the method described in Subclause 2.3.2, for example. In the case of Figure 6, the width of the semi-dirty region is set larger than the summation of the range of samples to be altered by the filtering process inside the semi-dirty region and that affect to the filtering process inside the clean region. Thus, the semi-dirty region stops error propagation from the dirty region to the clean region in the deblocking filter process.

The coding efficiency reduces by the second method since the number of intra-coded CU increases.

Figure 6: Semi-dirty region for avoiding error propagation in deblocking filtering

Adaptive in-loop filter (ALF)

The sources of error propagation are the filtering process and derivation process of filter index. Samples outside the CTU is referred in those processes. It is noted that the disabling scheme of reference across a slice boundary is not defined in the current design of ALF.

One possible method for avoiding error propagation from the dirty region to the clean region in ALF is to disable ALF by setting alf_ctb_flag equal to 0 for all CTUs in which the clean region boundary exists either inside or at the CTU boundary. The coding efficiency decreases as the number of CTU to which ALF is applied is decreased.

Summary

Possible non-normative methods for correct reconstruction at the recovery point picture and the degree of loss of coding efficiency due to those methods are summarized in Table 2. It should be noted that overall loss in coding efficiency are the summation of the loss of each method.

Table 2: Non-normative methods for VVC tools

| Tool | Non-normative methods | Source of loss in coding efficiency |
| --- | --- | --- |
| ATMVP | 1.1) Set slice_temporal_mvp_enable_flag to 0 at the random access picture | 1.1) Non-maximal MV prediction performance for CU in the dirty region of the random access picture. |
| Intra prediction | 2.1) Set constrained_intra_pred_flag = 1 2.2) Restricted intra prediction mode selection 2.3) Insert slice boundary | 2.1) Non-maximal intra prediction performance for all intra CU. 2.2) Non-maximal intra prediction performance for CU inside the clean region and adjacent to the clean region boundary 2.3) Overhead of multiple slices |
| Intra block copy | 3.1) Limited search range | 3.1) Non-maximal prediction performance especially when optimal displacement vector points near the clean region boundary. |
| Inter prediction | 4.1) Limited search range | 4.1) Non-maximal prediction performance especially when optimal motion vector points near the clean region boundary. |
| SAO | 5.1) Limited SAO mode 5.2) Insert slice boundary | 5.1) Non-maximal filtering performance for CTU in which the clean region boundary exists. 5.2) Overhead of multiple slices |
| Deblocking filter | 6.1) Insert slice boundary 6.2) Insert semi-dirty region | 6.2) Overhead of multiple slices 6.3) Non-maximal coding performance cause by increased number of intra CU |
| ALF | 7.1) Disable ALF at CTU level | 7.1) Non-maximal filtering performance for CTU in which the clean region boundary exists. |

Possible normative methods to further mitigate loss in coding efficiency

As described above, the essential restriction is to disable the reference of samples and coding modes from the clean region to the dirty region. Non-normative methods described in Subclause 2.3, such as slice insertion or picture/CTU-based control can satisfy the restriction. However, such restriction by the non-normative methods also applied to a CU which does not need the restriction (e.g. a CU in the dirty region, intra CU far from the clean region boundary). Therefore, the coding efficiency of gradual intra refresh with the non-normative methods decreases unnecessarily.

One possible method to solve the issue straightforwardly is to handle a clean region boundary normatively, similarly as a slice boundary or picture boundary. By doing so, it is expected that the loss in coding efficiency is mitigated because of the relaxation or removal of restrictions on each CU. It is commented that this method works similarly to intra prediction and in-loop filtering at face discontinuities, which is now evaluated in CE13 “Coding tools for 360° omnidirectional video”. It is also commented that this idea has already been proposed in [1][2].

The qualitative analysis of possible mitigation loss in each coding tool, enabled by the normative handling of the clean region boundary, is summarized in Table 3.

Table 3: Possible mitigation of loss in coding efficiency enabled by normative clean region boundary

| Tool | Relaxed restriction by normative clean region boundary handling | Possible mitigation |
| --- | --- | --- |
| ATMVP | The reference of a temporal MV from CU in the clean region to a CU in the dirty region is normatively specified as “not available”. 1.1) No need to set slice_temporal_mvp_enable_flag to 1 at the random access picture | 1.1) CU in dirty region can always use a temporal MV in ATMVP. |
| Intra prediction | Samples in the opposite side of the clean region boundary is normatively specified as “not available”. 2.1) No need to set constrained_intra_pred_flag to 0. 2.2) No need to restrict intra prediction direction at intra CU at the clean region boundary. 2.3) No need to insert slices. | 2.1) All intra CU can refer to inter CU 2.2) All intra prediction directions can be chosen in an intra CU at clean region boundary 2.3) No overhead by inserting slice. |
| Intra block copy | Samples in the opposite side of the clean region boundary is normatively regarded as “outside picture”. 3.1) “Unrestricted motion vector” concept can be applied to a displacement vector points to samples near the clean region boundary by introducing existing padding schemes. | 3.1) A displacement vector can point near clean region boundary. |
| Inter prediction | Samples in the dirty region in the reference picture is normatively regarded as “outside picture”. 4.1) “Unrestricted motion vector” concept can be applied to a motion vector points to samples near the clean region boundary by introducing existing padding schemes. | 4.1) A motion vector can point near clean region boundary. |
| SAO | Samples in the opposite side of the clean region boundary is normatively specified as “outside picture”. 5.1) No need to restrict SAO mode of a CTU contains the clean region boundary 5.2) No need to insert slices. | 5.1) Optimized mode can be chosen in a CTU contains clean region boundary 5.2) No overhead by inserting slices. |
| Deblocking filter | Samples in the opposite side of the clean region boundary is normatively specified as “not available”. 6.1) No need to insert a slice boundary 6.2) No need to have a semi-dirty region | 6.1) No overhead by inserting slices. 6.2) No increase of the number of intra CU |
| ALF | Samples in the opposite side of the clean region boundary is normatively specified as “not available”. 7.1) No need to disable ALF at CTU level | 7.1) Optimized ALF selection in a CTU contains the clean region boundary |

Lastly, it is commented that the quantitative analysis will be performed in further study.

Conclusion

In this contribution, both non-normative and normative methods for perfect reconstruction (i.e. exact match) at a recovery point picture with the progressive (or gradual) intra refresh scheme are studied, according to the mandates of AHG14.

Firstly, various non-normative methods such as mode restriction and slice boundary insertion are enumerated. It is stated that such non-normative methods decrease coding efficiency unnecessarily since the restrictions brought by the non-normative methods overdo.

Secondly, it is stated that the normative handling of a clean region boundary in the decoding process is one possible way to mitigate the loss with the non-normative methods. The quantitative analysis is for further study.

The author proposes to continue the activity of AHG14 so to further investigate the issue of perfect reconstruction in gradual intra refresh, including the software development for the quantitative analysis of both the non-normative and normative methods described in this contribution.

References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2] K.Kazui, J.Koyama, A.Nakagawa, “Proposal of requirement on very low delay coding”, JCTVC-B031, March 2012.

[3] B.Bross, J.Chen, S.Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001, July 2018.

Patent rights declaration(s)

Fujitsu Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).