| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0092-v1 |
| --- | --- |

| Title: | CE4-related: A simplification algorithm for ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In hardware decoder designs, alternative temporal motion vector prediction (ATMVP) is challenging for small coding units (CUs) because a small CU has to be processed within a limited number of clock cycles to meet the real-time requirement. It is proposed to disable ATMVP for small CUs. When the proposed method is applied to VTM2.0.1 to disable ATMVP with CU size equal to or smaller than 128 luma samples, the luma BD-rates are reportedly 0.03% and 0.02% for RA and LB, respectively. When the proposed method is applied to VTM 2.0.1 to disable ATMVP with CU size equal to or smaller than 256 luma samples, the luma BD-rates are reportedly 0.07%, and 0.06% for RA and LB, respectively.

# Introduction

ATMVP is more complex than traditional spatial and temporal merge candidates. Sequential steps are:

- Find an initial vector according to the spatial merge candidates, up to four candidates are searched

- Constrain and modify the initial vector

- Find the corresponding block pointed by the modified initial vector

- Check the availability of the center motion vector (MV) of the corresponding block

- Find MVs of all collocated sub-CUs; if not available, use the center MV

- Scale collocated sub-CU MVs to reference index 0

- Use the center vector to prune the following temporal candidate

If the center MV is unavailable in Step 4, the process is terminated and ATMVP is not allowed. Table 1 shows the clock cycle budgets for small CUs to support real-time decoding. In hardware designs, the clock cycle budgets are already very tight to support real-time 8K 60fps or 120fps decoding for traditional merge modes. It is even more challenging for ATMVP. Moreover, to achieve higher coding gain, merge mode is getting more and more complex, and therefore simplifying ATMVP is highly suggested.

Table 1. Clock cycle budgets for one CU

| Assuming Clock @ 1GHz | 8K 60fps decoder | 8K 120fps decoder |
| --- | --- | --- |
| 4x4 CU | 8 | 4 |
| 4x8 or 8x4 CU | 16 | 8 |
| 8x8 CU | 32 | 16 |
| Assuming Clock @ 500MHz | 8K 60fps decoder | 8K 120fps decoder |
| 4x4 CU | 4 | 2 |
| 4x8 or 8x4 CU | 8 | 4 |
| 8x8 CU | 16 | 8 |

# Proposed method

A simplification algorithm for ATMVP is proposed. The algorithm simply disables ATMVP for small CUs. A threshold, THD, is pre-defined. If the current CU size in number of luma samples is equal to or smaller than THD, ATMVP is disabled for this CU.

# Simulation results

The proposed method is integrated on VTM2.0.1. All tests were conducted under the common test condition [1]. Table 2 shows the results of THD = 128, and Table 3 shows the results of THD = 256. By disabling ATMVP for small CUs, ATMVP can be easier to be supported by hardware decoders. In addition, slight software runtime reduction is also observed.

# Conclusions

This contribution proposes to disable ATMVP for small CUs to make real-time hardware design less challenging. It is shown that the coding loss can be controlled within 0.1% while the problem is alleviated.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-K1010.

Table 2. Results of disabling ATMVP when the CU size <= 128 luma samples

Table 3. Results of disabling ATMVP when the CU size <= 256 luma samples