| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0096-v1 |
| --- | --- |

| Title: | CE7-related: Context modeling of the position of last significant coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a context modelling method to code the position of the last significant coefficient of the coding block (CB). In this proposal, both x and y co-ordinate of the position of last coefficient share same context variables under certain conditions. This proposal reduces the number of context variables to code last_sig_coeff_x and last_sig_coeff_y syntax elements from 48 to 34 with 0.01% (Y), 0.05% (U), and 0.09 % (V) BD-rates for RA.

# Introduction

In VVC Draft 2 [1], the horizontal (x) and vertical (y) position of the last non-zero coefficient of a CB is sent by using two syntax elements, last_sig_coeff_x and last_sig_coeff_y, respectively. The last position, given by its coordinates in both x- and y- direction, is composed of prefix and suffix. At a given coordinate, the prefix value can be found from Table 1. The prefixes, last_sig_coeff_x_prefix and last_sig_coeff_y_prefix, are both regular coded using truncated unary binarization.

Separate context variable sets are used to code last_sig_coeff_x_prefix and last_sig_coeff_y_prefix. Also separate context variable sets are used for luma and chroma. The context variable depends on the width and height of the CB. Table 2 shows the context index increment (ctxInc) values of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix with different sizes. From Table 2, it is observed that, in VTM2.0.1, there are total 48 contexts to code last_sig_coeff_x_prefix and last_sig_coeff_y_prefix whereas the corresponding number of context variables in HEVC is 36.

# Proposed method

## Context variable sharing

In this proposal, one context variable set (CtxSetLastXY) is used for both last_sig_coeff_x_prefix and   last_sig_coeff_y_prefix, and under certain conditions both X- and Y-coordinate use the same context variables. The condition for sharing context variables is defined as follows.

if( channel type is chroma || width > 16 || height > 16 )

{

last_sig_coeff_x_prefix and last_sig_coeff_y_prefix share the same context variables.

}

else

{

last_sig_coeff_x_prefix and last_sig_coeff_y_prefix use different context variables.

}

Table 3 shows the ctxInc of the proposed method. It is shown that the number of contexts is reduced to 34. Table 4 shows the comparison of the numbers of context variables of HEVC, VTM2.0.1 and this proposal.

## Context modelling

Same as VTM2.0.1, context indices are computed from width and height of the CB without using any look-up table. Assume CtxSetLastXY is the joint context variable set for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix.

Following pseudo-code is used to compute four parameters shiftX, shiftY, offsetX, and offsetY.

if (m_chType == CHANNEL_TYPE_CHROMA)

{

shiftX = Clip3( 0, 2, width >> 3 );

shiftY = Clip3( 0, 2, height >> 3 );

offsetX = 31;

offsetY = 31;

}

else

{

log2sizeX = log2(width);

log2sizeY = log2(height);

prefix_ctx[8] = { 0, 0, 0, 3, 6, 10, 15, 21 };

offsetX = prefix_ctx[log2sizeX];

offsetY = prefix_ctx[log2sizeY];

shiftX = (log2sizeX + 1) >> 2;

shiftY = (log2sizeY + 1) >> 2;

}

Context variable selection to code a bin (binNumX) of last_sig_coeff_x_prefix is computed as follows:

ctxIncX= offsetX + ( binNumX >> shiftX );

ctxX = CtxSetLastXY(ctxIncX);

Context variable selection to code a bin (binNumY) of last_sig_coeff_y_prefix is computed as follows:

ctxIncY= offsetY + ( binNumY >> shiftY );

if (m_chType == CHANNEL_TYPE_LUMA && height < 32)

ctxIncY+= 21;

ctxY = CtxSetLastXY(ctxIncY);

# Simulation results

The proposed method is implemented in BMS2.0.1 with VTM configuration. All of the tests are performed under common test conditions described in [2]. Table 5 shows the simulation results. The run time numbers are not accurate. It is believed that the run time change, if accurately measured, should be negligible.

# Conclusions

Compared to VTM2.0.1, this proposal reduces the number of context variables for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix from 48 to 34 with 0.01% (Y), 0.05% (U), and 0.09 % (V) BD-rates for RA.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team, JVET-K1001.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.

Table 1.  Prefix values

| Coordinate | 0 | 1 | 2 | 3 | 4~5 | 6~7 | 8~11 | 12~15 | 16~23 | 24~31 | 32~47 | 48~63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prefix | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

Table 2.  ctxInc in VTM2.0.1

| binNum | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| last_sig_coeff_x_prefix (Luma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 0 | 1 | 2 |  |  |  |  |  |  |  |  |
| Size = 8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |  |  |
| Size = 16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |  |  |
| Size = 32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |  |  |
| Size = 64 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | 19 | 20 |
| last_sig_coeff_y_prefix (Luma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 21 | 22 | 23 |  |  |  |  |  |  |  |  |
| Size = 8 | 24 | 24 | 25 | 25 | 26 |  |  |  |  |  |  |
| Size = 16 | 27 | 27 | 28 | 28 | 29 | 29 | 30 |  |  |  |  |
| Size = 32 | 31 | 31 | 32 | 32 | 33 | 33 | 34 | 34 | 35 |  |  |
| Size = 64 | 36 | 36 | 37 | 37 | 38 | 38 | 39 | 39 | 40 | 40 | 41 |
| last_sig_coeff_x_prefix (Chroma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 2 | 42 |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 42 | 43 | 44 |  |  |  |  |  |  |  |  |
| Size = 8 | 42 | 42 | 43 | 43 | 44 |  |  |  |  |  |  |
| Size = 16 | 42 | 42 | 42 | 42 | 43 | 43 | 43 |  |  |  |  |
| Size = 32 | 42 | 42 | 42 | 42 | 43 | 43 | 43 | 43 | 44 |  |  |
| last_sig_coeff_y_prefix (Chroma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 2 | 45 |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 45 | 46 | 47 |  |  |  |  |  |  |  |  |
| Size = 8 | 45 | 45 | 46 | 46 | 47 |  |  |  |  |  |  |
| Size = 16 | 45 | 45 | 45 | 45 | 46 | 46 | 46 |  |  |  |  |
| Size = 32 | 45 | 45 | 45 | 45 | 46 | 46 | 46 | 46 | 47 |  |  |

Table 3.  ctxInc in this proposal

| binNum | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| last_sig_coeff_x_prefix (Luma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 0 | 1 | 2 |  |  |  |  |  |  |  |  |
| Size = 8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |  |  |
| Size = 16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |  |  |
| Size = 32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |  |  |
| Size = 64 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | 19 | 20 |
| last_sig_coeff_y_prefix (Luma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 21 | 22 | 23 |  |  |  |  |  |  |  |  |
| Size = 8 | 24 | 24 | 25 | 25 | 26 |  |  |  |  |  |  |
| Size = 16 | 27 | 27 | 28 | 28 | 29 | 29 | 30 |  |  |  |  |
| Size = 32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |  |  |
| Size = 64 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 | 19 | 19 | 20 |
| Both for last_sig_coeff_x_prefix and last_sig_coeff_y_prefix (Chroma) |  |  |  |  |  |  |  |  |  |  |  |
| Size = 2 | 31 |  |  |  |  |  |  |  |  |  |  |
| Size = 4 | 31 | 32 | 33 |  |  |  |  |  |  |  |  |
| Size = 8 | 31 | 31 | 32 | 32 | 33 |  |  |  |  |  |  |
| Size = 16 | 31 | 31 | 31 | 31 | 32 | 32 | 32 |  |  |  |  |
| Size = 32 | 31 | 31 | 31 | 31 | 32 | 32 | 32 | 32 | 33 |  |  |

Table 4.  Number of context variables to code the position of last significant coefficient

| Color component | Syntax element | HEVC | VTM2.0.1 | Proposed |
| --- | --- | --- | --- | --- |
| Luma | last_sig_coeff_x_prefix | 15 | 21 | 31 |
|  | last_sig_coeff_y_prefix | 15 | 21 |  |
| Chroma | last_sig_coeff_x_prefix | 3 | 3 | 3 |
|  | last_sig_coeff_y_prefix | 3 | 3 |  |
| Total |  | 36 | 48 | 34 |

Table 5.  Simulation results of the proposed method (runtime is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.08% | 0.04% | 100% | 102% |
| Class A2 | 0.03% | -0.02% | 0.02% | 100% | 104% |
| Class B | 0.05% | 0.14% | 0.13% | 100% | 102% |
| Class C | 0.00% | 0.07% | 0.03% | 100% | 100% |
| Class E | 0.07% | 0.09% | -0.07% | 100% | 100% |
| Overall | 0.04% | 0.05% | 0.04% | 100% | 102% |
| Class D | 0.00% | 0.24% | -0.20% | 100% | 100% |
| Class F (optional) | 0.02% | -0.01% | 0.09% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.03% | 101% | 106% |
| Class A2 | 0.01% | -0.01% | 0.05% | 100% | 104% |
| Class B | 0.02% | -0.01% | -0.03% | 100% | 103% |
| Class C | 0.01% | 0.16% | 0.31% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.09% | 100% | 103% |
| Class D | 0.00% | -0.03% | 0.39% | 100% | 100% |
| Class F (optional) | 0.02% | 0.01% | 0.26% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | -0.17% | 101% | 103% |
| Class C | 0.05% | -0.10% | -0.11% | 100% | 101% |
| Class E | -0.09% | -0.23% | -0.64% | 100% | 100% |
| Overall | 0.00% | -0.13% | -0.27% | 100% | 102% |
| Class D | 0.05% | 0.41% | -0.22% | 100% | 100% |
| Class F (optional) | -0.05% | 0.11% | -0.42% | 100% | 101% |