| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0100-v2 |
| --- | --- |

| Title: | CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, multi-hypothesis prediction is proposed to improve the existing prediction modes in inter pictures, including uni-prediction of advanced motion vector prediction (AMVP) mode, skip and merge mode, and intra mode. The general concept is to combine an existing prediction mode with an extra merge indexed prediction. The merge indexed prediction is performed as in merge mode, where a merge index is signaled to acquire motion information for the motion compensated prediction. The final prediction is the weighted average of the merge indexed prediction and the prediction generated by the existing prediction mode, where different weights are applied depending on the combinations. It is reported that, compared to VTM2.0.1, the proposed CE10.1.1.d achieves -1.05% and -0.65% luma BD-rates with 15% and 18% encoding time increases and 5% and 4% decoding time increases for RA and LB, respectively. When CE10.1.1.d is further combined with CE4.4.12, -1.29% and -0.70% luma BD-rates with 16% and 18% encoding time increases and 5% and 4% decoding time increases for RA and LB, respectively, are reported.

# Introduction

Three inter combined prediction modes are tested to combine merge indexed prediction with uni-prediction of AMVP mode, merge mode, and intra mode, respectively. When combined with uni-prediction of AMVP mode, one motion is acquired using the original syntax elements in AMVP mode while the other is acquired using the merge index, where totally two hypotheses are used. When combined with merge mode, implicit selection of succeeding merge candidate without signaling any additional merge index is used to acquire the additional motion information, where at most two more hypotheses are used. When combined with intra mode, explicit signaling of intra mode for merge mode in inter coding unit (CU) is applied to generate the combined predictions, where one more hypothesis from intra mode is used. Different weights to generate the final prediction depending on the combinations are also tested.

# Proposed methods

## CE10.1.1.a: Multi-hypothesis prediction for uni-prediction of AMVP mode

When the multi-hypothesis prediction is applied to improve uni-prediction of AMVP mode, one flag is signaled to enable or disable multi-hypothesis prediction for inter_dir equal to 1 or 2, where 1, 2, and 3 represent list 0, list 1, and bi-prediction, respectively. Moreover, one more merge index is signaled when the flag is true. In this way, multi-hypothesis prediction turns uni-prediction into bi-prediction, where one motion is acquired using the original syntax elements in AMVP mode while the other is acquired using the merge scheme. The final prediction uses 1:1 weights to combine these two predictions as in bi-prediction. The merge candidate list is first derived from merge mode with sub-CU candidates (e.g., affine, alternative temporal motion vector prediction (ATMVP)) excluded. Next, it is separated into two individual lists, one for list 0 (L0) containing all L0 motions from the candidates, and the other for list 1 (L1) containing all L1 motions. After removing redundancy and filling vacancy, two merge lists are generated for L0 and L1 respectively. There are two constraints when applying multi-hypothesis prediction for improving AMVP mode. First, it is enabled for those CUs with the luma coding block (CB) area larger than or equal to 64. Second, it is only applied to L1 when in low delay B pictures.

## CE10.1.1.b: Multi-hypothesis prediction for skip/merge mode

When the multi-hypothesis prediction is applied to skip or merge mode, whether to enable multi-hypothesis prediction is explicitly signaled. An extra merge indexed prediction is selected in addition to the original one. Therefore, each candidate of multi-hypothesis prediction implies a pair of merge candidates, containing one for the 1st merge indexed prediction and the other for the 2nd merge indexed prediction. However, in each pair, the merge candidate for the 2nd merge indexed prediction is implicitly derived as the succeeding merge candidate (i.e., the already signaled merge index plus one) without signaling any additional merge index. After removing redundancy by excluding those pairs, containing similar merge candidates and filling vacancy, the candidate list for multi-hypothesis prediction is formed. Then, motions from a pair of two merge candidates are acquired to generate the final prediction, where 5:3 weights are applied to the 1st and 2nd merge indexed predictions, respectively. Moreover, a merge or skip CU with multi-hypothesis prediction enabled can save the motion information of the additional hypotheses for reference of the following neighboring CUs in addition to the motion information of the existing hypotheses. Note that sub-CU candidates (e.g., affine, ATMVP) are excluded from the candidate list, and for low delay B pictures, multi-hypothesis prediction is not applied to skip mode. Moreover, when multi-hypothesis prediction is applied to merge or skip mode, for those CUs with CU width or CU height less than 16, or those CUs with both CU width and CU height equal to 16, bi-linear interpolation filter is used in motion compensation for multiple hypotheses. Therefore, the worst-case bandwidth (required access samples per sample) for each merge or skip CU with multi-hypothesis prediction enabled is calculated in Table 1 and each number is less than half of the worst-case bandwidth for each 4x4 CU with multi-hypothesis prediction disabled.

Table 1. The worst-case of bandwidth (required access samples per sample) for each merge or skip CU with multi-hypothesis prediction enabled

| CU height CU width | 4 | 8 | 16 | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 6.25 | 5.63 | 5.31 | 5.16 | 5.08 | 5.04 | 5.02 |
| 8 | 5.63 | 5.06 | 4.78 | 4.64 | 4.57 | 4.54 | 4.52 |
| 16 | 5.31 | 4.78 | 4.52 | 7.01 | 6.38 | 6.06 | 5.91 |
| 32 | 5.16 | 4.64 | 7.01 | 5.94 | 5.41 | 5.14 | 5.01 |
| 64 | 5.08 | 4.57 | 6.38 | 5.41 | 4.92 | 4.68 | 4.56 |
| 128 | 5.04 | 4.54 | 6.06 | 5.14 | 4.68 | 4.45 | 4.33 |
| 256 | 5.02 | 4.52 | 5.91 | 5.01 | 4.56 | 4.33 | 4.22 |

## CE10.1.1.c: Multi-hypothesis prediction for intra and inter mode

When the multi-hypothesis prediction is applied to improve intra mode, multi-hypothesis prediction combines one intra prediction and one merge indexed prediction. In a merge CU, one flag is signaled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the intra candidate list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is exclusive of the intra mode list and when the CU height is larger than the double of CU width, vertical mode is removed from the intra mode list. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted average. For chroma component, DM is always applied without extra signaling. The weights for combining predictions are described as follow. When DC or planar mode is selected or the CB width or height is smaller than 4, equal weights are applied. For those CBs with CB width and height larger than or equal to 4, when horizontal/vertical mode is selected, one CB is first vertically/horizontally split into four equal-area regions. Each weight set, denoted as (w_intrai, w_interi), where i is from 1 to 4 and (w_intra1, w_inter1) = (6, 2), (w_intra2, w_inter2) = (5, 3), (w_intra3, w_inter3) = (3, 5), and (w_intra4, w_inter4) = (2, 6), will be applied to a corresponding region. (w_intra1, w_inter1) is for the region closest to the reference samples and (w_intra4, w_inter4) is for the region farthest away from the reference samples. Then, the combined prediction can be calculated by summing up the two weighted predictions and right-shifting 3 bits. Moreover, the intra prediction mode for the intra hypothesis of predictors can be saved for reference of the following neighboring CUs.

## CE10.1.1.d: Multi-hypothesis prediction

The multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode is enabled for those CBs with the CB area larger than or equal to 64.

## Combination of CE10.1.1.b and CE10.1.1.c

CE10.1.1.b and CE10.1.1.c are combined and the multi-hypothesis prediction is applied for improving skip or merge mode, and intra mode.

## Combination of CE10.1.1.d and CE4.4.12

CE10.1.1.d and CE4.4.12 are combined with merge list size as 6. Multi-hypothesis prediction is applied to AMVP, skip, merge, and intra mode. In addition, the candidates for multi-hypothesis are generated from the merge list where HEVC combined candidates are replaced with pairwise average candidates.

# Syntax and semantics

The proposed syntax changes are highlighted in blue as follows.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] |  |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) { |  |
| if( mvd_l1_zero_flag == 0 ) |  |
| mh_merge_flag[ x0 ][ y0 ] | ae(v) |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| mh_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && mh_merge_flag[ x0 ][ y0 ] == 0 ) |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) |  |
| mh_intra_mode_idx [ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| if(inter_affine_flag = = 0 && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) |  |
| mh_uni_inter_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_uni_inter_flag[ x0 ][ y0 ] ) |  |
| mh_uni_inter_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

mh_merge_flag equal to TRUE specifies that multi-hypothesis prediction is applied to skip or merge mode.

mh_intra_flag equal to TRUE specifies that multi-hypothesis prediction is applied to intra mode.

mh_intra_mode_idx is a function to signal the intra prediction mode for multi-hypothesis prediction with contexts as follows:

–	A flag is first signaled to indicate whether the selected intra prediction mode is one of three MPMs or not. Note that when the size of intra mode list is 3, the flag needs not to be signaled and can be inferred to be TRUE.

–	If the above flag is TRUE, the selected intra prediction mode can be acquired by an index, which indicates into the derived MPM list.

–	If the above flag is FALSE, the selected intra prediction mode is the only one intra prediction mode, which does not belong to the derived MPM list.

mh_uni_inter_flag equals to TRUE specifies that multi-hypothesis prediction is applied to uni-prediction for AMVP mode.

mh_uni_inter_idx specifies the merge index to generate the merge-indexed prediction for multi-hypothesis prediction.

# Simulation results

The proposed multi-hypothesis prediction methods have been implemented on BMS2.0.1 [1]. Table 2, Table 3, Table 4, Table 5, Table 6, and Table 7 show the results of CE10.1.1.a, CE10.1.1.b, CE10.1.1.c, CE10.1.1.d, the combination of CE10.1.1.b and CE10.1.1.c, and the combination of CE10.1.1.d and CE4.4.12, respectively. The simulations were performed following the JVET common test conditions (CTC) [2].

# Conclusions

In this contribution, it is proposed to improve the existing prediction modes in inter pictures, including uni-prediction of AMVP mode, skip and merge mode, and intra mode through combining an existing prediction mode with an extra merge indexed prediction. Compared against VTM2.0.1, the proposed mechanism (CE10.1.1.d) can achieve -1.05% and -0.65% luma BD-rates with 15% and 18% encoding time increases and 5% and 4% decoding time increases for RA and LB, respectively. When CE10.1.1.d is further combined with CE4.4.12, -1.29% and -0.70% luma BD-rates with 16% and 18% encoding time increases and 5% and 4% decoding time increases for RA and LB, respectively, are reported.

# References

[1] BMS-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2. Results of CE10.1.1.a: Multi-hypothesis prediction for uni-prediction of AMVP mode

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.29% | -0.20% | 108% | 97% |
| Class A2 | -0.21% | -0.28% | -0.26% | 109% | 103% |
| Class B | -0.46% | -0.48% | -0.48% | 109% | 101% |
| Class C | -0.21% | -0.01% | -0.18% | 110% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.27% | -0.30% | 109% | 101% |
| Class D | -0.23% | -0.32% | -0.29% | 110% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.12% | -0.22% | 110% | 100% |
| Class C | -0.11% | -0.23% | -0.35% | 110% | 102% |
| Class E | -0.27% | -0.05% | -0.29% | 111% | 103% |
| Overall | -0.17% | -0.14% | -0.28% | 110% | 102% |
| Class D | -0.08% | 0.18% | -0.08% | 110% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

Table 3. Results of CE10.1.1.b: Multi-hypothesis prediction for skip/merge mode

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -0.69% | -0.96% | 104% | 101% |
| Class A2 | -0.35% | -0.60% | -0.63% | 106% | 107% |
| Class B | -0.90% | -0.95% | -0.92% | 106% | 103% |
| Class C | -0.21% | -0.18% | -0.38% | 108% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.55% | -0.62% | -0.73% | 106% | 104% |
| Class D | -0.03% | -0.08% | -0.13% | 109% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -0.39% | -0.17% | 108% | 101% |
| Class C | -0.27% | -0.04% | -0.34% | 109% | 104% |
| Class E | -0.17% | 0.00% | -0.10% | 107% | 103% |
| Overall | -0.35% | -0.18% | -0.21% | 108% | 103% |
| Class D | -0.02% | 0.57% | -0.14% | 110% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

Table 4. Results of CE10.1.1.c: Multi-hypothesis prediction for intra and inter mode

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.29% | -0.30% | 107% | 102% |
| Class A2 | -0.44% | -0.46% | -0.65% | 109% | 103% |
| Class B | -0.57% | -0.49% | -0.46% | 108% | 103% |
| Class C | -0.53% | -0.37% | -0.60% | 110% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.51% | -0.41% | -0.50% | 108% | 103% |
| Class D | -0.33% | -0.23% | -0.48% | 110% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -1.06% | -0.83% | 108% | 100% |
| Class C | -0.56% | -0.69% | -1.08% | 110% | 105% |
| Class E | -0.37% | -0.91% | -0.51% | 107% | 105% |
| Overall | -0.50% | -0.90% | -0.83% | 109% | 103% |
| Class D | -0.37% | -0.97% | -1.43% | 110% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

Table 5. Results of CE10.1.1.d: Multi-hypothesis prediction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -1.24% | -1.40% | 113% | 101% |
| Class A2 | -0.77% | -1.12% | -1.08% | 116% | 106% |
| Class B | -1.50% | -1.70% | -1.41% | 115% | 106% |
| Class C | -0.66% | -0.62% | -0.75% | 117% | 106% |
| Class E |  |  |  |  |  |
| Overall | -1.05% | -1.20% | -1.17% | 115% | 105% |
| Class D | -0.34% | -0.32% | -0.29% | 119% | 105% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.94% | -1.23% | -0.90% | 118% | 102% |
| Class C | -0.59% | -0.49% | -0.85% | 118% | 105% |
| Class E | -0.26% | -0.54% | -0.64% | 117% | 105% |
| Overall | -0.65% | -0.81% | -0.82% | 118% | 104% |
| Class D | -0.31% | -0.16% | -1.00% | 119% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

Table 6. Results of combining CE10.1.1.b and CE10.1.1.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.88% | -1.07% | 105% | 106% |
| Class A2 | -0.57% | -0.76% | -0.74% | 107% | 106% |
| Class B | -1.15% | -1.28% | -1.14% | 106% | 107% |
| Class C | -0.46% | -0.43% | -0.58% | 108% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -0.87% | -0.90% | 106% | 107% |
| Class D | -0.18% | -0.09% | -0.03% | 109% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.75% | -0.92% | -1.07% | 108% | 102% |
| Class C | -0.43% | -0.51% | -0.70% | 108% | 105% |
| Class E | -0.19% | -0.27% | -0.74% | 107% | 105% |
| Overall | -0.50% | -0.62% | -0.86% | 108% | 104% |
| Class D | -0.18% | -0.39% | -0.63% | 108% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |

Table 7. Results of combining CE10.1.1.d and CE4.4.12

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.33% | -1.50% | -1.55% | 113% | 105% |
| Class A2 | -1.02% | -1.19% | -1.26% | 116% | 107% |
| Class B | -1.73% | -1.81% | -1.62% | 115% | 104% |
| Class C | -0.92% | -0.83% | -0.97% | 117% | 106% |
| Class E |  |  |  |  |  |
| Overall | -1.29% | -1.36% | -1.36% | 116% | 105% |
| Class D | -0.64% | -0.70% | -0.61% | 118% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.96% | -1.14% | -1.25% | 118% | 102% |
| Class C | -0.69% | -0.64% | -0.85% | 119% | 105% |
| Class E | -0.26% | -0.54% | -0.55% | 117% | 105% |
| Overall | -0.70% | -0.82% | -0.94% | 118% | 104% |
| Class D | -0.29% | -0.17% | -0.83% | 119% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 100% | 100% |