| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0104 |
| --- | --- |

| Title: | AHG5: Reducing VVC worst-case memory bandwidth by restricting bi-directional 4x4 inter CUs/Sub-blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang 8910 University Center Ln, Suit 400 San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-705-2600 yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In HEVC, no 4x4 motion compensation (MC) is allowed and 8x4/4x8 PUs are restricted to be uni-directional motion compensation. Currently there is no restriction on the block size for allowing bi-directional motion compensation in VVC. To reduce the worst-case memory bandwidth of VVC, we propose some restrictions that the smallest bi-directional MC block size is enlarged from 4x4 to 8x4/4x8. Specifically, the restrictions include: 1) only uni-directional motion compensation is allowed for each 4x4 inter CU, 2) the smallest sub-block size for ATMVP coded CU is enlarged to 8x8, and 3) 4x4 is used as the sub-block size for a uni-directional affine CU while 8x4/4x8 is used for a bi-directional affine CU. The proposed restrictions can reduce the worst-case memory access bandwidth by about 32% for the current VVC, with the luma losses of 0.14% and 0.16% with 98% and 96% decoding time respectively under RA and LDB cases.

# Introduction

Motion compensation (MC) is usually the largest consumer of memory bandwidth in the decoder implementations. Therefore, for a video coding standard, providing a reasonable limit on MC bandwidth requirement is extremely important to facilitate its cost-effective implementations and ensure its success in industry.

Memory access bandwidth requirement for MC is usually determined by the operating block size and the type of prediction (e.g. uni-directional or bi-directional). In the current VVC, there is no limitation in this regard. As a result, the worst-case bandwidth requirement is more than 2x the corresponding worst-case for HEVC.

In VVC, the worst case of MC memory bandwidth occurs with the bi-directional MC of a 4x4 block, which is utilized by the following coding modes.

- 4x4 CU coded as bi-directional inter mode (AMVP or normal merge mode)

- CUs coded as ATMVP merge mode with 4x4 size of sub-blocks

- CUs coded as bi-directional affine mode (where sub-block size is always 4x4)

# Proposed method

The proposal recommends restricting a 4x4 block from using bi-directional motion compensation. Such a restriction results in 32% reduction of the worst-case bandwidth requirement in current VVC. The details of the proposal are as follows:

- Disallow bi-directional inter mode (AMVP mode and Merge mode) for each 4x4 inter CU.

- Enlarge the minimum sub-block size of ATMVP merge mode from 4x4 to 8x8; ATMVP is not allowed when either width or height of a CU is equal to 4.

- 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4/4x8 block is used as the sub-block size for a bi-directional affine coded CU.

With the proposed restrictions, the bi-directional 8x4/4x8 size MC is the new worst-case, which is about 32% higher than the corresponding worst-case for HEVC.

# Simulation results

The tables below show the BD-rate performance and processing time of the proposed restrictions. The simulations are conducted following the common test conditions [1] on top of reference software BMS-2.0.1 [2] using the VTM configurations.

Table 1. Performance of the proposed restrictions on bi-directional inter mode (AMVP and Merge)

Table 2. Performance of the proposed restrictions on ATMVP

Table 3. Performance of the proposed restrictions on Affine modes

Table 4. Performance of all the proposed restrictions

# Conclusions

In this proposal, to reduce the worst-case memory access bandwidth for motion compensation in VVC, we propose restrictions of bi-directional MC block size to be at least 4x8/8x4 or larger. The proposed restrictions result in 32% reduction of the worst-case bandwidth requirement in current VVC, with 0.14% and 0.16% BD-rate increase respectively under RA and LDB cases. It is recommended to consider adoption of these restrictions to reduce the worst-case memory bandwidth requirement.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-K1010, July 2018.

- BMS-2.0.1 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).