| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0106 |
| --- | --- |

| Title: | CE4-related: Modified History-based MVP to support parallel processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang 8910 University Center Ln, Suit 400 San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-705-2600 yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

A history-based motion vector prediction (HMVP) method for inter coding was proposed in the 11th JVET meeting. In the scheme of HMVP, a table of HMVP candidates is emptied before coding each slice. The HMVP table is then maintained and updated on-the-fly during the coding of the slice. However, such slice-based update of the HMVP table may become an obstacle to wave-front parallel processing. In this proposal, it is proposed to reset the HMVP table before coding each CTU row. The proposed modification causes only 0.03% and 0.03% BD rate increase with negligible run time differences under RA and LDB cases.

# Introduction

In the 11th JVET meeting, a history-based MVP (HMVP) method was proposed [1] wherein a HMVP candidate is defined as the motion information of a previously coded block. A table with multiple HMVP candidates is maintained during the encoding/decoding process. The table is emptied only when starting a new slice. Such update of the HMVP table may become an obstacle to wave-front parallel processing (WPP), because the HMVP table for the next WPP is unavailable until a current WPP is fully coded. As shown in Figure1, due to the wait for the HMVP table update from the last CU of the 1st WPP thread (the first CTU row), the 2nd and other remaining threads can not be started.

Figure 1: Interaction between current HMVP update scheme and WPP parallel processing

# Proposed method

In this proposal, we simply propose to reset this HMVP table before coding each CTU row. With the proposed modification, coding of each WPP thread/CTU row can be started without waiting for the HMVP table update from the upper WPP thread/CTU row.

# Simulation results

The tables below show the BD-rate performance and processing time of the proposed method. The simulations are conducted following the common test conditions [2] on top of reference software VTM-2.0.1 [3]. The performance of HMVP is shown in Table 1. And the proposed modification introduces 0.03% and 0.03% BD-rate loss over the HMVP scheme under RA and LDB cases as shown in Table 2.

Table 1. The performance of HMVP (CE4.4.7.c) with MaxNumMergeCand=6 compared to VTM-2.0.1

Table 2. The performance of the modified HMVP with MaxNumMergeCand=6 compared to the HMVP (CE4.4.7.c) with MaxNumMergeCand=6

# Conclusions

In the current HMVP scheme, the update mechanism of the HMVP table is not friendly to WPP parallel processing. In this propose, it is suggested the HMVP table be reset before coding each CTU row to accommodate WPP processing. Results show that the proposed changes have minor coding performance impact. It is recommended to integrate the proposed modification into the HMVP implementation for future investigation of the tool.

# Reference

- “CE4-related: History-based Motion Vector Prediction”, L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, doc. JVET-K0104, July 2018.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-K1010, July 2018.

- VTM-2.0.1 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).