| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0107-v2 |
| --- | --- |

| Title: | [Non-CE3: CCLM Performance Of Extended Neighboring Region] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan Junyan Huo Xiaoyan Chai Yanzhuo Ma YuanFang Yu Yang Liu | Email: | swan@ nwpu.edu.cn jyhuo@mail.xidian.edu.cn xychai@stu.xidian.edu.cn yzma@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In this contribution, the coding performance of CCLM is reported through adding the top-right and bottom-left neighbouring samples into the derivation process of the linear model parameters. The simulation results reportedly show that -0.04%, -0.31% and -0.33% BD rate savings on Y, Cb and Cr components respectively for All Intra configuration.

Introduction

The linear model parameters α and β in CCLM adopt by VVC are based on a least means square algorithm given by the following formula:

α= 2N·Ln·Cn-L(n)C(n)2N·Ln·Ln-L(n)L(n)

β= Cn-αL(n)2N

where Ln represents the down-sampled top and left neighbouring reconstructed luma samples, and Cn represents the top and left neighbouring reconstructed chroma samples. Figure 1 shows the location of the left and top samples and the samples of the current block involved in the CCLM mode.

Figure 1 Locations of the samples used for the derivation of α and β

Proposed method

In this proposed method, the neighbouring reconstructed luma samples used for the derivation of α and β are extended to the top-right and bottom-left. As shown in Figure 2, the number of neighbouring samples becomes twice as the original. The down-sampling method for the neighboring samples is the same as that in CCLM.

Figure 2 The extended neighbouring reconstructed samples

Experimental results

In Table 1 given below experimental results of proposed method are summarized.

Anchor: VTM2.0.1

Tested: The proposed method

Table 1. Summary of experimental results (Anchor: VTM 2.0.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.59% | -0.54% |  | 101% |
| Class A2 | -0.04% | -0.32% | -0.18% |  | 100% |
| Class B | -0.03% | -0.24% | -0.42% |  | 100% |
| Class C | -0.02% | -0.28% | -0.26% |  | 99% |
| Class E | -0.03% | -0.16% | -0.24% |  | 99% |
| Overall | -0.04% | -0.31% | -0.33% |  | 100% |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

In this contribution, the top-right and bottom-left neighbouring reconstructed luma samples are added for the derivation of α and β in CCLM. The proposed method gives -0.04%, -0.31% and -0.33% BD rate savings on Y, Cb and Cr components respectively for All Intra configuration.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).