| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0108-v2 |
| --- | --- |

| Title: | [Non-CE3:Current luma related-CCLM (CCCLM)] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Xinwei Li Jianglin Wang Yanzhuo Ma Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn jlwang_2@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

This contribution presents a Cross-Component Linear Model method based on Current reconstructed luma (CCCLM). The linear model parameters α and β in CCCLM are derived according to the reconstructed luma of current block and reconstructed luma/chroma of neighboring region. The reported BD-rates over VTM-2.0.1 are −0.14% (Y), −0.57% (U), −0.63% (V) with runtimes 116% (Dec) for AI configuration.

This contribution further introduces a restriction which uses CCCLM only for blocks with the size no larger than 64 samples. The reported BD-rates over VTM-2.0.1 are −0.13% (Y), −0.55% (U), −0.65% (V) with runtimes 102% (Dec) for AI configuration.

Introduction

The linear model parameters α and β in CCLM adopted by VVC are based on a least means square algorithm given by the following formula:

α= N·Ln·Cn-L(n)C(n)N·Ln·Ln-L(n)L(n)                            (1)

β= Cn-αL(n)N                                      (2)

where Ln represents the down-sampled top and left neighbouring reconstructed luma samples, and Cn represents the top and left neighbouring reconstructed chroma samples. Figure 1 shows the location of the left and top samples and the samples of the current block involved in the CCLM mode.

Figure 1 Locations of the samples used for the derivation of α and β

Proposed method

Before CCLM, the reconstructed luma of the current block has been determined. In CCCLM, the reconstructed luma of the current block joins the CCLM modelling process. Specifically, the reconstructed luma and the temporary chroma of the current block are used to derive the parameters α and β. By employing the reconstructed luma in CCLM modeling, the statistics in the current luma can be well explored.

Assuming the size of the current block is MxN, the parameters α and β are obtained according to the following equation:

α= MN·Lx,y·Cx,y-L(x,y)C(x,y)MN·Lx,y·Lx,y-L(x,y)L(x,y)                     (3)

β= Cx,y-αL(x,y)MN                                 (4)

where x and y are the coordinates of the current block. Lx,y represents the down-sampled reconstructed luma samples of the current block, and Cx,y represents the temporary chroma samples derived from the best matched pixel, as specified in 2.1.

2.1 Derivation of the temporary chroma

The temporary chroma is determined as follows:

For each pixel of the current block, the best matched pixel in the neighboring region is found. The best matched pixel is defined as the pixel which has the smallest absolute luma difference with the current luma value.

After the best matched pixel is determined, the chroma value of the best matched pixel is utilized as the temporary chroma sample of the current pixel.

In this proposal, the neighbouring region is extended into the top, top-right, left, and left-bottom samples, as discussed in JVET-L0107.

2.2 Implementation of the CCCLM

In this proposal, we replace the CCLM in VTM2.0.1 by the proposed CCCLM.

2.3 Chroma block size restriction

It is well known that chroma blocks with large size are usually flat when they are selected as the best CU. In this way, the statistics of the current block and neighboring region are similar with a high probability. To alleviate the complexity, a block size restriction criterion is introduced. Specifically, for the chroma block smaller than or equal to 64 samples, the CCCLM is used; for the chroma blocks with size more than 64, the existing CCLM in VTM 2.0.1 is used.

Experimental results

The proposed scheme is implemented based on BMS 2.0.1 with the VTM configuration.

The coding performance of All Intra configuration is provided in Table 1 and Table 2. Test #1 reports the coding performance of the proposed CCCLM. Test #2 reports the coding performance of the proposed method with chroma block size restriction.

Test #1:

Anchor: VTM2.0.1

Tested: The proposed CCCLM method

Table 1. Summary of experimental results (CCCLM vs. VTM2.0.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.13% | -0.06% |  | 133% |
| Class A2 | -0.07% | -0.55% | -0.51% |  | 115% |
| Class B | -0.10% | -0.62% | -0.83% |  | 117% |
| Class C | -0.21% | -0.82% | -0.99% |  | 112% |
| Class E | -0.06% | -0.58% | -0.51% |  | 108% |
| Overall | -0.14% | -0.57% | -0.63% |  | 116% |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test #2:

Anchor: VTM2.0.1

Tested: CCCLM with chroma block size restriction

Table 2. Summary of experimental results(CCCLM with Restriction vs. VTM2.0.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.38% | -0.32% | 100% | 101% |
| Class A2 | -0.10% | -0.57% | -0.50% | 100% | 103% |
| Class B | -0.08% | -0.54% | -0.68% | 100% | 102% |
| Class C | -0.20% | -0.86% | -1.09% | 100% | 105% |
| Class E | -0.05% | -0.30% | -0.47% | 100% | 100% |
| Overall | -0.13% | -0.55% | -0.65% | 100% | 102% |
| Class D | -0.17% | -0.92% | -1.34% | 101% | 102% |
| Class F (optional) |  |  |  |  |  |

Conclusions

In this contribution, it is proposed to use the reconstructed samples in current block to derive the linear model parameters α and β in CCLM. The proposed method gives -0.14%, -0.57%, and -0.63% BD rate savings on Y, Cb and Cr components for All Intra configuration. With the consideration of block size restriction, -0.13%, -0.55%, and -0.65% BD rate saving can be achieved on Y, Cb and Cr components.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).