| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0115-v2 |
| --- | --- |

| Title: | CE5: Per-context CABAC initialization with double windows (Test 5.1.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Jie Dong, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for Test CE5.1.6. In the current VVC, the CABAC engine uses a “parallel double window” to estimate the binary symbol probabilities, where the window parameters are same for all contexts. Test CE5.1.6 extends the current CABAC engine design by allowing different pairs of window parameters optimized for each context. Under the CE5 test conditions [1], the average coding gains are -0.21% AI, -0.42% RA, and -0.32% LDB. Compared with BMS 2.0.1 VTM configuration, the average gains are -1.12% AI, -0.93% RA, -0.93% LDB.

# Introduction

The CABAC probability estimation method of this test was proposed in JVET-K0380 [2]. Maintaining the original notation, U[k] and V[k] represent the probabilities estimated by the two windows, respectively. They have 15-bit precision, as they are obtained by scaling the real probabilities by 215 and rounding. These probability estimates are updated using the recursive equations, as below.

If the current bin is 1, U[k] and V[k] are updated to U[k+1] and V[k+1], by (1) and (2), respectively.

U[k+1] = U[k] + ((215 – U[k]) >> a)                                                 (1)

V[k+1] = V[k] + ((215 – V[k]) >> b)                                                 (2)

Otherwise, if the current bin is 0, U[k] and V[k] are updated to U[k+1] and V[k+1], by (3) and (4), respectively.

U[k+1] = U[k] – (U[k] >> a)                                                       (3)

V[k+1] = V[k]  – (V[k]) >> b)                                                       (4)

The probability used for binary arithmetic coding, P[k], is the average of U[k] and V[k], as in (5).

P[k] = (U[k] + V[k]) >> 1                                                           (5)

The two positive integer window shift parameters a and b can be different for each context and are store in a look-up table(LUT). In each entry of the LUT, a 4-bit parameter c is stored to represent a and b, as shown in (6) and (7), respectively.

a = (c >> 2) + 2                                                                       (6)

b = a + 1 + (c & 3)                                                                    (7)

Since the value of c for each context are never modified, in custom hardware implementation they can be stored in less expensive and power-efficient read-only memory (ROM).

# Results

The method is integrated into the modified BMS2.0.1 software, called CEM1, where CABACEngine 1 is used, the parameters for double windows are 4 and 7, respectively, and the interval subdivision table size is 512×64. VTM configuration is used.

Table 1 Comparison of CEM1 with the proposed method integrated against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.05% | 0.11% | 95% | 97% |
| Class A2 | -0.37% | 0.05% | -0.18% | 95% | 96% |
| Class B | -0.20% | 0.35% | 0.04% | 94% | 96% |
| Class C | -0.15% | 0.19% | -0.11% | 101% | 101% |
| Class E | -0.18% | 0.22% | -0.31% | 101% | 97% |
| Overall | -0.21% | 0.18% | -0.08% | 97% | 98% |
| Class D | -0.14% | -0.15% | -0.45% | 102% | 101% |
| Class F (optional) | -0.06% | 0.15% | -0.17% | 101% | 99% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.23% | 0.03% | 107% | 98% |
| Class A2 | -0.41% | 0.36% | -0.02% | 107% | 98% |
| Class B | -0.45% | 0.43% | -0.43% | 106% | 96% |
| Class C | -0.45% | 0.21% | -0.45% | 109% | 100% |
| Overall | -0.42% | 0.32% | -0.26% | 107% | 98% |
| Class D | -0.49% | -0.43% | -0.67% | 118% | 99% |
| Class F (optional) | -0.26% | 0.01% | -0.28% | 121% | 101% |
|  |  |  | Low Delay B |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.30% | 0.49% | -0.86% | 100% | 102% |
| Class C | -0.32% | 0.15% | -0.63% | 104% | 93% |
| Class E | -0.37% | 0.25% | -1.67% | 99% | 93% |
| Overall | -0.32% | 0.31% | -0.99% | 101% | 97% |
| Class D | -0.45% | 0.53% | -1.50% | 116% | 100% |
| Class F (optional) | -0.28% | -0.13% | -0.58% | 99% | 91% |

In addition, the performance of the proposed method using an 8×8 table (CE5.2.4) is also tested and compared with CEM1 anchor, as shown in Table 2.

Table 2 Comparison of the proposed method using an 8×8 table against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.01% | 0.14% | 104% | 100% |
| Class A2 | -0.34% | 0.08% | -0.15% | 119% | 107% |
| Class B | -0.16% | 0.39% | 0.08% | 108% | 104% |
| Class C | -0.11% | 0.23% | -0.06% | 124% | 105% |
| Class E | -0.14% | 0.26% | -0.27% | 123% | 104% |
| Overall | -0.17% | 0.21% | -0.04% | 115% | 104% |
| Class D | -0.09% | -0.10% | -0.39% | 125% | 106% |
| Class F (optional) | -0.03% | 0.19% | -0.13% | 122% | 102% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.27% | 0.07% | 126% | 103% |
| Class A2 | -0.37% | 0.39% | 0.01% | 123% | 101% |
| Class B | -0.41% | 0.46% | -0.40% | 122% | 98% |
| Class C | -0.42% | 0.24% | -0.41% | 121% | 101% |
| Overall | -0.39% | 0.35% | -0.23% | 123% | 100% |
| Class D | -0.45% | -0.40% | -0.64% | 110% | 95% |
| Class F (optional) | -0.23% | 0.04% | -0.25% | 118% | 94% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.27% | 0.53% | -0.83% | 107% | 103% |
| Class C | -0.29% | 0.17% | -0.60% | 102% | 107% |
| Class E | -0.32% | 0.29% | -1.62% | 102% | 96% |
| Overall | -0.29% | 0.35% | -0.95% | 104% | 102% |
| Class D | -0.41% | 0.57% | -1.46% | 97% | 98% |
| Class F (optional) | -0.24% | -0.10% | -0.55% | 99% | 96% |

When the anchor is changed to BMS 2.0.1 with VTM configuration, the average coding gains are shown in Tables 3 and 4.

Table 3 Comparison of CEM1 with the proposed method integrated against BMS2.0.1 VTM cfg

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.28% | -1.31% | -1.35% | 111% | 113% |
| Class A2 | -1.01% | -1.31% | -1.50% | 109% | 97% |
| Class B | -1.18% | -1.03% | -1.44% | 88% | 93% |
| Class C | -1.08% | -0.97% | -1.19% | 84% | 94% |
| Class E | -1.02% | -0.84% | -1.51% | 85% | 91% |
| Overall | -1.12% | -1.08% | -1.39% | 93% | 97% |
| Class D | -0.96% | -0.63% | -1.28% | 84% | 96% |
| Class F (optional) | -0.83% | -0.87% | -1.17% | 85% | 96% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.03% | -0.55% | -0.87% | 114% | 112% |
| Class A2 | -0.79% | -0.18% | -0.76% | 105% | 100% |
| Class B | -0.98% | -0.38% | -0.85% | 89% | 94% |
| Class C | -0.90% | -0.37% | -0.50% | 90% | 98% |
| Overall | -0.93% | -0.37% | -0.74% | 97% | 99% |
| Class D | -0.73% | -0.90% | -0.82% | 97% | 93% |
| Class F (optional) | -0.63% | -0.57% | -0.81% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.92% | 0.42% | 0.66% | 104% | 123% |
| Class C | -0.79% | -0.66% | -0.53% | 102% | 93% |
| Class E | -1.13% | 0.01% | -1.80% | 82% | 97% |
| Overall | -0.93% | -0.04% | -0.35% | 98% | 106% |
| Class D | -0.73% | 0.92% | -0.71% | 94% | 94% |
| Class F (optional) | -0.69% | -0.03% | -0.44% | 82% | 92% |

Table 4 Comparison of the proposed method using an 8×8 table against BMS2.0.1 VTM cfg

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.25% | -1.27% | -1.31% | 121% | 117% |
| Class A2 | -0.98% | -1.28% | -1.47% | 136% | 108% |
| Class B | -1.15% | -0.99% | -1.41% | 101% | 100% |
| Class C | -1.04% | -0.93% | -1.15% | 103% | 99% |
| Class E | -0.98% | -0.80% | -1.47% | 103% | 97% |
| Overall | -1.09% | -1.04% | -1.36% | 110% | 103% |
| Class D | -0.91% | -0.58% | -1.23% | 103% | 102% |
| Class F (optional) | -0.80% | -0.84% | -1.13% | 103% | 100% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -0.52% | -0.84% | 133% | 118% |
| Class A2 | -0.75% | -0.15% | -0.73% | 121% | 102% |
| Class B | -0.94% | -0.34% | -0.81% | 102% | 96% |
| Class C | -0.87% | -0.34% | -0.47% | 99% | 99% |
| Overall | -0.89% | -0.34% | -0.71% | 110% | 102% |
| Class D | -0.70% | -0.87% | -0.78% | 90% | 89% |
| Class F (optional) | -0.60% | -0.54% | -0.78% | 98% | 93% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.88% | 0.46% | 0.69% | 111% | 123% |
| Class C | -0.76% | -0.63% | -0.50% | 100% | 107% |
| Class E | -1.09% | 0.06% | -1.75% | 84% | 100% |
| Overall | -0.89% | 0.00% | -0.32% | 100% | 111% |
| Class D | -0.69% | 0.96% | -0.67% | 78% | 92% |
| Class F (optional) | -0.66% | 0.00% | -0.41% | 82% | 97% |

# References

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-K1025, 11th Meeting, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, Y.-H. Chao, M. Karczewicz, V. Seregin, “CE5: Per-context CABAC initialization with double-windows (test A1),” JVET-K0380, 11th Meeting, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).