| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3-12 Oct. 2018 | Document: JVET-L0137-v2 |
| --- | --- |

| Title: | CE1-related: Minimum block size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim, Seung-Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM2.0.1, a CU can be split into 4x4 luma block in both intra and inter slices. A size of minimum block (CU or PU) plays an important role for worst case complexity in aspects of memory bandwidth and throughput in hardware architecture. In addition, the coding performance impact of small block coding is getting less significant but requires higher worst case complexity as input video resolution increases. Therefore, this contribution suggests minimum block size restriction methods as follows:

-Method 1: chroma 2x2 block is restricted in I slice, and luma 4x4 and chroma 2x2 are restricted in P   and B slice.

-Method 2: luma 4x4 block and chroma 2x2 block are restricted in all slices.

The experimental results for Method 1 reportedly show luma BD-rate losses of 0.00%, 0.22% and 0.18% compared to VTM2.0.1 in AI, RA and LB configurations, respectively. The experimental results for Method 1 reportedly show luma BD-rate losses of 0.46%, 0.36% and 0.29% compared to VTM2.0.1 in AI, RA and LB configurations, respectively. In addition, the BD-rate changes for Method 2 were found to be reduced to 0.19%, 0.17% and 0.12% for UHD and HD test sequences.

Introduction

Currently, a CU block in VTM2.0.1 can be split into 4x4 luma and 2x2 chroma block in both intra and inter slices[1], and these small blocks become the cause of data throughput overhead in the hardware architecture, especially when the input video has high resolution (i.e., UHD video or 8K 360° video). And because the structure of VVC is more complicated than the previous codec standards, such as HEVC, it is necessary to reduce the maximum data throughput for the hardware architecture of VVC.

Proposed method

In this proposal, two methods of minimum block restriction as follows;

-Method 1: chroma 2x2 block is restricted in I slice, and luma 4x4 and chroma 2x2 are restricted in P   and B slice.

-Method 2: luma 4x4 block and chroma 2x2 block are restricted in all slices.

In Method 1, the minimum chroma block size is restricted to 2x4 and 4x2 (i.e., chroma block cannot be split into 2x2) to reduce the maximum data throughput in the block structure. To restrict the chroma block size over 2x2 in inter slices, the minimum luma block size should also restricted to 8x4 and 4x8 (i.e., luma block cannot split to 4x4) for the inter slices, and it brings more data throughput reduction in both luma and chroma coding.

In Method 2, the minimum block size restriction for both luma 4x4 block and chroma 2x2 block are applied for all slices so that it is more effective to reduce the maximum data throughput of luma and chroma coding with the reasonable coding loss in the high resolution video coding.

The block restriction information can be inserted into SPS(sequence parameter set) high level syntax, as follows :

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_cu_pixel_num_luma_minus4 | ue(v) |
| log2_min_cu_pixel_num_chroma_minus2 | ue(v) |
| … |  |
| } |  |

log2_min_cu_pixel_num_luma_minus4 specifies the pixel number of minimum luma CU block.

log2_min_cu_pixel_num_chroma_minus2 specifies the pixel number of minimum chroma CU block.

For example, log2_min_cu_pixel_num_luma_minus4 equal to 0 specifies that the pixel number of minimum luma block are 16 (i.e., the minimum luma block size is 4x4), and log2_min_cu_pixel_num_luma_minus4 equal to 1 specifies that the pixel number of minimum luma block are 32 (i.e., the minimum luma block size is 4x8 and 8x4).

And log2_min_cu_pixel_num_ chroma _minus2 equal to 0 specifies that the pixel number of minimum chroma block are 4 (i.e., the minimum chroma block size is 2x2), and log2_min_cu_pixel_num_chroma _minus2 equal to 1 specifies that the pixel number of minimum chroma block are 8 (i.e., the minimum chroma block size is 2x4 and 4x2).

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [1] and Core Experiment description in JVET-K1021 [2]. For the VTM based test, the VTM configuration was enabled.

Table 1 shows the summary result of the method 1 and 2.

|  | Method 1 |  |  | Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | EncT | DecT | Y BD-rate | EncT | DecT |
| AI | 0.00% | 99% | 99% | 0.46% | 88% | 95% |
| RA | 0.22% | 97% | 100% | 0.36% | 95% | 96% |
| LDB | 0.18% | 98% | 107% | 0.29% | 96% | 101% |

f the method 1 and 2od 1 and 2.  2. the proposed methods

In addition, Table 2 shows the summary result of the method 1 and 2 with the UHD and HD test sequences only.

Table 2 Summary result of the method 1 and 2; anchor is VTM2.0.1

|  | Method 1 |  |  | Method 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | EncT | DecT | Y BD-rate | EncT | DecT |
| AI | 0.00% | 99% | 100% | 0.19% | 90% | 95% |
| RA | 0.09% | 98% | 102% | 0.17% | 96% | 98% |
| LDB | 0.10% | 98% | 104% | 0.12% | 96% | 100% |

Test1(Method 1) : chroma 2x2 block is restricted in I slice, luma 4x4 and chroma 2x2 are restricted in P and B slices

In the test 1, chroma 2x2 block is restricted in the I slice and both luma 4x4 and chroma 2x2 are restricted in the P and B slices. Table 3 to Table 5 show the test results in AI, RA, and LDB configuration, and Table 6 to Table 8 show the test result with the high resolution test sequences only (i.e., ClassA1, ClassA2, and ClassB) in AI, RA, and LDB configuration.

Table 3 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.04% | 99% | 100% |
| Class A2 | 0.00% | 0.06% | 0.07% | 99% | 100% |
| Class B | 0.00% | 0.04% | 0.12% | 100% | 99% |
| Class C | -0.01% | 0.32% | 0.57% | 98% | 99% |
| Class E | -0.01% | 0.00% | -0.05% | 100% | 100% |
| Overall | 0.00% | 0.11% | 0.17% | 99% | 99% |
| Class D | 0.01% | 0.64% | 0.66% | 98% | 96% |

Table 4 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.16% | 100% | 102% |
| Class A2 | 0.07% | 0.27% | 0.21% | 98% | 101% |
| Class B | 0.14% | 0.29% | 0.20% | 97% | 101% |
| Class C | 0.57% | 0.98% | 1.23% | 95% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.22% | 0.42% | 0.47% | 97% | 100% |
| Class D | 0.94% | 1.48% | 2.13% | 93% | 91% |

Table 5 Experimental results of Test1 for low-delay B (LDB) test condition; anchor is VTM2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.06% | 0.37% | 96% | 99% |
| Class C | 0.55% | 0.77% | 0.68% | 95% | 100% |
| Class E | -0.18% | 0.28% | -0.90% | 99% | 101% |
| Overall | 0.18% | 0.36% | 0.15% | 96% | 100% |
| Class D | 0.98% | 1.76% | 1.35% | 91% | 96% |

Table 6 Experimental results of Test1 for all-intra (AI) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.04% | 99% | 100% |
| Class A2 | 0.00% | 0.06% | 0.07% | 99% | 100% |
| Class B | 0.00% | 0.04% | 0.12% | 100% | 99% |
| Overall | 0.00% | 0.06% | 0.08% | 99% | 100% |

Table 7 Experimental results of Test1 for random-access (RA) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.16% | 100% | 102% |
| Class A2 | 0.07% | 0.27% | 0.21% | 98% | 101% |
| Class B | 0.14% | 0.29% | 0.20% | 97% | 101% |
| Overall | 0.09% | 0.22% | 0.19% | 98% | 102% |

Table 8 Experimental results of Test1 for low-delay B (LDB) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.10% | 0.06% | 0.37% | 96% | 99% |
| Overall | 0.10% | 0.06% | 0.37% | 96% | 99% |

Test2 (Method 2) : luma 4x4 and chroma 2x2 are restricted in all slices

In the test 2, both luma 4x4 and chroma 2x2 are restricted in the all slices. Table 9 to Table 11 show the test results in AI, RA, and LDB configuration, and Table 12 to Table 14 show the test result with the high resolution test sequences only (i.e., ClassA1, ClassA2, and ClassB) in AI, RA, and LDB configuration.

Table 9 Experimental results of Test2 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.01% | -0.07% | 92% | 97% |
| Class A2 | 0.12% | 0.04% | -0.01% | 90% | 97% |
| Class B | 0.30% | 0.15% | 0.17% | 88% | 93% |
| Class C | 1.08% | 0.71% | 0.90% | 85% | 91% |
| Class E | 0.63% | 0.12% | 0.25% | 91% | 99% |
| Overall | 0.46% | 0.23% | 0.28% | 88% | 95% |
| Class D | 1.42% | 1.27% | 1.03% | 83% | 90% |

Table 10 Experimental results of Test2 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.03% | 0.17% | 98% | 99% |
| Class A2 | 0.15% | 0.02% | 0.04% | 94% | 97% |
| Class B | 0.25% | 0.34% | 0.44% | 95% | 97% |
| Class C | 0.89% | 1.20% | 1.37% | 94% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.36% | 0.45% | 0.55% | 95% | 96% |
| Class D | 1.42% | 1.58% | 2.16% | 91% | 88% |

Table 11 Experimental results of Test2 for low-delay B (LDB) test condition; anchor is VTM2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.03% | 0.26% | 96% | 100% |
| Class C | 0.66% | 0.73% | 0.89% | 94% | 100% |
| Class E | 0.08% | -0.52% | -0.24% | 98% | 102% |
| Overall | 0.29% | 0.10% | 0.34% | 96% | 101% |
| Class D | 1.07% | 1.74% | 1.88% | 91% | 99% |

Table 12 Experimental results of Test2 for all-intra (AI) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.01% | -0.07% | 92% | 97% |
| Class A2 | 0.12% | 0.04% | -0.01% | 90% | 97% |
| Class B | 0.30% | 0.15% | 0.17% | 88% | 93% |
| Overall | 0.19% | 0.08% | 0.06% | 90% | 95% |

Table 13 Experimental results of Test2 for random-access (RA) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.03% | 0.17% | 98% | 99% |
| Class A2 | 0.15% | 0.02% | 0.04% | 94% | 97% |
| Class B | 0.25% | 0.34% | 0.44% | 95% | 97% |
| Overall | 0.17% | 0.17% | 0.26% | 96% | 98% |

Table 14 Experimental results of Test2 for low-delay B (LDB) test condition with Class A1, A2 and B only; anchor is VTM2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.12% | -0.03% | 0.26% | 96% | 100% |
| Overall | 0.12% | -0.03% | 0.26% | 96% | 100% |

Conclusions

To reduce the data throughput overhead of VVC, minimum block size restriction is proposed in the luma and chroma block structure. The proposed method which restricts the 2x2 chroma and 4x4 luma block can reduce the maximum data throughput significantly in the worst cases with reasonable coding loss in the high resolution videos.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- J. Ma, et al., “Description of Core Experiment 1 (CE1): Partitioning” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1021, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).