| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3-12 Oct. 2018 | Document: JVET-L0139-v1 |
| --- | --- |

| Title: | CE3-related : Simplified MDMS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a simplified MDMS. In this method, chroma intra candidate modes are derived with the simplified DM point searching and the reduced number of context models are used in the binarization of chroma intra modes, compared to the JEM MDMS. Experimental results show 0.11%, 1.01%, and 0.99% bit rate savings on Y, Cb, and Cr components respectively, compared to VTM2.0.1 in All Intra configuration, and xx%, xx%, and xx% bit rate savings, compared to VTM2.0.1 in Random Access configuration.

Introduction

The MDMS (multiple DM modes signaling) method was proposed in JVET-D0111 [1]. In the MDMS, the list of chroma mode candidates except CCLM mode includes the following two parts:

Figure 1 Example of corresponding sub-blocks for a chroma CB in I slice.

- intra prediction modes derived from luma CBs covering the co-located five positions of the current chroma block: the five positions to be checked in order are defined as: center (CR), top-left (TL), top-right (TR), bottom-left (BL) and bottom-right (BR) 4x4 block within the corresponding luma block of current chroma block for I slices. For P and B slices, only one of these five sub-blocks is checked since they have the same mode index. An example of five co-located luma positions is shown in Figure 1.

- chroma prediction modes from spatial neighboring blocks to unify the luma and chroma mode derivation process as follows:

  - 5 chroma prediction modes from left, above, below-left, above-right, and above-left spatial neighboring blocks of merge mode

  - Planar and DC modes

  - derived modes are added, those intra modes are obtained by adding -1 or +1 to the angular modes which are already included into the list

  - default modes are added in the order of: Vertical (Mode 18), Horizontal (Mode 50), Mode 2, Mode 34, Mode 66, Mode 10, Mode 26

  - if any of the four default modes (Planar, Horizontal, Vertical and DC modes) is not included in the list, the missing default modes are used to replace the last one or more candidates

Pruning process is applied whenever a new chroma intra mode is added to the candidate list.

In the VTM2, the chroma intra modes are constructed in the same way of HEVC[2], which constructed with the one DM modes and the 4 default modes (i.e. Planar, Vertical, Horizontal, DC, and Mode 66).

Table 1 shows the complexity comparison of the number of pruning check, point access, and context models in the worst case between VTM2 and MDMS.

Table 1 Complexity analysis of VTM2 and MDMS

|  | VTM2 | MDMS |
| --- | --- | --- |
| Number of pruning check | 4 | 40 |
| Number of luma DM point access | 1 | 5 |
| Number of chroma neighbor spatial access | 0 | 5 |
| Number of context models | 1 | 5 |

Proposed method

This contribution proposes a simplified MDMS method, which uses the reduced number of point access and context models. In the proposed method, 5 chroma intra modes are constructed with the 2 DM point (BL, and TR as shown in Figure 1) searching method as follows:

- If BL = TR and BL is Planar or DC 		: BL, !BL, Vertical, Horizontal, and Mode 66

- If BL = TR and BL is not Planar or DC 		: BL, BL-1, BL+1, Planar, DC

- If BL != TR and BL+TR < 2 			: BL, TR, Vertical, Horizontal, and Mode 66

- If BL != TR and BL, TR is not Planar or DC 	: BL, TR, Planar, DC, BL-1

- Else 					: BL, TR, Planar or DC, max(BL,TR)-1, max(BL,TR)+1

Figure 2 shows the flow chart of the proposed method.

Figure 2 Flow chart of the proposed method.

For the simplification of complexity, the context models which are used in the binarization of the chroma intra modes are removed all in this method (i.e., reduced 5 to 0).

Table 2 shows the complexity comparison of the number of pruning (or condition) check, point access, and context models in the worst case of VTM2, MDMS, and proposed method.

Table 2 Complexity analysis of VTM2, MDMS, and proposed method

|  | VTM2 | MDMS | Proposed method |
| --- | --- | --- | --- |
| Number of pruning (or condition) check | 4 | 40 | 6 |
| Number of luma DM point access | 1 | 5 | 2 |
| Number of chroma neighbor spatial access | 0 | 5 | 0 |
| Number of context models | 1 | 5 | 0 |

As shown in Table 2, proposed algorithm is very simple compare to the JEM MDMS, and has slightly increased complexity compare to the VTM2 chroma intra mode generation.

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [3] and Core Experiment description in JVET-K1023 [4]. For the VTM based test, the VTM configuration was enabled.

Table 3 and Table 4 show the test results of proposed method in AI and RA configuration.

Table 1 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.61% | -0.76% | 101% | 101% |
| Class A2 | -0.10% | -0.94% | -0.88% | 101% | 101% |
| Class B | -0.09% | -1.01% | -1.04% | 101% | 103% |
| Class C | -0.09% | -0.89% | -0.73% | 101% | 105% |
| Class E | -0.21% | -1.65% | -1.59% | 101% | 102% |
| Overall | -0.11% | -1.01% | -0.99% | 101% | 103% |
| Class D | -0.10% | -0.54% | -1.03% | 101% | 107% |

Table 2 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Conclusions

To reduce the complexity of the MDMS, simplified MDMS is proposed. In this algorithm, all chroma intra modes are constructed with the two DM point searching, and the reduced number of context models are used in the chroma mode coding. This method can reduce the operation complexity of the MDMS similar to the VTM2 with some coding gains, such as 0.11%, 1.01%, and 0.99% bit rate savings on Y, Cb, and Cr components respectively, compared to VTM2.0.1 in All Intra configuration

Reference

- Y. Chen, et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JVET-J0021, 10th Meeting, San Diego, CA, 10-20 April 2017.

- B. Bross, et al., “Versatile Video Coding (Draft 2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1001, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1021, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).