| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0148-v1 |
| --- | --- |

| Title: | CE10: Multi-Hypothesis Inter Prediction (Tests 1.2.a – 1.2.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Winken, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-570 martin.winken@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results for tests CE10.1.2.a – CE10.1.2.c on multi-hypothesis inter prediction.  The total number of motion-compensated prediction signals (per block) which constitute the overall prediction signal is limited to 4.  The multi-hypothesis inter prediction mode is restricted to be available only for blocks that are larger than 8x8 luma samples.  For test CE10.1.2.a, the ordinary interpolation filters (8-tap for luma and 4-tap for chroma) are also used for the additional hypotheses.  The following results are reported for this test relative to VTM-2.0.1:

RA:	-1.08% (Y),		-0.99% (Cb),	-1.03% (Cr), at 114% encoder and 102% decoder run-time

LB:		-1.09% (Y),		-0.48% (Cb),	-0.50% (Cr), at 122% encoder and 104% decoder run-time

For test CE10.1.2.b, the 4-tap chroma interpolation filter is used for both the luma and the chroma component of the additional hypotheses.  The following results are reported for this test relative to VTM-2.0.1:

RA:	-1.03% (Y),		-1.00% (Cb),	-1.03% (Cr), at 112% encoder and 101% decoder run-time

LB:		-1.00% (Y),		-0.49% (Cb),	-0.41% (Cr), at 119% encoder and 103% decoder run-time

For test CE10.1.2.c, the motion vectors of the additional hypotheses are restricted to full-pel (i.e., there is no interpolation filtering for the additional hypotheses).  The following results are reported for this test relative to VTM-2.0.1:

RA:	-0.96% (Y),		-0.99% (Cb),	-1.08% (Cr), at 109% encoder and 101% decoder run-time

LB:		-1.04% (Y),		-0.71% (Cb),	-0.71% (Cr), at 113% encoder and 102% decoder run-time

# Introduction

In the multi-hypothesis inter prediction mode, one or more additional prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.  With the uni/bi prediction signal puni/bi and the first additional inter prediction signal/hypothesis h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The weighting factor α is specified by the new syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Note that for the additional prediction signals, the concept of prediction list0/list1 is abolished, and instead one combined list is used.  This combined list is generated by alternatingly inserting reference frames from list0 and list1 with increasing reference index, omitting reference frames which have already been inserted, such that double entries are avoided.

Analogously to above, more than one additional prediction signal can be used.  The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n).

Note that also for inter prediction blocks using MERGE mode (but not SKIP mode), additional inter prediction signals can be specified.  Further note, that in case of MERGE, not only the uni/bi prediction parameters, but also the additional prediction parameters of the selected merging candidate are used for the current block, if there are any.

## Multi-Hypothesis Motion Estimation

First, the inter modes with no explicitly signaled additional inter prediction parameters are tested.  For the best of these modes (i.e., the one having lowest RD cost), an additional inter prediction hypothesis is searched.  For that purpose, for all combinations of the following parameters, a motion estimation with a restricted search range of 16 is performed:

- Weighting factor α

- Reference frame for the additional prediction hypothesis

For determining the best combination of these two parameters, a simplified RD cost using Hadamard distortion measure and approximated bit rate is used.  The chosen parameter combination is then used to compute a more accurate RD cost, using forward transform and quantization, which is compared against the so-far best found coding mode for the current block.  If the resulting RD cost is lower than the current best, an additional hypothesis is signaled for the current block.  Note that there is an upper limit for the number of additional hypotheses, such that for blocks using MERGE mode, the resulting total number of merged and explicitly signaled additional hypotheses does not exceed this limit. E.g. in the case of this limit equal to 1, for MERGE blocks where the merging candidate already has one additional inter prediction hypothesis, no further additional hypothesis is tested.  If this limit has not yet been reached for the current block and if the last tested additional hypothesis resulted in a reduced RD cost, the multi-hypothesis estimation process starts all over again, adding one more additional hypothesis also to be tested.

# Description of the tests

For all the three tests CE10.1.2.a, CE10.1.2.b, and CE10.1.2.c, up to 4 motion-compensated prediction signals can be used for all inter-coded blocks that are larger than 8x8 luma samples.  The additional prediction hypotheses always use the translational motion model, also in case of an affine coded block.  In the following, the specifics of the individual tests are described:

- CE10.1.2.a: the additional prediction hypotheses use the ordinary interpolation filters (8-tap for luma, 4-tap for chroma)

- CE10.1.2.b: the additional hypotheses use the 4-tap chroma interpolation filter both for luma and chroma

- CE10.1.2.c: for the additional hypotheses, the motion vectors are always full-pel (i.e., there is no interpolation filtering for the additional hypotheses)

# Software and Settings

The source code was made available to all CE10 participants in the branches CE10.1.2.a, CE10.1.2.b, CE10.1.2.c of the git repository:

https://vcgit.hhi.fraunhofer.de/JVET-K-CE10/VVCSoftware_BMS.git

The encoder can be run using the default config files, all necessary settings are accordingly pre-defined in the corresponding branches.

# Test results

According to the CE10 description in JVET-K1030 [1] and the common test conditions in JVET-K1010 [2], simulations were performed for random-access (RA) and low-delay B (LB) configuration with BMS-2.0.1 software in VTM configuration.  The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test CE10.1.2.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.72% | -1.01% | 110% | 103% |
| Class A2 | -0.66% | -0.99% | -0.96% | 114% | 101% |
| Class B | -1.81% | -1.68% | -1.50% | 116% | 102% |
| Class C | -0.82% | -0.34% | -0.50% | 117% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.08% | -0.99% | -1.03% | 114% | 102% |
| Class D | -0.75% | -0.46% | -0.33% | 120% | 104% |
| Class F (optional) | -1.48% | -1.12% | -1.09% | 118% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.43% | -1.15% | -0.91% | 121% | 104% |
| Class C | -0.85% | -0.37% | -0.67% | 121% | 104% |
| Class E | -0.85% | 0.51% | 0.42% | 127% | 102% |
| Overall | -1.09% | -0.48% | -0.50% | 122% | 104% |
| Class D | -0.71% | -0.17% | -0.16% | 122% | 105% |
| Class F (optional) | -2.85% | -1.79% | -2.02% | 122% | 102% |

## Test CE10.1.2.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.79% | -1.04% | 108% | 100% |
| Class A2 | -0.68% | -1.07% | -1.01% | 111% | 101% |
| Class B | -1.72% | -1.62% | -1.50% | 114% | 101% |
| Class C | -0.71% | -0.34% | -0.44% | 114% | 102% |
| Class E |  |  |  |  |  |
| Overall | -1.03% | -1.00% | -1.03% | 112% | 101% |
| Class D | -0.54% | -0.24% | -0.20% | 117% | 104% |
| Class F (optional) | -1.49% | -1.11% | -1.12% | 114% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.32% | -1.16% | -0.96% | 118% | 103% |
| Class C | -0.70% | -0.20% | -0.22% | 118% | 103% |
| Class E | -0.86% | 0.25% | 0.25% | 123% | 103% |
| Overall | -1.00% | -0.49% | -0.41% | 119% | 103% |
| Class D | -0.55% | 0.17% | -0.45% | 119% | 104% |
| Class F (optional) | -2.91% | -2.29% | -1.84% | 119% | 102% |

## Test CE10.1.2.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.81% | -1.14% | 105% | 101% |
| Class A2 | -0.71% | -1.05% | -1.06% | 108% | 101% |
| Class B | -1.55% | -1.59% | -1.49% | 110% | 100% |
| Class C | -0.61% | -0.31% | -0.52% | 110% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.96% | -0.99% | -1.08% | 109% | 101% |
| Class D | -0.37% | -0.31% | -0.13% | 112% | 104% |
| Class F (optional) | -1.41% | -1.11% | -1.08% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.32% | -1.19% | -0.95% | 112% | 102% |
| Class C | -0.69% | -0.26% | -0.61% | 113% | 103% |
| Class E | -1.04% | -0.52% | -0.44% | 114% | 102% |
| Overall | -1.04% | -0.71% | -0.71% | 113% | 102% |
| Class D | -0.46% | -0.04% | -0.63% | 113% | 102% |
| Class F (optional) | -2.69% | -2.37% | -2.48% | 111% | 101% |

# Computational complexity and memory bandwidth analysis

In this section, the computational complexity of the proposed methods in terms of additions and multiplications as well as the resulting memory bandwidth is studied.  Bit-shifts are not counted as they are considered to be almost cost-free from a hardware implementation perspective.  A similar approach as done in [3] for DMVR is followed.  Note that the worst-case computational complexity and memory bandwidth of the decoder is studied here, not the complexity of the search at the encoder.

## Computational complexity of interpolation filtering

For a prediction block of with w and height h and a K-tap interpolation filter, the following numbers of additions and multiplications are required in the worst case:

Additions:			(K – 1) * w * (2*h + K – 1)

Multiplications:		K * w * (2*h + K – 1)

## Computational complexity of signal superposition for bi-prediction

For bi-prediction, two prediction signals p0 and p1 are equally weighted and superposed in order to obtain the resulting overall prediction signal q, i.e. like this:

q = (p0 + p1 + 1) >> 1

Consequently, for a prediction block of with w and height h the total number of additions amounts to:

Additions:			w * h

(Note that the extra rounding “+ 1” is not separately counted in here, but is instead considered to be part of the specifically designed adder.)

## Computational complexity of signal superposition for weighted multi-hypothesis prediction

For multi-hypothesis prediction, two prediction signals p0 and p1 are unequally weighted and superposed in order to obtain the resulting overall prediction signal q.  As described above, the sets of weights {3/4, 1/4} and {9/8, -1/8} can be used. It is indicated in the bit-stream per additional hypothesis which set of weights is to be used.  For the set {3/4, 1/4}, the superposition can be done without multiplication as follows:

q = (3*p0 + p1 + 2) >> 2

= (p0 + (p0<<1) + p1 + 2) >> 2

Similarly, for the set {9/8, -1/8}, the superposition can be done without multiplication as follows:

q = (9*p0 - p1 + 4) >> 3

= (p0 + (p0<<3) - p1 + 4) >> 3

Consequently, in both cases, for a prediction block of with w and height h the total number of additions amounts to:

Additions:			2 * w * h

(Again, the rounding “+ 2” and “+ 4” are not separately counted, but are instead considered to be part of the specifically designed adder.  Furthermore, the subtraction is also counted as one specifically designed adder.)

## Memory bandwidth for interpolation filtering

For a prediction block of with w and height h and a K-tap interpolation filter, the following number of reference samples has to be accessed in the worst cases:

(w + K – 1) * (h + K – 1)

## Worst-case computational complexity and memory bandwidth

In the following table, the worst-case computational complexity and memory bandwidth with respect to (w.r.t.) 4x4 and 8x8 bi-prediction is presented for the tests CE10.1.2.a – CE10.1.2.c.  For a detailed derivation, reference is made to the accompanying Excel file.  (In the Excel file, the computational complexity is derived as the number of multiplications per luma sample where an addition is counted as 0.25 multiplications, as is done in [3]).  Note that multi-hypothesis prediction is disabled for

| Test | worst-case computational complexity w.r.t. 4x4 bi | worst-case memory bandwidth w.r.t. 4x4 bi | worst-case computational complexity w.r.t. 8x8 bi | worst-case memory bandwidth w.r.t. 8x8 bi |
| --- | --- | --- | --- | --- |
| CE10.1.2.a (8-tap IF for multi-hypothesis prediction) | 155% | 73% | 202% | 154% |
| CE10.1.2.b (4-tap IF for multi-hypothesis prediction) | 118% | 63% | 153% | 133% |
| CE10.1.2.c (only full-pel for multi-hypothesis prediction) | 79% | 51% | 102% | 107% |

# Syntax changes

The changes to the coding unit syntax structure are shown below:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] |  |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) { |  |
| i = 0 |  |
| readMore = 1 |  |
| while( i < MaxNumAdditionalHypotheses && readMore ) { |  |
| additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( additional_hypothesis_flag[ x0 ][ y0 ][ i ] ) { |  |
| ref_idx_add_hyp[ x0 ][ y0 ][ i ] | ae(v) |
| mvd_coding( x0, y0, 2+I, 0 ) |  |
| mvp_add_hyp_flag[ x0 ][ y0 ][ i ] | ae(v) |
| add_hyp_weight_idx[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| readMore = additional_hypothesis_flag[ x0 ][ y0 ][ i ] |  |
| i++ |  |
| } |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

additional_hypothesis_flag[ x0 ][ y0 ][ i ] equal to 1 specifies that the i-th explicit additional inter prediction hypothesis is signaled for this CU.

ref_idx_add_hyp[ x0 ][ y0 ][ i ] specifies the reference frame to be used for the i-th explicit additional inter prediction hypothesis.

mvp_add_hyp_flag [ x0 ][ y0 ][ i ] specifies the motion vector predictor index to be used for the i-th explicit additional inter prediction hypothesis.

add_hyp_weight_idx[ x0 ][ y0 ][ i ] specifies the weighting factor αi to be used for the i-th explicit additional inter prediction hypothesis, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

# Conclusion

This contribution reports results for tests CE10.1.2.a – CE10.1.2.c on multi-hypothesis inter prediction with up to 4 motion-compensated prediction signal per block.

For test CE10.1.2.a, the ordinary interpolation filters (8-tap for luma and 4-tap for chroma) are also used for the additional hypotheses.  The following results are reported for this test relative to VTM-2.0.1:

RA:	-1.08% (Y),		-0.99% (Cb),	-1.03% (Cr), at 114% encoder and 102% decoder run-time

LB:		-1.09% (Y),		-0.48% (Cb),	-0.50% (Cr), at 122% encoder and 104% decoder run-time

For test CE10.1.2.b, the 4-tap chroma interpolation filter is used for both the luma and the chroma component of the additional hypotheses.  The following results are reported for this test relative to VTM-2.0.1:

RA:	-1.03% (Y),		-1.00% (Cb),	-1.03% (Cr), at 112% encoder and 101% decoder run-time

LB:		-1.00% (Y),		-0.49% (Cb),	-0.41% (Cr), at 119% encoder and 103% decoder run-time

For test CE10.1.2.c, the motion vectors of the additional hypotheses are restricted to full-pel (i.e., there is no interpolation filtering for the additional hypotheses).  The following results are reported for this test relative to VTM-2.0.1:

RA:	-0.96% (Y),		-0.99% (Cb),	-1.08% (Cr), at 109% encoder and 101% decoder run-time

LB:		-1.04% (Y),		-0.71% (Cb),	-0.71% (Cr), at 113% encoder and 102% decoder run-time

# References

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-K1030, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, July 2018.

- M. Zhou, B. Heng, “Non-CE9: A computational complexity analysis for DMVR,” JVET-K0480 Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Fraunhofer may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).