| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0157 |
| --- | --- |

| Title: | CE10: Uniform Directional Diffusion Filters For Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Anastasia Henkel, Jonathan Pfaff, Michael Schäfer, Anastasia Henkel, Heiko Schwarz, Mischa Siekmann, Martin Winken, Philipp Helle, Detlev Marpe, Thomas Wiegand Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de, anastasia.henkel@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results on the tool under test in CE10 (JVET-Jxxxx) for studying the performance of uniform directional diffusion filters on top of VTM and BMS.

In this document, diffusion filters which were introduced in [2],[3] are modified in two ways: First, the filter coefficients are not signal adaptive. Thus, the diffusion filters are implemented by applying a fixed filter mask to the prediction signal. Second, the number of iterations is reduced and it is also explained that whether or not implementing the filter iteratively is merely a choice of implementation but does not affect the coding efficiency of the tool. A detailed description of the filter masks as well as an analysis of the maximal number of operations (additions/bit-shifts/multiplications) per pixel are provided.

# Proposed method: Decoder algorithm description

Uniform Directional Diffusion Filters have been integrated to the test model according to the description of sub-tests CE10.4.1 [1].

## Uniform Diffusion Filter

Let pred ∈RMxN be the prediction signal on a given block obtained by intra or motion compensated prediction. Let predext∈R(M+1)x(N+1) denote its extended prediction where at the top and left boundary the reconstructed samples are used and on the right and bottom reflected boundary conditions are used. If in inter case, it is not desired to use the reconstructed samples of the current frame, the boundary condition can be replaced by using reflected samples here as well (CE 10.4.1). In this case, only the prediction signal itself comprises the input for the Diffusion Filter.

prediction block

reconstructed samples

reflected samples

prediction block

reconstructed samples

reflected samples

Figure 1 Extended prediction signal: the blue boundary denotes the boundary of reconstructed samples, the white block is the given prediction signal, the red boundary are the reflected samples.

As has been described in [2],[3] for inner points p the uniform diffusion filter is realized applying the filter mask

h =    14 010101010

to predext. To realize two iterations of the applying filter mask h in one step, the following is done: Considering the boundary conditions above, the starting point up,0=predext  for the iterations is defined on

{ x1,x2∈Z2 | -1<x1≤N, -1 < x2≤M }

For an inner point p = x1,x2 with 1 ≤ x1≤N-2  and  1≤ x2≤M-2 one has that

up,2= h2* up,0

where h2 denotes the convolution of mask h with itsself and * the application of the filter mask on image up,0 in point p. If the position of p is given by

0000000000 00p000000000000.

The filter mask h2is given by

h2=hinner2=0.252 0010002020104010202000100.

For the boundary values special cases have to be considered.

If point p is the top left corner point of the prediction block (i.e. x1=0, x2=0), the filter mask h2  is given by

htopleft2=0.252 0410420110200100,

where the position of p (correlating to the image mask) is given by

00000p0000000000.

If point p is on the left boundary of the prediction block (i.e. x1=0, 0< x2<M-1), the filter mask is given by

hleftBdry2=0.252 01001020430110200100,

where the position of p (correlating to the image mask) is given by

000000000p0000000000.

If point p is situated on the top border of the prediction block (i.e. 0< x1<N-1, x2=0), the corresponding image mask is the transposed of hleftBdry2.

If point p is the left bottom corner point of the prediction block (i.e. x1=0, x2=M-1), considering Dirichlet boundary conditions on the left side and Neumann boundary conditions on the bottom where the second last row is mirrored and used as a boundary, the filter mask is given by

hbottomLeft2=0.252 010014304201,

where the position of p (correlating to the image mask) is given by

000000000p00.

If point p is the top right corner point of the prediction block (i.e. x1=N-1, x2=0), the corresponding image mask is the transposed of hbottomLeft2

If point p is situated on the right border of the prediction block (i.e. x1=N-1, 0< x2<M-1 the filter mask is given by

hrightBdry2=0.252 001030143030001

considering the Neumann boundary condition where the second last column of the prediction block is mirrored and used as a boundary. Here, the position of p (correlating to the image mask) is given by

00000000p000000.

If point p is situated on the lower border of the prediction block (i.e. 0< x1<N-1,  x2=M-1), the filter mask is the transposed of hrightBdry2.

If point p is the bottom right corner point of the prediction block (i.e. x1=N-1, x2=M-1), the filter mask is given by

hBTcorner2=0.252 0010211110,

where the position of p (correlating to the image mask) is given by

00000000p.

## Directional Diffusion Filter

Instead of using signal adaptive diffusion filters as has been done in [2],[3] previously, here directional filters are used which still have a fixed mask. More precisely, the uniform diffusion filtering of the previous section is simply restricted to either be applied only along the horizontal direction or along the vertical direction. The horizontal filter is realized using the filter mask

h =    12 000101000

and the vertical filter accordingly using

h =    12 010000010 .

As has been done in the section above, two iteration steps of applying these directional filter masks can be implemented in one step. For an inner point p = x1,x2 with 0 ≤ x1≤N-1  and  1≤ x2≤M-2 one has that two iteration steps using the vertical filter result in filter mask

hinner2=0.52 10201,

where the position of p (correlating to the image mask) is given by

00p00.

If point p is in the top border, i.e. 0 ≤ x1≤N-1  and   x2=0, the filter mask is given by

htopBorder2=0.52 2101,

where the position of p (correlating to the image mask) is given by

0p00.

If point p is in the bottom border, i.e. 0 ≤ x1≤N-1  and   x2=M-1, the filter mask is given by

hbottomBorder2=0.52 121,

where the position of p (correlating to the image mask) is given by

00p.

For the horizontal filter this is done analogously using transposed filter masks.

# Implementation

The filtering is implemented in a way such that symmetries within the filter-masks are exploited. The multiplications that are factors of two are implemented using bitshifts. Using the method described above, the iteration numbers of the uniform and directional diffusion filters can be reduced and result in filters depicted in the table below.

| Cu_diffusion_filter_idx | filter type | Number of iterations n |  |
| --- | --- | --- | --- |
|  |  | intra | Inter |
| 0 | no filter | - | - |
| 1 | uniform | 2 | 2 |
| 2 | uniform | 8 | 8 |
| 3 | directional horizontal | 2 | 2 |
| 4 | directional vertical | 2 | 2 |

Table Interpretation of the syntax element diffFilterIdx

Generalizing the methods described above, one can summarize four iterations to the correlation with a fixed mask. Then, the matrix for the inner points is given by

h4=hinner4=0.254000010000000404000006016060004024024040101603601601040240240400060160600000404000000010000.

Accordingly, the table above would look as follows

| Cu_diffusion_filter_idx | filter type | Number of iterations n |  |
| --- | --- | --- | --- |
|  |  | intra | inter |
| 0 | no filter | - | - |
| 1 | uniform | 1 | 1 |
| 2 | uniform | 4 | 4 |
| 3 | directional horizontal | 1 | 1 |
| 4 | directional vertical | 1 | 1 |

Table  Interpretation of the syntax element diffFilterIdx

Thus, a maximum number of 4 iterations, i.e. filtering two times with the filter mask hinner4 is needed.

# Complexity

For a general MxN prediction block the uniform diffusion filter needs

8 additions + 3 bitshifts + (M-2)*(9 additions +3 bitshifts + 1 multiplication) + (N-2)*(9 additions +3 bitshifts + 1 multiplication) + 2*(7 additions +3 bitshifts + 1 multiplication) + (M-2)*(7 additions +2 bitshifts + 3 multiplications) + (N-2)*(7 additions +2 bitshifts + 3 multiplications) + 6 additions +2 bitshifts + 1 multiplication + (N-2)*(M-2)(9 additions +3 bitshifts)

which results in

28 + (M+N-4)*16 + (N-2)*(M-2)*9 additions

11 + (M+N-4)*5 + (N-2)*(M-2)*3 bitshifts

3+(M+N-4)*4 multiplications

per iteration step. 	
It should be noted here that the filter is restricted to larger blocks as depicted in the following table.

| BlkSizes | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 16 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 32 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 64 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 128 | 0 | 0 | 0 | 0 | 0 | 0,1,2,3,4 |

Table  Restricted number of tested Filters ordered by block sizes, width versus height

The maximal complexity per pixel in terms of number of additions, number of bit-shifts and number of multiplications to produce the overall prediction signal when applying the diffusion filter is depicted for different block sizes below.

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 17 | 17.25 | 17.37 | 17.44 | - |
| 16 | 17.25 | 70 | 70.5 | 70.75 | - |
| 32 | 17.37 | 70.5 | 71 | 71.25 | - |
| 64 | 17.44 | 70.75 | 71.25 | 71.5 | - |
| 128 | - | - | - | - | 71.75 |

Table  Maximal number of additions for different block sizes (case of  8 iterations if allowed)

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 5.59 | 5.67 | 5.71 | 5.73 | - |
| 16 | 5.67 | 23.09 | 23.30 | 23.40 | - |
| 32 | 5.71 | 23.30 | 23.52 | 23.64 | - |
| 64 | 5.73 | 23.40 | 23.64 | 23.76 | - |
| 128 | - | - | - | - | 23.88 |

Table  Maximal number of bit-shifts for different block sizes (case of  8 iterations if allowed)

| BlkSizes | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- |
| 8 | 1.59 | 1.29 | 1.15 | 1.07 | - |
| 16 | 1.29 | 3.59 | 2.80 | 2.40 | - |
| 32 | 1.15 | 2.80 | 1.90 | 1.45 | - |
| 64 | 1.07 | 2.40 | 1.45 | 0.97 | - |
| 128 | - | - | - | - | 0.49 |

Table  Maximal number of multiplications for different block sizes (case of  8 iterations if allowed)

The fractional numbers come frome the fact that at the boundary of the block, slightly different masks are used than at the interior.

# Results

According to the CE10 description in JVET-K1030 [1] and the common test conditions in JVET-K1010 [4], the proposed method is evaluated for intra-only, random-access and low delay configuration with BMS-1.0 software in VTM and BMS configuration.

In the following table an overview over the scheduled CE tests with their corresponding config parameters is shown.

|  | Test |  |
| --- | --- | --- |
| CE10.4.1 | All Intra, Random Access and Low Delay, VTM +BMS - No neighboring block samples used for inter blocks | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 3 NMBDRYDiffusion : 1 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |
| CE10.4.2 | Random Access and Low Delay, VTM +BMS - Using reconstructed samples for inter | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 3 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |

## CE10.4.1

In case there are no neighboring samples used, the diffusion filtering obtains the following results.

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.80% | -0.54% |  | -0.49% |  |
| Class A2 | -0.44% | -0.25% |  | -0.24% |  |
| Class B | -0.28% | -0.21% |  | -0.12% |  |
| Class C | -0.12% | -0.16% |  | 0.03% |  |
| Class E | -0.23% | -0.04% |  | -0.10% |  |
| Overall | -0.35% | -0.23% |  | -0.17% |  |
| Enc Time [%] | 152% |  |  |  |  |
| Dec Time [%] | 102% |  |  |  |  |
| Class D | -0.09% |  | 0.08% |  | -0.34% |
| Class F | -0.05% |  | -0.12% |  | 0.06% |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.70% | -0.99% |  | -0.82% |  |
| Class A2 | -0.46% | -0.89% |  | -0.69% |  |
| Class B | -0.66% | -1.00% |  | -1.05% |  |
| Class C | -0.17% | -0.32% |  | -0.31% |  |
| Overall | -0.50% | -0.79% |  | -0.73% |  |
| Enc Time [%] | 125% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.08% |  | -0.46% |  | 0.12% |
| Class F | -0.11% |  | -0.20% |  | 0.06% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.25% | -0.14% |  | -0.28% |  |
| Class C | -0.13% | 0.15% |  | 0.14% |  |
| Class E | -0.13% | 0.51% |  | 0.36% |  |
| Overall | -0.18% | 0.12% |  | 0.02% |  |
| Enc Time [%] | 123% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.02% |  | 1.00% |  | 0.08% |
| Class F | -0.09% |  | 0.59% |  | 0.07% |

|  | BMS, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.78% | -0.57% |  | -0.36% |  |
| Class A2 | -0.42% | -0.30% |  | -0.23% |  |
| Class B | -0.28% | -0.23% |  | -0.22% |  |
| Class C | -0.09% | -0.01% |  | -0.05% |  |
| Class E | -0.20% | -0.20% |  | -0.08% |  |
| Overall | -0.33% | -0.24% |  | -0.18% |  |
| Enc Time [%] | 160% |  |  |  |  |
| Dec Time [%] | 102% |  |  |  |  |
| Class D | -0.06% |  | -0.05% |  | -0.10% |
| Class F | -0.03% |  | -0.01% |  | -0.08% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.64% | -0.97% |  | -0.73% |  |
| Class A2 | -0.51% | -0.89% |  | -0.63% |  |
| Class B | -0.66% | -0.85% |  | -1.12% |  |
| Class C | -0.18% | -0.35% |  | -0.24% |  |
| Overall | -0.50% | -0.75% |  | -0.71% |  |
| Enc Time [%] | 135% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.13% |  | 0.11% |  | -0.28% |
| Class F | -0.08% |  | -0.16% |  | -0.14% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.28% | -0.01% |  | -0.15% |  |
| Class C | -0.20% | 0.14% |  | 0.18% |  |
| Class E | -0.05% | -0.07% |  | 0.46% |  |
| Overall | -0.19% | 0.03% |  | 0.11% |  |
| Enc Time [%] | 134% |  |  |  |  |
| Dec Time [%] | 99% |  |  |  |  |
| Class D | -0.03% |  | 0.67% |  | 1.07% |
| Class F | -0.14% |  | 0.44% |  | -0.24% |

## CE10.4.2

If in inter case the reconstructed samples left and above are used, the diffusion filtering obtains the following results.

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.71% | -1.16% |  | -0.90% |  |
| Class A2 | -0.54% | -1.20% |  | -0.80% |  |
| Class B | -0.77% | -1.28% |  | -1.34% |  |
| Class C | -0.27% | -0.41% |  | -0.36% |  |
| Overall | -0.58% | -1.01% |  | -0.88% |  |
| Enc Time [%] | 125% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.17% |  | -0.42% |  | 0.09% |
| Class F | -0.17% |  | -0.30% |  | -0.03% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.52% | 0.00% |  | -0.07% |  |
| Class C | -0.25% | -0.01% |  | 0.08% |  |
| Class E | -0.32% | 0.57% |  | 0.24% |  |
| Overall | -0.38% | 0.14% |  | 0.06% |  |
| Enc Time [%] | 122% |  |  |  |  |
| Dec Time [%] | 100% |  |  |  |  |
| Class D | -0.12% |  | 0.75% |  | 0.13% |
| Class F | -0.23% |  | 0.34% |  | -0.09% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.70% | -1.27% |  | -0.72% |  |
| Class A2 | -0.61% | -1.10% |  | -0.83% |  |
| Class B | -0.87% | -1.04% |  | -1.41% |  |
| Class C | -0.36% | -0.25% |  | -0.34% |  |
| Overall | -0.65% | -0.89% |  | -0.87% |  |
| Enc Time [%] | 133% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.20% |  | 0.18% |  | -0.01% |
| Class F | -0.12% |  | -0.19% |  | -0.17% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.58% | 0.07% |  | 0.16% |  |
| Class C | -0.36% | -0.07% |  | 0.26% |  |
| Class E | -0.29% | 0.28% |  | 0.85% |  |
| Overall | -0.43% | 0.08% |  | 0.37% |  |
| Enc Time [%] | 133% |  |  |  |  |
| Dec Time [%] | 100% |  |  |  |  |
| Class D | -0.18% |  | 0.35% |  | 0.51% |
| Class F | -0.34% |  | -0.44% |  | -0.58% |

# References

- C.W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” JVET-K1030, Ljubljana, SI, 10–18 July 2018.

- J. Pfaff et al., “Signal Adaptive Diffusion Filters For Video Coding,” JVET-J0038, San Diego, USA, April 2018.

- J. Rasch et al., “CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5)” JVET-K323, Ljubljana, SI, 10–18 July 2018.

- F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-K1010, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).