| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0161 |
| --- | --- |

| Title: | AHG14: Normative Intra Refresh Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jean-Marc Thiesse, Didier Nicholson, David Gommelet VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 jean-marc.thiesse@vitec.com didier.nicholson@vitec.com david.gommelet@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution studies the integration of a normative handling of Intra Refresh into VVC. This is an answer to the last mandate of the AhG14 on Low-latency random access. The proposed modification includes the signalization of the Intra Refresh CUs thanks to a simple syntax element addition at PPS and Slice header level. Those CUs are then encoded according to this knowledge (non-transmission of prediction mode, activation of CIP only on those CUs and disabling of deblocking filters at the Intra Refresh boundary).

The reported performance against the Intra Refresh anchor proposed in JVET-L0160 shows a significant improvement of 6% in average on Luma component and 8% in Chroma component.

# Introduction

This contribution is based on the contribution JVET-K0212 [1] which proposed to handle Intra Refresh feature in a normative way. Intra Refresh is a classical feature used by encoder manufacturer for achieving low-latency and ultra-low-latency: instead of using big Intra picture which requires defining a large buffer at the decoder side which significantly increases the latency, Intra blocks are progressively introduced in Inter picture. Then, the consumption size of each picture remains quite small and comparable, allowing the reduction of the latency while maintaining the random accessibility of the stream. As studied in [2], the way this is handled by encoder manufacturer is a hack associated to strong losses. This can be significantly improved thanks to a straightforward modification as proposed in this contribution.

It is also important to notice that the issue of latency increase due to big Intra picture grows with the standard generation increment since the improvement of Intra prediction is smaller than the Inter. This was observed when moving from AVC to HEVC and it is expected to be even more critical with VVC.

# Proposed Improved Intra Refresh

## Description of the method

The proposed solution adds the minimum signalization needed to determinate the position of Intra Refreshed pixels and then exploits this position knowledge to improve the quality within this low-latency use-case. The modifications at the encoder side involved:

- Avoid the transmission of pred_mode_flag for Coding Unit with at least one pixel within the Intra Refresh Area.

- Enable the Constrained Intra Prediction (CIP) tool adaptively depending on the position and direction of the Intra Refresh Area.

- Disabling of Deblocking Filter at the Intra Refresh boundary.

Other possible modifications (not tested yet), as also suggested in [3], include:

- Take care of the Intra Refresh position for the motion vectors derivation.

- Define two deltas QP by slice depending on the Intra Refresh Area position.

## Signalization

The proposed additional signalization is:

- at PPS level:

  - intra_refresh_enabled_flag specifies if the Intra Refresh tool is enabled.

  - intra_refresh_mode specifies the mode of Intra Refresh (Column, Line, pseudo-random, …)

  - intra_refresh_size  specifies the size of the refresh pattern (width of the Column, height of the Line, width x height for block-based refresh). This can be signaled as a multiple of the CU min size (0 for 8 pixels, 1 for 16 pixels, …).

  - intra_refresh_delta_qp specifies a delta QP for the Intra Refresh blocks. This allows to optimize the encoder quality balancing between Intra Refresh blocks and none.

- at Slice level:

  - intra_refresh_direction specifies the direction of the refresh (right to left/left to right for Column, top to bottom/bottom to top for Line, …). This is used to determine motion vectors constraints during the encoding/decoding.

  - intra_refresh_position specifies the position of the Intra Refresh blocks, this is given according to intra_refresh_size.

The corresponding draft modifications are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| intra_refresh_enabled_flag | u(1) |
| if( intra_refresh_enabled_flag ){ |  |
| intra_refresh_mode | ue(v) |
| intra_refresh_size | ue(v) |
| intra_refresh_delta_qp | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if (intra_refresh_enabled_flag ){ |  |
| intra_refresh_direction | u(1) |
| intra_refresh_position | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if( slice_type != I && !intra_refresh_cu) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| … |  |

….

CuIntraRefresh[ xNbY ][ yNbY ] is derived at frame level according to intra_refresh_position value. CuIntraRefresh[ xNbY ][ yNbY ] is equal to 1 if the CU belongs to the Intra Refresh area.

….

- Each sample p[ x ][ y ] is derived as follows:

– If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

– The variable availableN is equal to FALSE.

– CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

– CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and CuIntraRefresh[ xNbY ][ yNbY ]  is equal to 1.

– Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

Figure 1: Proposed signalization and modification.

# Results

The proposed normative Intra Refresh has been tested against the reference as proposed in contribution JVET-L0160 [2] with encoder-only support of Intra Refresh. Both tests have been run using an encoding configuration based on Low-Delay B configuration with an Intra Refresh period of 32.

|  | Low delay B Intra Refresh Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class C | -5,01% | -7,76% | -8,68% | 93% | #DIV/0! |
| Class E | -7,80% | -8,05% | -9,17% | 88% | #DIV/0! |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | -5,17% | -7,46% | -7,74% | 89% | #DIV/0! |
| Class F (optional) | -7,81% | -9,88% | -10,20% | 90% | #DIV/0! |

Table 1 Normative Intra Refresh against Encoder-only Intra Refresh [2].

As reported in Table 1, the proposed modifications provide a significant improvement of more than 6% in average on Luma and 8% in Chroma. The coding efficiency is more improved for simple sequence with small motion thanks to the better handling of Intra Refresh constraints, up to 11% on SlideEditing sequence and 8% on Johnny sequence.

# Conclusion

A normative handling of Intra Refresh feature has been proposed in this document. Basic signalization is added on headers for deriving the position of Intra Refreshed CUs which are then encoded according to this knowledge (non-transmission of prediction mode, activation of CIP only on those CUs and disabling of deblocking filters at the Intra Refresh boundary). A significant improvement has been reported against encoder-only Intra Refresh with an average improvement of 6% on Luma Component.

As reported in [3], there are several other possible improvements allowed with such a signalization. Therefore it is suggested to further study the integration of a normative handling of Intra Refresh into VVC in order to provide this key low-latency feature at a cost of minor coding efficiency loss.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2]	J.-M. Thiesse, D. Nicholson, D. Gommelet, “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

[3] K. Kazui, “AHG14: Study of methods for progressive intra refresh“,JVET-L0079, October 2018.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).