| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0168 |
| --- | --- |

| Title: | Motion vector representing bit reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution presents a bit control method for motion vector representing to reduce the memory usage of the motion vector. In the last JVET meeting, affine prediction with 1/16-pel mv precision was adopted. It means that it is required to store the motion vector of 18-bits in VTM software. In the proposed method, motion vector representing bit is reduced to 16-bits when storing the motion vector which is used as a candidate for TMVP or ATMVP. Additionally, the proposed method is also applied to the spatial neighboring motion vector. The simulation results reportedly show no noticeable BD-rate changes on both of temporal and spatial motion vector bit reduction.

Proposed method

From the proposed method, temporal motion vector i.e. ATMVP and TMVP is represented to 16-bits. Additionally, this method is also applied on spatial motion vector. The proposed motion vector representing bit reduction is only applied to motion vector storing process while motion compensation uses a conventional 1/16-pel motion vector with 18-bit representation.

Remove LSB bit

The proposed LSB removing scheme is described in Figure 1. From this method, a derived mv from memory should be 1/4-pel precision.

offset = 1 << ( 2 − 1 ) mvX[ 0 ] = ( mvX[ 0 ] >= 0 ? ( mvX[ 0 ] + offset) >> 2 : − ( (− mvX[ 0 ] + offset ) >>2 ) ) << 2 mvX[ 1 ] = ( mvX[ 1 ] >= 0 ? ( mvX[ 1 ] + offset) >> 2 : − ( (− mvX[ 1 ] + offset ) >> 2 ) ) << 2

Figure 1

Remove MSB bit

The proposed MSB removing scheme is described in Figure X. a derived mv has 1/16-pel precision with 16-bits mv representation while a mv has 1/16-pel precision with 18-bits mv representation in the current VTM.

mvX[0] = Clip3( -32768, 32767, mvX[0]) mvX[1] = Clip3( -32768, 32767, mvX[1])

Figure 2

Remove LSB/MSB adaptively

The proposed LSB/MSB adaptive removing scheme is represented in Figure X. MSB bits removing method proposed in 1.2 causes restriction of mv representing range. To overcome this restriction, adaptive MSB/LSB removing method is proposed. From this method, a derived mv can have 1/4 or 1/16 precision adaptively using 16-bits representation.

mvX[i] = (mvX[i] < -32768 ¦¦ mvX[i] > 32767) ? (Remove LSB) : (mvX[i])

Figure 3

Experimental results

|  | Proposed removing method | Target storage |
| --- | --- | --- |
| Test1.1 | LSB reduction | For Temporal MV storage |
| Test1.2 | MSB reduction | For Temporal MV storage |
| Test1.3 | Adoptive LSB/MSB reduction | For Temporal MV storage |
| Test2.1 | LSB reduction, temp | For Temporal/Spatial MV storage |
| Test2.2 | MSB reduction | For Temporal/Spatial MV storage |
| Test2.3 | Adoptive LSB/MSB reduction | For Temporal/Spatial MV storage |

|  | Over VTM2.0.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1.1 |  |  |  |  | Test 2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.06% | 101% | 102% |  |  |  |  |  |
| Class A2 | 0.03% | 0.02% | -0.06% | 99% | 100% |  |  |  |  |  |
| Class B | 0.04% | 0.06% | 0.05% | 100% | 101% | 1.62% | 1.27% | 1.21% | 99% | 97% |
| Class C | 0.10% | 0.12% | 0.03% | 99% | 97% | 0.91% | 0.69% | 0.61% | 99% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.04% | 0.04% | 0.02% | 100% | 100% |  |  |  |  |  |
| Class D | 0.21% | 0.20% | 0.05% | 99% | 83% | 1.33% | 0.83% | 0.94% | 99% | 83% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! |

|  | Over VTM2.0.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1.2 |  |  |  |  | Test 2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 102% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 84% | 0.00% | 0.00% | 0.00% | 98% | 83% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! |

|  | Over VTM2.0.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1.3 |  |  |  |  | Test 2.3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 95% | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 82% | 0.00% | 0.00% | 0.00% | 98% | 83% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! | #VALUE! | #VALUE! | #VALUE! | #REF! | #REF! |

Conclusion

This contribution proposes to reduce mv representing bit from 18-bits to 16-bits for motion vector storage. The simulation results reportedly show no noticeable BD-rate change on CTC configuration for temporal and spatial motion vector representation. It is recommended to consider this proposal for the next version of VTM.

Reference

- H. Jang, J. Lim, J. Nam, S. Kim, “Non-CE: Temporal motion vector data storage reduction,” JVET-K0086, Ljubljana, SI, July 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-K1024, Ljubljana, SI, July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).