| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0169 |
| --- | --- |

| Title: | CE4.8.1 Temporal motion data storage reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This proposal presents a motion data storage reduction method for temporal prediction.. Current VVC apply 8x8 grid motion data storage compression for temporal motion data. In this contribution, more temporal motion data memory reduction by a factor of 4 times is proposed with additional candidate positions compared with current VTM software. This motion data storage reduction scheme is proposed as JVET-K0086[1] and three methods are testedas described in Table 1. The experimental results for Test #4.8.1 reportedly show no bit rate changes. The experimental results on Test #4.8.2 reportedly show 0.29% BD-rate increase with 4 times temporal memory reduction. Test #4.8.3 is represented as Test #4.8.2 with proposed additional spatial candidate and it reportedly shows 0.17% BD-rate increase with 4 times temporal memory reduction also. From the proposed spatial candidate based motion data compression method, it is shown that 0.12% BD-rate saving compared with the conventional motion data compression method which uses only top-left spatial candidate when the compression grid is 16x16.

Table 1

| Test # | Description |
| --- | --- |
| 4.8.1 | Motion information storage with alternative spatial candidates for each 8x8 region |
| 4.8.2 | Motion information storage for each 16x16 region |
| 4.8.3 | Motion information storage with alternative spatial candidates for each 16x16 region |

The alternative spatial candidates are depicted in Figure 1.

Figure 1

Experimental results

CE4.8.1 Motion information storage with alternative spatial candidates for each 8x8 region

Table 2 Random access configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.01% | 100% | 102% |
| Class A2 | 0.02% | -0.02% | -0.12% | 99% | 100% |
| Class B | 0.00% | 0.06% | -0.04% | 99% | 101% |
| Class C | -0.01% | 0.10% | 0.02% | 99% | 96% |
| Overall | 0.00% | 0.04% | -0.03% | 98% | 82% |
| Class D | 0.06% | 0.12% | 0.09% | 99% | 100% |

Table 3 Low delay configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.02% | -0.07% | 0.10% | 99% | 99% |
| Class C | 0.06% | -0.02% | 0.04% | 98% | 97% |
| Class D | 0.02% | 0.13% | -0.47% | 99% | 89% |
| Class E | -0.04% | 0.11% | -0.02% | 100% | 100% |
| Overall | 0.02% | -0.01% | 0.05% | 99% | 99% |

CE4.8.2 Motion information storage for each 16x16 region

Table 4 Random access configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.11% | 0.11% | 100% | 101% |
| Class A2 | 0.25% | 0.25% | 0.16% | 99% | 99% |
| Class B | 0.23% | 0.25% | 0.18% | 100% | 100% |
| Class C | 0.55% | 0.61% | 0.56% | 100% | 95% |
| Overall | 0.29% | 0.32% | 0.27% | 100% | 99% |
| Class D | 0.74% | 0.68% | 0.79% | 99% | 83% |

Table 5 Low delay configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.35% | 0.35% | 0.18% | 99% | 98% |
| Class C | 0.73% | 0.76% | 0.78% | 100% | 96% |
| Class D | 0.82% | 1.21% | 0.35% | 100% | 88% |
| Class E | 0.90% | 1.36% | 1.18% | 101% | 98% |
| Overall | 0.61% | 0.74% | 0.63% | 100% | 97% |

CE4.8.3 Motion information storage with alternative spatial candidates for each 16x16 region

Table 6 . Random access configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.03% | 0.04% | 100% | 101% |
| Class A2 | 0.15% | 0.11% | 0.10% | 99% | 99% |
| Class B | 0.16% | 0.17% | 0.11% | 100% | 100% |
| Class C | 0.30% | 0.38% | 0.29% | 99% | 95% |
| Overall | 0.17% | 0.18% | 0.14% | 100% | 99% |
| Class D | 0.53% | 0.45% | 0.43% | 99% | 82% |

Table 7 Low delay configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.27% | 0.28% | 0.09% | 99% | 98% |
| Class C | 0.51% | 0.40% | 0.40% | 99% | 96% |
| Class D | 0.66% | 0.86% | 0.48% | 99% | 89% |
| Class E | 0.68% | 0.73% | 0.97% | 100% | 99% |
| Overall | 0.45% | 0.43% | 0.42% | 99% | 98% |

Conclusion

An alternative spatial position based motion data compression method is proposed. Experimental results reportedly show that the proposed method gives 0.17% BD-rate increase when the motion data is compressed based on 16x16 grid while the conventional top-left motion data based compression method shows 0.29% BD-rate increase on 16x16 grid. It is recommended that the proposed 16x16 grid motion data compression is considered to be adopted in the next VVC software.

Reference

- H. Jang, J. Lim, J. Nam, S. Kim, “Non-CE: Temporal motion vector data storage reduction,” JVET-K0086, Ljubljana, SI, July 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-K1024, Ljubljana, SI, July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).