| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0171 |
| --- | --- |

| Title: | CE4-related: Merge mode with Regression based Motion Vector Field (RMVF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin Ghaznavi-Youvalari Alireza Aminlou Jani Lainema | Emails: | ramin.ghaznavi-youvalari@nokia.com alireza.aminlou@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes Regression based Motion Vector Field (RMVF) to improve motion vector derivation for merge mode in VTM-2.0.1. The proposed tool attempts to model motion behavior of a block using motion vectors of the neighboring blocks by a linear model. RMVF tool is used as new merge mode in the VTM-2.0.1 software. When the mode is selected, motion vectors area generated using the linear model for 8×8 sub-blocks in order to provide sub-PU level motion granularity. The proposed method provides on average -0.66%, -0.10% and -0.26% Luma BD-Rate impact in RA, LDB, and LDP configurations, respectively. The encoding runtimes of 106%, 108% and 106% and decoding runtimes of 103%, 100% and 99% in RA, LDB, and LDP configurations were observed, respectively. Similar gains have been observed in Chroma BD-Rates.

# Introduction

Motion behavior may vary inside a block; hence it is not efficient to derive only one motion vector for each block. This appears to be important particularly for larger blocks. Thus, calculating motion vectors in a finer granularity in each block can provide a smooth and more realistic motion vector field inside that block. To that end, the RMVF tool is proposed in this contribution in order to model such motion behavior based on the motion vectors of the spatially neighboring blocks.

# Proposed Method

In order to provide a fine granularity of motion vectors inside a block, Regression based Motion Vector Field (RMVF) tool is proposed. This tool attempts to model the motion vectors of each block on a sub-block level based on the spatially neighboring motion vectors.

Figure 1 illustrates the neighboring 4×4 motion blocks that are used for motion parameter derivation of the proposed method. As seen, one line and row of immediate neighboring motion vectors in 4×4 sub-block basis (and their center locations) from each side of the block are used in the regression process.

Five conventional regions (bottom-left, left, top-left, top, top-right) as shown in the figure are used when collecting the motion information for motion parameter derivation. The above-right and bottom-left reference motion regions are limited to only half of the corresponding width or height of the current block.

Figure 1. Neighboring motion vectors that are used in RMVF motion parameter derivation.

In RMVF mode, motion of the block is defined by a 6-parameter motion model. These parameters axx, axy, ayx, ayy, bx and by are calculated by solving a linear regression model in mean square error (MSE) sense. The input to the regression model consists of the center locations (x, y) and motion vectors (mvx and mvy) of the available neighboring 4×4 sub-blocks as defined above.

Motion vector (MVX_subPU, MVY_subPU) for an 8×8 sub-block with center location at (XsubPU, YsubPU) is then calculated as:

MVX_subPUMVY_subPU= axxaxyayxayyXsubPUYsubPU+ bxby

The motion vectors are calculated for 8×8 sub-blocks relative to the center location of each sub-block. Consequently, motion compensation is also applied in 8×8 sub-block accuracy in the RMVF mode.

In order to have efficient modelling for the motion vector field, RMVF tool is applied only in the cases that at least one motion vector from at least three of the candidate regions are available.

Further details:

- The minimum coding unit size for RMVF mode is 8×8

- Decision for uni- or bi-directional prediction is done based on the picture type (P/B picture) and availability of neighboring motion information for that prediction direction.

- Reference index in both prediction directions is set to 0.

- When calculating the motion parameters, the conventional motion vector scaling used in VTM-2.0.1 is applied to the motion vectors that have different reference pictures than the current block.

- All the calculated sub-block motion vectors are set to be in high precision.

- The sub-block motion compensation is done as in VTM-2.0.1 (but with 8×8 sub-block size).

- In merge mode, a flag is signalled in the bitstream that indicates whether the RMVF tool is used in each block as a new merge mode or not. The signalling order for the RMVF flag is done as following: Merge flag → Affine flag → RMVF flag

# Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the RMVF tool on top of the VTM-2.0.1 compared to the anchor with RA, LDB and LDP configurations are shown in following tables. The full results are provided in the attached excel sheets.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.69% | -0.92% | 101% | 100% |
| Class A2 | -0.96% | -0.98% | -1.18% | 103% | 107% |
| Class B | -0.62% | -0.59% | -0.67% | 108% | 101% |
| Class C | -0.32% | -0.35% | -0.38% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.62% | -0.74% | 106% | 103% |
| Class D | -0.52% | -0.69% | -0.51% | 109% | 107% |
| Class F (optional) | -0.07% | -0.18% | -0.15% | 108% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.04% | 0.05% | 107% | 96% |
| Class C | -0.09% | -0.22% | -0.23% | 109% | 102% |
| Class E | 0.02% | 0.19% | -0.11% | 109% | 107% |
| Overall | -0.10% | -0.04% | -0.09% | 108% | 100% |
| Class D | 0.06% | 0.88% | 0.08% | 109% | 103% |
| Class F (optional) | 0.10% | 0.45% | -0.52% | 106% | 104% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 0.09% | 0.18% | 104% | 99% |
| Class C | -0.19% | -0.13% | -0.50% | 108% | 98% |
| Class E | -0.12% | 0.03% | 0.65% | 108% | 102% |
| Overall | -0.26% | 0.00% | 0.07% | 106% | 99% |
| Class D | -0.02% | 0.28% | -0.11% | 111% | 107% |
| Class F (optional) | -0.01% | -0.23% | 0.61% | 106% | 101% |

# Conclusion

This contribution proposed Regression based Motion Vector Field (RMVF) method for improving the motion vector prediction of the VTM-2.0.1 test model. Proposed RMVF method provided on average -0.66%, -0.10% and -0.26% Luma BD-Rate improvements for RA, LDB and LDP configurations, respectively with a reasonable complexity overhead. It is proposed to test the impact of the tool in a core experiment during the next meeting cycle.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).