| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0172_v2 |
| --- | --- |

| Title: | CE14: Reduced latency, LUT-free bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Jack Enhorn Du Liu Kenneth Andersson Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a modified version of the bilateral filter from JVET-K0274. The main modification is a replacement of the look-up table in JVET-K0247 by a clipped line. This way, it is asserted, no look-up table is used on a per-sample basis. The contribution claims that this reduces the latency introduced by the filter by 21% in the inter case and 9% in the intra case compared to JVET-K0274, according to a clock cycle model described in the paper. Furthermore, 4×4 blocks are not filtered, and the contribution claims that this helps hardware implementation considerations further. The BD-rate figures for an implementation in BMS-2.0.1 running in VTM configuration are reported to be -0.39% / -0.67% / -0.64% for AI/RA/LD respectively. The associated decoder run times are reported to be 104% / 102% / 103%.

# Introduction

The bilateral filter presented in JVET-D0069, JVET-E0032, JVET-F0034 and JVET-F0096 was included into the JEM test model at the Hobart meeting. At the Ljubljana meeting it was suggested in JVET-K0274 to include it into the VTM. Two objections were raised at the meeting: The first was that the extra latency introduced in the post-reconstruction phase (i.e., before samples are available to predict the next block) was too large for full custom ASIC hardware implementations. This was especially a problem for small block sizes such as 4×4, since these represent the worst case for such an implementation. The second objection raised was that the largest per-sample look-up table was larger than 16 bytes and that is therefore was hard to efficiently implement using SIMD instructions.

In this proposal we address both of these issues. Firstly, the look-up tables are replaced by a clipped line. This eliminates the SIMD implementation issue with big LUT sizes. Secondly, filtering is avoided for inter- and intra-blocks of size 4×4. This avoids increasing the latency in the worst case situation when all blocks are of size 4×4. Thirdly, as described in Appendix A, even for blocks that are filtered, the latency is reduced by 21% (inter case) and 9% (intra case) compared to JVET-K0274, according to a clock cycle model described in Appendix B.

# Removal of the look-up table

As described in JVET-K0274, the bilateral filter for intra blocks can be written in the form

IF=IC+dσd×ρqpΔIA×ΔIA+ρqpΔIB×ΔIB+ρqpΔIL×ΔIL+ρqpΔIR×ΔIR,        (Eq. 1)

where IF is the filtered sample intensity value, and IC is the center sample value, i.e., the sample value before filtering. The value ΔIA is the difference between the center sample value IC and the sample value of the sample above, IA, ΔIA=IA-IC. Analogously, IB, IL and IR are the values of the samples immediately below, left and right of the center pixel respectively, and ΔIB=IB-IC, ΔIL=IL-IC and ΔIR=IR-IC. Furthermore,

dσd=256255e- 12σd21+4e- 12σd2         (Eq. 2)

and

ρqpΔI=255256e- ΔI28qp-172.        (Eq. 3)

In JVET-K0274, Equation 3 was stored as a two-dimensional look-up table (LUT). For each block, a row of the LUT was selected to obtain the values for Equation 3. Since the look-ups from these LUT rows were done per pixel, they were in the inner loop and would need to be sped up by SIMD instructions. However, since these table rows were up to 235 bytes in size, they were hard to implement efficiently in SIMD. Therefore, in this proposal, we approximate Equation 3 by a clipped line,

λqpΔI=max0, kqp×ΔI+mqp.      Eq.4

An example can be seen for qp=38 in Figure 1. Here the LUT row ρqpΔI is plotted in blue and the approximation λqpΔI is plotted in red.

(a)                                                                                        (b)
Figure 1: (a): A row of the look-up table (blue) approximated using a piecewise linear function (red). (b): The curve LUTΔI*ΔI (blue) and the approximation (red).

Since the curve in Figure 1a always crosses the y-axis in 255/256, mqp=255/256 for all qps. We find kqp for each qp by minimizing the summed squared error between the curves ρqpΔI*ΔI (blue curve in Figure 1b) and λqpΔI*ΔI (red curve in Figure 1b).

Using the approximation in Equation 4, Equation 1 becomes

IF=IC+dσdλqpΔIAΔIA+dσdλqpΔIBΔIB+dσdλqpΔILΔIL+dσdλqpΔIRΔIR,      (Eq. 5)

where we have multiplied dσd inside the brackets. Now if we look at the first two factors of the first term, we see that

dσd×λqpΔIA=dσd×max0, kqp×ΔI+mqp=max⁡(0, dσd×kqp×ΔI+dσd×mqp)=max0, kqp×ΔI+mqp,      (Eq. 6)

where kqp=dσd×kqp and mqp= dσd×mqp. Since kqp and mqp are constant over the block they can be calculated outside the inner loop. Alternatively, they can be precalculated and retrieved. Equation 5 therefore simplifies to

IF=IC+zkΔIA+mΔIA+zkΔIB+mΔIB+zkΔIL+mΔIL+zkΔIR+mΔIR,        (Eq. 7)

where zx=max0,x clips to zero and where k and m are calculated as described above.

Likewise, for inter blocks, a sample is filtered according to

IF=IC+zkNLA+mΔIA+zkNLB+mΔIB+zkNLL+mΔIL+zkNLR+mΔIR,        (Eq. 8)

where NL is the average of the absolute value differences in a 4×3 or 3×4 pixel window, as described in JVET-J0021 and JVET-K0274.

Eight fractional bits are used for the m-value, thus represented by 255. The k-values are stored per qp using twelve fractional bits. The largest value is < 0.25, hence ten bits are sufficient:

k_neg[46] = {816, 408, 291, 215, 170, 141, 124, 107, 95, 85, 78, 72, 66, 61, 57, 54, 50, 47, 45, 43, 41, 39, 37, 36, 34, 33, 32, 31, 30, 29, 28, 27, 26, 25, 25, 24, 23, 23, 22, 21, 21, 20, 20, 20, 19, 19}

The d(σd)-values have ten bit fractional precision. The largest one is smaller than 0.25, hence eight bits are sufficient for storage. They take the following values:

|  | 4×n and n×4 blocks, n>4 | 8×n and n×8 blocks, n≥8 | 16×n and n×16 blocks and larger, n≥16 |
| --- | --- | --- | --- |
| dσd intra | 134 | 119 | 71 |
| dσd inter | 122 | 111 | 111 |

Table 1: The distance factors used in this proposal.

The k-value is calculated (once per block) as

k=dσd*k+128≫8      (Eq. 9)

where ≫  denotes arithmetic rightward shift. The largest such value is therefore 134*816+128 div 256=427, meaning that 9 bits are sufficient to store it. Given that there are only five possible values for d(σd), if an implementation instead wants to store pre-calculated k-values, the storage requirements are 46*5*9/8=259  bytes. Otherwise it is sufficient to store the 46 9-bit values of k_neg listed above, which would require 52 bytes.

Likewise, the m-value is calculated (once per block) as

m=dσd*m+8≫4,     (Eq. 10)

and the largest value is therefore 134*255+8 div 16=2136. This means that if an implementation wants to store these in pre-calculated form, 46*6*12/8 =345 bytes would be required. However, in a hardware implementation, a multiplication x*255 can be efficiently calculated as x≪8-x, so it may more efficient to implement this as a subtraction. Note also that in a hardware implementation, k and m can be calculated as soon as qp is known for the block, so this will not add latency to the filtering process.

Also note that even though the number of values in the k_neg[46] variable above is larger than 16, this poses no problem for an efficient SIMD implementation since this value is fetched only once per block.

# Calculation of distance factors d(σd)

In the inter case, the distance factors are calculated the same way as in JVET-K0274, i.e, using Equation 2, using σd=0.58 for the 4×n case, yielding dσd=122 and using σd=0.55 for the 8×n case, yielding dσd=111. For the largest case, 16×n, dσd=111 is reused.

In the intra case, JVET-K0274 had the following distance factors for corner, border and middle samples (Table 2).

|  | σd=0.59 4xn and nx4, n>4 | σd=0.56 8xn and nx8, n ≥ 8 | σd=0.46 16xn and nx16, n ≥ 16 |
| --- | --- | --- | --- |
| dσdintra middle | 125 | 115 | 70 |
| dσdintra border | 143 | 130 | 75 |
| dσdintra corner | 166 | 148 | 81 |

Table 2: The distance factors used in JVET-K0247

This is no longer possible if we want to share computation between, say, a border and a middle sample. This is due to the fact that k=dσd*k is held constant over the block. Instead we use an average between the middle and the border value. For 4×8 blocks around 50% of the pixels are border pixels, and around 50% are middle pixels, so we use dσd=(125+153)/2 =134. For 8x16 blocks around 25% of the pixels are border pixels, and around 75% are middle pixels, so we use dσd=(3*115+130)/4 ≈119. Finally for 16x32 blocks we use dσd=7*70+75/8≈71. This results in the distance factors shown in Table 1.

# The three sub-tests

At the Ljubljana meeting, it was asked that the sub-CE 14.1 investigate the following three items:

- What is the performance if filtering is turned off for 4×4 blocks?

- What is the performance if filtering is also turned off for 4×8 and 8×4 blocks?

- Can the bilateral filter be constructed without a LUT that is bigger than 16 values?

To answer these questions, we have conducted three experiments:

14.1a: Here we use the LUT-free version described above. We also do not filter 4×4 blocks (intra or inter). BD-rate figures are -0.39% / -0.67% / -0.64% for AI/RA/LD.

14.1b: The same as 14.1a, but we also avoid filtering 4x8 and 8x4 blocks. BD-rate figures are -0.30% / -0.58% / -0.64%.

14.1c: This is the same filter as JVET-K0274, but implemented in VTM-2.0.1 instead of VTM-1.0. BD-rate figures are -0.42% / -0.71% / -0.75%

By comparing 14.1a with 14.1b it is possible to see how much BD-rate is lost when removing also 4×8 and 8×4 blocks from the filtering, and this figure is about 0.1% for AI and RA. By comparing 14.1a with 14.1c it is possible to see how much is lost by approximating the LUT with a clipped linear function. The result is about 0.03% for AI and RA, and about 0.1% for LD.

We believe the extra gain of 0.1% in RA for 14.1a is worth it, and we therefore propose to adopt CE14.1a into the VTM.

# Evaluation and complexity analysis

The CE description also asks to fill out the table in Section 2.4. We will go through each term and describe our results.

## Filter shape

In the intra case the filter has a five pixel support shaped as a plus. In the inter case it uses a 5x5 pixel area to calculate the filter weights, but the same five pixels are filtered in the end.

## Computational Complexity (#of mult/add/shifts/checks) per sample

The inner loop of the intra filtering code for 14.1a looks as follows:

1. centerPixel = rightPixel;
      2. rightPixel = *(blockRightPixelPtr++);
      3. dIR = rightPixel - centerPixel;
      4. dIB = *(blockNextLinePixelPtr++) - centerPixel;
      5. rightInfluenceCalc = std::max(0, my_k * abs(dIR) + my_m) * dIR;
      6. bottomInfluenceCalc = std::max(0, my_k * abs(dIB) + my_m) * dIB;
      7. currentInfluenceSumCalc = rightInfluenceCalc - leftInfluenceCalc + bottomInfluenceCalc - *(influenceRowPtrCalc);
      8. leftInfluenceCalc = rightInfluenceCalc; 
      9. *(influenceRowPtrCalc++) = bottomInfluenceCalc; 
      10. int32_t new_style_filtering_calc = (int16_t)(centerPixel + (uint16_t)((currentInfluenceSumCalc + 8192) >> 14));
      11. *(blockCurrentPixelPtr++) = new_style_filtering_calc;

This involves four multiplications (rows 5 and 6) per pixel, nine additions (one each on lines 3, 4, 5 and 6, three on line 7 and two on line 10), two abs() operations (lines 5 and 6) and two max() operations (line 5 and 6).

For the inter case, calculating dNLIR and dNLIB requires another 14 additions, two shifts and six abs()-operations per pixel. For dNLIR, the relevant operations are

delta = abs(*(blockCurrentPixelPtr+offsetY0+1) - *(blockCurrentPixelPtr+offsetY0+2)) + abs(*(blockCurrentPixelPtr+1) - *(blockCurrentPixelPtr+2)) + abs(*(blockCurrentPixelPtr-offsetY0+1) - *(blockCurrentPixelPtr-offsetY0+2));

omega = omega - alpha + delta;

dNLIR = (omega) >> 3;

Here three additions are needed to form the absolute differences, two are needed to sum together the abs-values to an omega value and another two additions are needed to sum together the delta value. In total seven extra additions and one shift are needed. The same goes for dNLIB, bringing the extra sum to 14 additions and two shifts.

## Precision of multiplications

Intra case:

For the first multiplication on line 5, the value my_k always negative, and at most -426, so it is possible to instead multiply by –my_k. Thus the first operand needs 9 bits. The second operand, abs(dIR), is at most 1023 (10-bit unsigned), so here 9×10 should be sufficient. Note however that the value of the max()-statement is 0 if abs(dIR)>227, so only a 9x8 multiplication is needed.

For the second multiplication on line 5, the output of the max()-statement is in the range [0, 2136], so twelve bits are enough for the first operand. For the second operand, dIR is in the range [-1023, 1023]. However, again, if abs(dIR)>227 then the max()-statement is zero, so only 9 bits are needed. Hence, 12×9 is sufficient.

Inter case:

In the inter case, the line analogous to line 5 is the following:

rightInfluenceCalc = std::max(0, my_k * dNLIR + my_m) * dIR;

For the first multiplication, this figure is the same as in the intra case: 9×8 multiplication. Again, if dNLIR>227 then the max expression is zero.

For the second multiplication, we can no longer rely on the fact that the max()-result is zero if dIR>227. Hence the full 11 bits of  dIR must be used: The result is a 12×11 multiplication.

Note that, in order to minimize latency, it may be advantageous to reorder the multiplications. As is described in Appendix A on latency estimation, optimizing this way may change the sizes of the multiplications.

## Parallel friendly

The method is parallel friendly, i.e., each filtered pixel can be calculated independently of other pixels.

## Number of clock cycles

This calculation is described in more detail in Appendix A. As discussed in Ljubljana, the important figure is the latency, i.e., the number of clock cycles before the first filtered pixel is output. For this proposal, we have estimated these latencies to

|  | latency before first filtered sample |
| --- | --- |
| intra | 10 |
| inter | 11 |

After this, it is possible to output one sample per clock, or N samples per clock cycles if N parallel filter instantiations are used.

## Memory requirements

We need to store k_neg (46 values of 10 bits) and the values for d(σd) (5 eight-bit values), in total 46*910/8+5*8=57 bytes.

## How to derive filter coefficients

The filter coefficients are calculated as max0, kqp×ΔI+mqp for intra blocks and max0, kqp×NL+mqp for inter blocks.

## Minimal CU Size for applying filters

In 14.1a (our proposed version) the smallest filtered block is of size 4×8 or 8×4.

## Final table

All in all, this is the table to include in the CE-report.

| Test | filter shape | Compu-tational complex-ity per sample | Precision of mult | Parallel friendly | Latency (in clock cycles) | Memory require-ments | How to derive filter coeffs | Minimal CU size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1 | 5 pixel “plus”-shape | 4 mult 9 adds 2 abs 2 max 1 shift (intra) +14 add +6 abs +2 shifts (inter) | 9×8 and 12×9 (intra) 9×8 and 12×11 (inter) | yes | 10 (intra) 11 (inter) | 63 bytes | max0, kqp×ΔI+mqp (intra) max0, kqp×NL+mqp (inter) | 4×8 and 8×4 |

# Results

The filter was implemented in BMS-2.0.1. For the VTM-2.0.1 test, the vtm-configuration files were used, with BIF=1. The decoder run time measurements were done on the same computer for both anchor and test and are therefore more reliable than the encoder run time measurements.

## Results for 14.1a

### All-intra results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.13% | 0.13% | 99% | 106% |
| Class A2 | -0.27% | 0.15% | 0.15% | 102% | 107% |
| Class B | -0.33% | 0.17% | 0.16% | 103% | 106% |
| Class C | -0.65% | 0.07% | 0.26% | 106% | 102% |
| Class E | -0.46% | 0.06% | 0.05% | 107% | 107% |
| Overall | -0.39% | 0.12% | 0.16% | 103% | 105% |
| Class D | -0.49% | 0.25% | -0.12% | 105% | 98% |

### Random access results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.19% | -0.18% | 103% | 100% |
| Class A2 | -0.69% | -0.15% | -0.25% | 104% | 102% |
| Class B | -0.76% | -0.09% | -0.03% | 105% | 105% |
| Class C | -0.74% | -0.01% | -0.15% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | -0.10% | -0.14% | 105% | 102% |
| Class D | -0.67% | 0.00% | 0.24% | 100% | 99% |

### Low delay results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.58% | 0.52% | 0.53% | 105% | 104% |
| Class C | -0.66% | 0.34% | 0.27% | 105% | 104% |
| Class E | -0.72% | 1.03% | 0.30% | 99% | 100% |
| Overall | -0.64% | 0.59% | 0.39% | 103% | 103% |
| Class D | -0.89% | 0.33% | 0.50% | 103% | 98% |

## Results for 14.1b

### All-intra results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.07% | 0.11% | 102% | 104% |
| Class A2 | -0.23% | 0.16% | 0.13% | 104% | 104% |
| Class B | -0.27% | 0.10% | 0.15% | 107% | 103% |
| Class C | -0.46% | 0.02% | 0.16% | 105% | 101% |
| Class E | -0.33% | 0.10% | -0.01% | 108% | 103% |
| Overall | -0.30% | 0.09% | 0.11% | 105% | 103% |
| Class D | -0.32% | 0.24% | -0.12% | 99% | 100% |

### Random access results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.20% | -0.09% | 105% | 103% |
| Class A2 | -0.62% | -0.25% | -0.27% | 104% | 101% |
| Class B | -0.69% | -0.13% | -0.09% | 105% | 102% |
| Class C | -0.57% | 0.14% | 0.01% | 107% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -0.10% | -0.10% | 105% | 102% |
| Class D | -0.49% | 0.11% | 0.11% | 106% | 97% |

### Low delay results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | 0.45% | 0.48% | 109% | 101% |
| Class C | -0.55% | 0.40% | 0.34% | 104% | 103% |
| Class E | -0.87% | 0.54% | 0.78% | 99% | 100% |
| Overall | -0.64% | 0.45% | 0.51% | 105% | 101% |
| Class D | -0.69% | 0.70% | 0.05% | 104% | 99% |

## Results for 14.1c

### All-intra results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.11% | 0.22% | 105% | 106% |
| Class A2 | -0.28% | 0.12% | 0.10% | 109% | 105% |
| Class B | -0.37% | 0.18% | 0.28% | 102% | 104% |
| Class C | -0.69% | 0.07% | 0.15% | 106% | 103% |
| Class E | -0.51% | 0.19% | 0.12% | 107% | 105% |
| Overall | -0.42% | 0.14% | 0.18% | 105% | 104% |
| Class D | -0.55% | 0.25% | -0.06% | 101% | 104% |

### Random access results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.04% | -0.15% | 104% | 103% |
| Class A2 | -0.75% | -0.28% | -0.20% | 105% | 103% |
| Class B | -0.80% | -0.12% | -0.11% | 104% | 103% |
| Class C | -0.79% | 0.21% | 0.01% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.71% | -0.05% | -0.11% | 104% | 103% |
| Class D | -0.72% | -0.35% | -0.15% | 103% | 100% |

### Low delay results

For VTM:

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | 0.32% | 0.31% | 106% | 104% |
| Class C | -0.67% | 0.24% | 0.30% | 101% | 104% |
| Class E | -1.06% | 0.35% | 0.60% | 100% | 102% |
| Overall | -0.75% | 0.30% | 0.38% | 103% | 104% |
| Class D | -0.87% | 1.14% | 0.75% | 105% | 104% |

# Appendix A: Latency estimations

At the meeting in Ljubljana it was also requested that an analysis of the latency would be performed. In detail, the crucial measurement is the number of clock cycles that a hardware implementation will need from the moment when the unfiltered reconstructed samples are available until the moment when the border samples have been filtered and thus are available for prediction of the next block.

When estimating this latency we have used the clock cycle model described in Appendix B. This is a simplified model of what work can be done in a clock cycle. Generally, the same arithmetic operations can be implemented in many different ways, each with a trade-off of latency versus surface area. It may therefore not perfectly capture any given implementation, but gives a framework in which we can compare this proposal to JVET-K0274 from the previous meeting.

The border pixels of the block are the only ones used for prediction; hence we concentrate our analysis on how many clock cycles it takes to filter these.

## Inter case

To filter a sample in the inter case we use Equation 8, repeated here for the convenience of the reader

IF=IC+zkNLA+mΔIA+zkNLB+mΔIB+zkNLL+mΔIL+zkNLR+mΔIR.        (Eq. 8)

Figure 2: When filtering a border pixel, the R sample value will be equal to the C sample value since R is just a padded copy of C.

As can be seen in Figure 2, padding means that IR=IC, and hence ΔIR=IR-IC=0. Equation 8 thus simplifies to

IF=IC+zkNLA+mΔIA+zkNLB+mΔIB+zkNLL+mΔIL.        (Eq. 11)

If we look at the second term of Equation 11 involving the pixel above, we can write this as

max0, k*NLA+m*ΔIA.            (Eq. 12)

The first thing to calculate here is the multiplication k*NLA. This is unfortunate, since NLA takes five clock cycles to calculate (see Appendix C). However, if ΔIA is positive, we can instead calculate

max0, k*ΔIA*NLA+m*ΔIA,     (Eq. 13)

which gives the same result. Since ΔIA is available after just two clock cycles (calculating ΔIA=IA-IC in the first clock cycle and taking the abs(⋅) in the second), we can start the multiplication k*|ΔIA| already after two clock cycles. (Note that k is available already from the start since it can be stored in precalculated form or calculated when qp is known.)

We can use the modified Booth’s method with k as operand b (see Appendix B-8.2). Since k can be represented as a 10-bit two-complement’s number we need log210/2=3 cycles to perform the multiplication.

The next step is to multiply this by NLA. Since five clock cycles have passed, NLA is now available.

Since k is negative we know that for a sufficiently large NLA, Equation 13 will result in zero. The cut off happens at NLA=227 for the weakest slope. Hence we only need to consider values of NLA from 0 to 227, which means that an 8-bit multiplication is sufficient. Using modified Booth’s this takes log28/2=2 clock cycles.

We now sum together the two terms and clamp to zero, which takes one clock cycle (see Appendix B-8.1). If ΔIA really was negative, we have to negate the result, which takes another clock cycle. We should now sum together the four terms in Equation 11, which takes another two clock cycles.

| Operation | bit widths input | bit widths output | number of clock cycles | total number of clock cycles |
| --- | --- | --- | --- | --- |
| ΔIA=IA-IC | IA=10, IC=10 | ΔIA=11 | 1 | 1 |
| ΔIA=absΔIA | ΔIA=11 | ΔIA=10 | 1 | 2 |
| k*¦ΔIA¦ | k=10, ΔIA=10 | ans=20 | 3 | 5 |
| *NLA | ans=20, NLA=8 | NLA=28 | 2 | 7 |
| max⁡(0, ans+m*ΔIA) | ans=28, m*ΔIA=22 | ans = 22 | 1 | 8 |
| negate | 22 | 23 | 1 | 9 |
| sum Eq. 11 and shift 14 | 23 | 10 | 2 | 11 |

In total, we see that it takes 11 clock cycles to get to the filtered sample. As is described in Appendix D, a similar analysis of JVET-K0274 results in 14 clock cycles of latency. Hence the current proposal shows a latency decrease of (14-11)/14 =21%, a substantial decrease.

## Intra case

In the intra case, the filtering equation for an edge pixel is

IF=IC+zkΔIA+mΔIA+zkΔIB+mΔIB+zkΔIL+mΔIL,        (Eq. 14)

and there is therefore no need to hide the latency associated with calculating NLA. Looking again at the first term

max0, k*ΔIA+m*ΔIA,        (Eq. 15)

we start by calculating |ΔIA| (two clock cycles) and continue by multiplying; k*ΔIA. For the same reason as in the inter case, the result of the max-operation will be zero if ΔIA>227, so this multiplication costs log28/2=2 cycles. Next we add m and perform the max-operation, which takes one clock cycle. Next we multiply by ΔIA. Since ΔIA∈-1023, 1023 it is sufficient with an 11-bit two-complement’s number to describe ΔIA. We can use the modified Booth’s method (see Appendix B-8.2) which needs log2112=3 clock cycles to perform the multiplication. Finally we sum the terms in Equation 14 which takes another two cycles.

| Operation | bit width input | bit width output | number of clock cycles | total number of clock cycles |
| --- | --- | --- | --- | --- |
| ΔIA=IA-IC | IA=10, IC=10 | ΔIA=10 | 2 | 2 |
| k*ΔIA | k=10, ΔIA=8 | ans=18 | 2 | 4 |
| max0, ans+m | ans=18, m=13 | ans=13 | 1 | 5 |
| *ΔIA | ans=13, ΔIA=9 | ans=22 | 3 | 8 |
| sum Eq. 14. | 22 | 10 | 2 | 10 |

In total we get 10 clock cycles for the intra case. This should be compared with 11 cycles for JVET-K0274, a reduction by (11-10)/11=9%.

# Appendix B – Clock cycle model

In this appendix we go through our assumptions about how much work can be done in a clock cycle for a full custom ASIC implementation. This model is simplified, and may therefore not perfectly capture any given implementation, but it provides a framework in which we can make comparisons between this method and that of JVET-K0274.

## Additions

We assume that adding two integers together can be performed in one clock cycle. Also, there is time to conditionally zero the output using the result of the addition. Hence it is possible to calculate max(0, A+B) since the inverse of the sign bit resulting from A+B can be used to AND the output, creating 0 when A+B is negative.

It is assumed that abs(A-B) cannot be calculated in the same clock cycle, but that one is needed to calculate A-B and another one to calculate abs of the result.

## Multiplications

We assume that given two integers a and b of size m bits and n bits respectively, we can perform the multiplication m×n in log2minm,n clock cycles, where  ⋅  rounds upwards. This is due to the fact that a multiplication can be implemented as a series of adders, as can be seen in Figure B1.

Figure B1. Multiplication of a 6-bit number a and a 4-bit number b.

In the first stage we conditionally zero the bits in a using the LSB in b, b0. We do the same with a shifted version of a with bit b1. In the same clock cycle we add these to together using the adder α.  In parallel, i.e., also in the same clock cycle, we can also perform the analogous operations in adder β. In the second clock cycle the output from α and β are added together in the adder γ. It is simple to see in the figure how the method could be extended to eight bits in b by duplicating the logic and adding another adder δ, adding another clock cycle to the count. Hence n bits would take log2n clock cycles.

By using the Modified Booth’s algorithm [1] it is possible to reduce the number of partial sums by half. In this case we therefore need log2n/2 clock cycles if b is n bits long (assuming that b≤a).

It should also be noticed that in Figure A1 the LSBs in α and β are calculated first, and that some calculation in γ can hence be started before the last output bit of α is finished. This typically further lowers the clock cycle count. However, we have not used this fact in this simplified model in order to be on the conservative side.

## Look-up table

When comparing with the LUT-based approach in JVET-K0274 we also need to know how many cycles it takes to perform a look-up. Since LUTs with constant values are typically synthesized as pure logic, it is harder to estimate how many cycles a LUT operation takes. In order not to overestimate our latency saving versus JVET-K0274 we therefore opt on the conservative side and assume that a LUT look-up only takes a single clock cycle.

# Appendix C: Calculating NLA

In this appendix we describe the clock cycle count needed to calculate NLA using the clock cycle model we described in Appendix B.

Figure C1: Each arrow represents an absolute difference in intensity. As an example, the red arrow represents the value ΔIA=IA-IC. When calculating NLA, we calculate the average of all the arrows in the figure.

For the intra case, we need to calculate terms like

max0, k*ΔIA+m*ΔIA.      (Eq. C1)

Here ΔIA is the absolute difference in intensity between the center pixel and the one above. We represent that with the red arrow in Figure C1. However, for the inter case, we calculate terms like

max0, k*NLA+m*ΔIA.      (Eq. C2)

Here NLA is the average of nine ΔI-terms. In detail, it represents the average of all the arrows in Figure C1 (including the red one). In order to save computation, the division by 9 needed to form the average is replaced by a division by 8, implemented as a bit shift.

Calculating all the arrows takes two clock cycles (one for the difference operation and one for the abs()-operation). Summing together these nine terms takes another four clock cycles, making it six clock cycles in total.

Note however that for the edge samples it is sufficient with five clock cycles. The reason is that for edge samples, the right-most samples are padded.

Figure B2. In a padded block the number of unique differences go down from nine to six.

As can be seen in Figure B2, the three rightmost differences will be the same as the three middle ones. Hence it is sufficient to shift the middle three ones one bit and sum them together with the three first ones. Since we then only have six operands, these can be added together in three clock cycles, bringing the total clock cycle count to five.

When calculating NLL, three of the nine terms are instead zero, again making it possible to calculate the value in five clock cycles.

# Appendix D: Latency estimation for JVET-K0274

In JVET-K0274, the following equation was used to filter the pixel,

IF=IC+d*LUTNLA*ΔIA+LUTNLB*ΔIB+LUTNLL*ΔIL+LUTNLR*ΔIR.     (Eq. C1)

Since we are counting cycles for edge pixels, we can again simplify the equation to

IF=IC+d*LUTNLA*ΔIA+LUTNLB*ΔIB+LUTNLL*ΔIL.     (Eq. C2)

Calculating NLA takes five cycles for an edge sample (see Appendix C), and after that the LUT(x) operation takes place, which is assumed to take a single clock cycle. The value ΔIA is in range [-1023,1023] and thus representable with 11 bits, and the multiplication can therefore be performed in log211/2 =3 clock cycles. Adding together the terms in Equation C2 takes two cycles, multiplying by the 8-bit value d takes another log28/2=2 cycles and finally adding to IC takes one more clock cycle. As shown in the table below, this adds up to 14 clock cycles.

| operation | number of clock cycles | total number of clock cycles |
| --- | --- | --- |
| calculate NLA | 5 | 5 |
| apply LUT | 1 | 6 |
| *ΔIA | 3 | 9 |
| sum terms in Eqn C2 | 2 | 11 |
| *d | 2 | 13 |
| add to IC | 1 | 14 |

Table D1: Latency estimation for JVET-K0274, inter case.

For the intra case, the sum is 11 clock cycles, and the analogous table is

| operation | number of clock cycles | total number of clock cycles |
| --- | --- | --- |
| calculate ΔIA | 2 | 2 |
| apply LUT | 1 | 3 |
| *ΔIA | 3 | 6 |
| sum terms in Eqn C2 | 2 | 8 |
| *d | 2 | 10 |
| add to IC | 1 | 11 |

Table D2: Latency estimation for JVET-K0274, intra case.

# References

[1] M. R. Bhongade, M. M. Ramteke, V. G. Roy, A Review on Optimizing Efficiency of Fixed Point Multiplication using Modified Booth’s Algorithm, “National Conference On Research Trends In Electronics, Computer Science & Information Technology And Doctoral Research Meet”, Feb 21st  & 22nd, 2014

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).