| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0192 |
| --- | --- |

| Title: | CE11: Higher precision modification for VVC deblocking filter (Test 2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Guillaume Laroche Jonathan Taquet | Tel: Email: | christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution contains the results of the 6 subtests of the CE11 test 2.1 in VTM2.0 about the proposal JVET-K0203.

In a first aspect, the Subtest 1 is about the Chroma deblocking filter, where the decision of applying the deblocking filter on Chroma samples is related to the decision applied to Luma samples at a corresponding boundary.

The second aspect (related to Subtests 2, 3 and 4) gives the result of the tC parameter derivation as proposed in JVET-K0203. It is asserted that the tC parameter which is linearly derived according to bitdepth in HEVC from a table, suffers from a lack of precision. JVET-K0203 proposes a bitdepth-dependent scalar multiplication approach. A combination of Subtest 2 and Subtest 1 is performed in Subtest 5.

The third aspect (related to Subtest 6) show the result of filtering 2 additional samples for Chroma while accessing no more samples than in the current design. The results are given on top of Subtest 5.

# Substest 1: Chroma filtering decision based on Luma decision

## Problem statement

Although proposals were made for a more fine-grained chroma filtering, e.g. in the image of the luma filtering, it has remained relatively simpler. As a consequence, chroma filtering is often “all or nothing”, therefore with a conservative, weak, filter needed. This is a reasonable design, as the increased data flow required for any more precise filtering was not seen as a good trade-off. This trade-off is at the core of the proposed changes.

## Technical proposals

### Luma-based conditional filtering

It could be argued that the relative behaviour between “first” (traditionally luma) component, and others (traditionally chroma), is often dependent on the colourspace, yet the “Chroma from Luma” method has been a staple of new techniques beyond HEVC’s. Concerning deblocking, however, if such high-detail content as the luma component suffers from blocking artifacts, then more than likely, chroma also does, and conversely if it does not.

The value ‘d’ computed in 8-371 of [1] to decide whether to filter a Luma edge is simply re-used to decide whether to filter the corresponding edge in Chroma.

As a consequence, in our “Subtest 1”, a Chroma edge is never filtered if the corresponding Luma edge is not filtered, which certainly helps data flow.

# Subtest 2: tC parameter adaptation to 10-bit internal bitdepth

HEVC was primarily studied around 8 bits content, as even the Main10 tests mostly relied on such content left-shifted to 10 bits. Furthermore, VTM, BMS, and particularly JEM, are producing content that have much lower distortion at the same input quantization step. We consider that VVC suffers or will suffer from the HEVC deblocking filter under the current HEVC settings, and can be simply tuned to prevent some of its negative downsides.

## Relation to bitdepth

### Problem statement

Table 8-12, “Derivation of threshold variables β′ and tC′ from input Q” in HEVC specifications [1] was designed partially based on 8 bits content, and size considerations. One parameter, β, is thus obtained from said table, and scaled according to bitdepth. To the authors, β can be seen as a feature-detection parameter, e.g. the flatness of the sample value curves around an edge for the deblocking filter. It is therefore critical so as to not smear meaningful edges or features.

Especially compared to AVC, the tC parameter is interesting, because it clips the sample value output by the selected deblocking filter within a range around the value of the input (to be filtered) sample. To the authors, it is basically related to the (maximum) error after quantization, and its likelihood, but not only, as seen in the decision process 8.7.2.5.6 for dSam variable from the HEVC specifications.

The relation from the table values and the values used in filtering is laid out simply, for example for luma in equations (8-351) and (8-353) of HEVC Specification [1]:

val = val’ * ( 1 << ( BitDepthY − 8 ) ).

In all instances, there is this proportional increase of the parameters according to bitdepth of the concerned component. We assert that this rule is sub-optimal, at least, and demonstrably, concerning tC.

Furthermore, the higher the bitdepth, the coarser the values. Indeed, table 8-12 is a quantized, fixed point representation of an ideal curve. Simple modelling under the shape exp (A*QP+B) yields A=0.113 and B=-2.64. From this, a higher precision content for table 8-12 of [1] can be derived for tC’.

### Technical proposal

To achieve sufficient accuracy whatever the bitdepth, we propose the following derivation:

tC = ( tC’ * scale[ BitDepth ] ) >> N

The value of N and those in tables 8-12 (which provides tC’) and scale[ ] are co-dependant, but the core of the assertion comes from the values of the ‘scale’ table.

### Search for an “optimal” scale

Optimal is very debatable, in particular concerning the deblocking filter. The following may affect what that value is:

- Encoder configuration, e.g. All Intra vs Low-Delay vs Random Access;

- Quality evaluation, whether subjective (MOS, …) or objective (PSNR, SSIM, CIE*, …);

- Colorspace used, and the weighting of components thereof, if not accounted by the metrics;

- Sequences used.

During HEVC development, it was customary, such as in [1], to do a weighted average of the PSNR values of the Y/Cb/Cr components, with the weights being 6:1:1. Study of the impact of changing the Chroma delta QP from [4] suggests a nice linear relation with weights of around 14:1:1.

To alleviate initially the case for visual validation, an objective search for a given bitdepth can be performed according to any number of points from the above. In all cases, whatever the considered objective metrics and scenarios/configurations, the rule

scale[ BitDepth ] = 1 << ( BitDepth − 8 )

was found to be suboptimal. The next figure represents the variation observed for JEM, various objective metrics including MS-SSIM and PSNR and All Intra, Random Access and Low Delay P scenarios.

The figure below represents a scaling found during the development of JEM. CE results from tests 2, 3 and 4 also indicate a similar behaviour for VTM/BMS 2.0.

As mentioned above, in the Subtest2, tC parameter is derived as follows:

tC = ( tC’ * scale[ BitDepth ] ) >> N

The value of tc’ is derived from a table of higher precision than the one in HEVC/VTM1.0, but still following an equivalent modelling according to exp(0.113*QP-2.64).

# Subtest 3: tC adaptation for 8-bit internal bit depth

As current CTCs [3] only mandate testing with an internal bitdepth of 10 bits, other bitdepths, and the related results, are obviously subject to proper testing conditions definition.

This specific Subtest3 provides the results when the “internal bitdepth” is set to 8 bits and use the scaling function “scale” as proposed in JVET-K0203 to derive the tC parameter.

For this test, the results are compared to anchors generated with an “internal bit depth” (i.e. using the VTM/VMS ‘--InternalBitDepth’ option) set to 8 bits (instead of 10 for the CTC[3]).

# Subtest 4: tC adaption for 8-bit internal bit depth following HEVC rule

This subtest 4 is to show the case where a scaling rule as defined in the HEVC specifications [1] is applied to derive the tC value for 8 bits from those we defined for 10 bits. This scaling rule corresponds to the following one:

scale[BitDepth] = 1 << ( BitDepth − 8 ),

This relation above amounts to dividing by 4 the scaling for 8 bit internal bitdepth. From this subtest and our observations, this derivation of tC values for 8 bits internal bitdepth is not the proper derivation to do.

Both objective and subjective results of this subtest 4 show lower performance compared to our proposal in subtest 3 where a more accurate adaptation is performed to derive the tc parameter for an internal bitdepth set to 8 bits.

Similarly to subtest 3, the results of this subtest 4 are compared to anchors generated with an internal bit depth set to 8 bits (instead of 10 in the CTC[3]).

# Subtest 5: combination of subtest 1 + subtest 2

The subtest 5 corresponds to the combination of subtest 1 and subtest 2 combining the according to the CTC [3].

# Subtest 6: Subtest 5 and additional Chroma filtering

The subtest 6 is the combination of subtest 5 with a modification of the Chroma filtering which is unconditionally modified to filter 2 additional samples, albeit without accessing more samples than in the current design.

Current filtering process for Chroma samples (section 8.7.2.5.8 of H.265 v5) are as follows:

Δ = Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )				(8-405)

p0′ = Clip1C( p0 + Δ )													(8-406)

q0′ = Clip1C( q0 − Δ ) 													(8-407)

The added filtering operations in JVET- K0203 correspond to the following:

Δ1 = Δ >> 1

p1′ = Clip1C( p1 + Δ1 )

q1′ = Clip1C( q1 – Δ1 )

# Overall results with CTC conditions

Full results are provided in separated Excel sheets.

Subtest 1							Subtest 2			     Subtest 3

Subtest 4							Subtest 5			     Subtest 6

# CTC results with ALF OFF

Full results are provided in separated Excel sheets.

Subtest 1							Subtest 2			     Subtest 3

Subtest 4							Subtest 5			     Subtest 6

# Conclusion

We recommend including this Chroma/Luma decision and finer tC parameter derivation of the deblocking filter as proposed in K0203 in the VVC draft standard text after confirmation of the visual testing to be performed in Macao.

# References

- “High efficiency video coding”- Recommendation ITU-T H.265 - 02/18

- JVET-K0203, “Non-CE2: Higher-precision modifications to VVC deblocking filters”, C. Gisquet, J. Taquet, G. Laroche, P. Onno, Ljubljana, SL, July. 2018

- JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, Ljubljana, SL, July. 2018

- JVET-H0045, “EE3: Test on Chroma QP offset,” Macau, CN, Oct. 2017

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).