| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0194 |
| --- | --- |

| Title: | CE4-related: On Merge Index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Christophe Gisquet Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents results of modifying the context derivation for Merge index coding. In the proposed modification, only the first bit of the Merge index is CABAC coded as it was in VTM1.0 instead of 5 bits in VTM2.0. Consequently the 4 other bits are CABAC bypassed.

It is reported that the proposed modification obtains the average BDR results of 0.03% -0.04% for respectively RA and LD configuration compared to VTM2.0.1.

# Introduction

The alternative temporal motion vector prediction (ATMVP) contribution JVET-K0346 [1] was adopted during the previous meeting. This adoption changed the Merge index coding with 2 modifications:

- 6 candidates instead of 5

- 5 contexts instead of 1

The following table summarizes the difference of the Merge index coding for the worst case.

Table 1 Number of contexts for Merge index coding for VTM1.0 and VTM2.0

|  | Number of contexts |  |
| --- | --- | --- |
| Merge index bits position | VTM 1.0 | VTM 2.0 |
| 0 | 1 | 1 |
| 1 | Bypass | 1 |
| 2 | Bypass | 1 |
| 3 | Bypass | 1 |
| 4 | n.a. | 1 |

# Proposed modification

The proposed modification sets only one context for the first bit of the Merge index, the 4 other bits are CABAC bypassed. This offers a reduction of 4 contexts compared to the current VTM2 design.

Table 2 summarizes the proposed modification in term of number of contexts associated to each bit of the Merge index compared to VTM 2.0. As depicted, the proposal saves 4 contexts and removes the 4 last merge index bits which are bypass coded. So it offers worst case complexity reduction.

Table 2 Number of contexts for Merge index coding for VTM2.0, proposed modifications 1 and 2

|  | Number of contexts |  |
| --- | --- | --- |
| Merge index bits position | VTM 2.0 | Proposal 1 |
| 0 | 1 | 1 |
| 1 | 1 | Bypass |
| 2 | 1 | Bypass |
| 3 | 1 | Bypass |
| 4 | 1 | Bypass |
| Total contexts | 5 | 1 |
| Total Bits CABAC | 5 | 1 |

# Experimental results

The proposed modification is implemented on top of the VTM2.0.1.

Table 3 gives the results of setting the Merge index context derivation as it was implemented in VTM1.0. This gives minor coding efficiency change with 0.03% for RA and -0.04% gain for LD configuration.

Table 3 Results of Merge index coding as VTM1.0 1 compared to VTM2.0.1

# Conclusion

In this contribution, a modification of the Merge index coding is proposed. This proposed modification removes 4 contexts for the Merge index as it was in the VTM1. This modification reduces the worst case decoding and parsing complexity with minor impact on coding efficiency.

# References

- JVET-K0346, “CE4-related: One simplified design of advanced temporal motion vector prediction (ATMVP)”, X. Xiu, Y. He, Y. Ye, H. Jang, J. Nam, S. Kim, J. Lim, H. Chen, H. Yang, J. Chen, Ljubljana, SL, July. 2018

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).