| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 October 2018 | Document: JVET-L0196 |
| --- | --- |

| Title: | CE9.2.7: Complexity reduction on decoder-side motion vector refinement (DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong (Daniel) Luo, Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-609-436-0885 Daniel.luo@InterDigital.com Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 9.2.7 on complexity reduction of decoder-side motion vector refinement (DMVR). In this CE test, the existing DMVR design in the benchmark set BMS2.1rc1 is used as the base. Additionally, one early termination method is applied to terminate the DMVR process of one bi-prediction merge candidate if its L0 and L1 prediction signals are sufficiently similar. Simulation results show that compared to BMS2.1rc1 with VTM configuration anchors, the proposed DMVR method provides on average {Y, U, V} BD-rate savings of { -1.64%, -1.72%, -1.81%} for RA with average encoding and decoding time of 103% and 114%.

# Introduction

At the 10th JVET meeting, decoder-side motion vector refinement (DMVR) was included in the benchmark set BMS-1.0 reference software [1]. The DMVR tool is designed to improve the efficiency of motion compensation by refining the motion vectors (MVs) based on the prediction signals in list L0 and L1. In the BMS-1.0 [1], the DMVR is always enabled for each non-subblock merge candidate that is bi-predicted, which introduces significant complexity increase to both encoder and decoder. In [2], an early termination method is applied to the DMVR. Specifically, the sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the initial MVs of the current CU is calculated. If the SAD is no larger than a predefined threshold, i.e., 2BDepth-119 per sample, the DMVR is skipped; otherwise, the DMVR is still applied to refine the two motion vectors of the current block. At the 11th JVET meeting, the method is studied [3]. Core Experiment (CE-9) includes this method as test 2.7 for further evaluation [4]. This document reports the simulation results of the CE test.

# Simulation results

The proposed method was implemented based on the BMS-2.1rc1 reference software [5] and tested using VTM configurations. The test condition and evaluation criteria that are used in CE9 [4] are applied. Specifically, the proposed method is compared with the BD-rate performance of BMS2.1rc1 VTM configuration.

Table 1 The performance of the proposed DMVR method, compared to BMS2.1rc1 with VTM config

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.80% | -1.61% | -1.91% | 103% | 110% |
| Class A2 | -2.33% | -2.43% | -2.50% | 102% | 111% |
| Class B | -1.31% | -1.46% | -1.48% | 104% | 115% |
| Class C | -1.41% | -1.59% | -1.64% | 103% | 118% |
| Class E |  |  |  |  |  |
| Overall | -1.64% | -1.72% | -1.81% | 103% | 114% |
| Class D | -1.75% | -1.76% | -1.86% | 105% | 121% |

Additionally, one complementary comparison is provided in Table 2 where the results of the proposed method is compared against the results of BMS2.1rc1 with VTM config and DMVR being enabled.

Table 2 The performance of the proposed DMVR method, compared to the results of BMS2.1rc1 with VTM config and DMVR being enabled

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config and DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | 0.01% | 100% | 98% |
| Class A2 | 0.00% | 0.02% | 0.03% | 99% | 97% |
| Class B | 0.01% | 0.02% | 0.05% | 100% | 97% |
| Class C | -0.01% | 0.02% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.04% | 100% | 98% |
| Class D | -0.01% | -0.01% | -0.02% | 100% | 100% |

# References

BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

X. Xiu, Y. He, Y. Ye, “Complexity reduction on decoder-side motion vector refinement (DMVR)”, JVET-K0342, July 2018, Ljubljana, Slovenia

S. Esenlik, Y.-W. Chen, F. Chen, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-K1029, July 2018, Ljubljana, Slovenia.

BMS2.1rc1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.1rc1.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).