| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0200-v1 |
| --- | --- |

| Title: | CE9-Related: Region-based Template Matching for DMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gayathri Venugopal Karsten Müller Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002 287 gayathri.venugopal@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

The region-based template matching method is proposed for decoder-side motion vector derivation in this document. The proposed method achieves an overall BD-rate gain of x with y and z run-time for decoder and encoder respectively for random access configuration against BMS-2.1rc1 as VTM-2.0.1.

# Technical description

The region-based template matching (RTM) is one of the tools under study in the core experiment (CE8) group [1]. This contribution proposes RTM for decoder-side motion vector derivation (DMVD).

Consider Figure 1. The two lines of reconstructed samples present above and left of a block is referred as the template.  is the template of the current block and  is any template present in the reference picture. The best template match for  is called , which is the template  that gives the least error value when compared against . The error minimizing metric used is sum of squared differences (SSD).

The search window for finding  is divided into five regions as shown in Figure 2, where   is the central position and , ,  are the region sizes such that the widow size is given by,

(1)

Figure 1: Template matching for DMVD

Figure 2: Five regions of RTM where C is the central position

The central position  is decided by generating a list of candidates and choosing the one that leads to the least SSD error value when compared against . This list is formed by the templates of the candidates of the advanced motion vector prediction (AMVP) candidate list of the current block. In addition to that, the template in the same position of  in the reference picture is also included.

Given a region, the proposed method finds four predictors from each reference list through template matching. Thus, there are four and eight number of predictors for a uni-predictive and bi-predictive picture respectively. The final prediction signal is a linear combination of these predictor blocks as detailed in subsection 1.1.

## Prediction signal

As mentioned earlier, the proposed method finds four and eight number of predictors for uni-predictive and bi-predictive picture respectively. Then they are sorted in the ascending order of their SSD error values. The final prediction is given by,

(2)

where  is the set of predictors and  is the corresponding weight. l is the number of predictors in the after the sorted list which has an error value less than or equal to a threshold error. The threshold error value is twice the error value of the first predictor in the sorted list.

Table 1: No.of predictors and the corresponding weights

| No.of sorted predictors l |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 2 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 4 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 5 | 2 | 2 | 1 | 1 | 1 | 1 | 0 | 0 |
| 6 | 2 | 2 | 1 | 1 | 1 | 1 | 0 | 0 |
| 7 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

## The search method

The proposed RTM tool uses a linear search method. For each region, the search for the best template match starts from the starting position S and progresses towards the outer borders. The starting position of the search method for Region 1, 2, 3, 4, 5 are   respectively (Figure 3). The way the search progresses depends on the region type. For Region1 (Figure 4), the starting position is compared with the current template in the beginning. In the first iteration, the templates that are present at 1-sample positions from S are compared. In the second iteration, the templates that are present at 2-sample positions from S are compared. This continues until it reaches the outer borders of the region.

Figure 3: Starting position of the regions. are the starting positions of the regions 1, 2, 3, 4, 5 respectively

Figure 4: Search progression for Region 1

For other regions also the search is carried in the same manner. However, only in one direction. For Region2 and 4, the search progresses towards its bottom-right corner (Figure 5). In the case of Region3 and 5, the search is done towards its top-left corner (Figure 6).

Figure 5: Search progression for Region 2

Figure 6: Search progression for Region 3

As mentioned earlier, given a region, four predictors are found through template matching from each reference list. The rule for the search algorithm is explained in the flowchart in Figure 7.

Figure 7: The rule applied for finding the best template matches.

## Signaling

The proposed RTM method is added as a merge mode. Figure 3 shows the syntax elements for RTM as a DMVD method. The Context-Adaptive Binary Arithmetic Coding (CABAC) is applied for signaling all the bins.

Figure 8: Syntax of RTM as DMVD method where MCP is motion compensated prediction

# Results

According to the test conditions detailed in [2], the proposed method is implemented on BMS-2.1rc1 software and evaluated for the random-access (RA) configuration. The anchor is VTM (using BMS-2.1rc1 as VTM-2.0.1). The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, turbo boost off) with Scientific Linux 7.4 OS and GCC 7.2.1 compiler.

# Cross-check

The cross-check of this proposal is carried out by Tencent.

# Conclusions and recommendations

# References

[1] G. Venugopal, K. Müller, H. Schwarz, D. Marpe and T. Wiegand, " CE8: Intra Region-based Template Matching (Test 8.1), JVET-K0048," Ljubljana, SI, 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, "JVET common test conditions and software reference configurations", JVET-K1010, 11st JVET meeting, Jul. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).