| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0201-v2 |
| --- | --- |

| Title: | AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Bordes Edouard François Fabrice Le Léannec | Email: | philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document presents results of tools testing of Weighted Prediction (WP) and Generalized Bi-prediction (GBi) on Fade sequences generated from the common test sequences.

The WP has been specially designed to compensate global illumination variation in video sequences such as Fades. WP is only invoked at slice level, whereas GBi invokes different weights at CU level. At previous JVET Meeting (Ljubljana) it was suggested evaluating both tools with Fade sequences.

For that purpose, Fade sequences have been generated from common test set with same fading tool as used for HEVC in JCT-VC.

It is reported that under BMS-2.1 configurations, using WP tool (WP=1 and GBi=0), the BD rate changes (Y/Cb/Cr) relative to the BMS-2.1 anchors are:

- In RA, -12.0%/-13.1%/-12.7% and in LB -29.0%/-38.5%/-37.5% for CTC / WP tool / Fade Black sequences.

- In RA, -15.4%/-16.3%/-16.3% and in LB -29.9%/-39.0%/-38.1% for CTC / WP tool / Fade White sequences.

It is reported that under BMS-2.1 configurations, using GBi tool (WP=0 and GBi=1), the BD rate changes relative to the BMS-2.1 anchors are:

- In RA, -1.01%/-1.28%/-1.26% for CTC / GBi tool / Fade Black sequences.

- In RA, -0.88%/-1.19%/-1.23% for CTC / GBi tool / Fade White sequences.

Comparison with performance of WP in HM16.19 is also provided.

# Test Fade sequences

To test and validate the weighting prediction tool, a Fading tool has been provided in [1] and is also included in this contribution. This software tool creates linear fading sequences from common test sequences.

In our experiments, we use same conditions as used for AVC [2] and HEVC [3]: a linear fade is applied to the first 2 seconds of the regular sequence set and the experiments are done with the 2-second sequences. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half (Figure 1). The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

Figure 3: Linear Fade-to-black or Fade-to-white weights applied on the sequence and the black/white pictures respectively.

# Experimental results

Table 1-1: BD-rate gains of WP in BMS-2.1 .

| WP | Over BMS-2.1 - WP on/off - FadeBlack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.1% | -2.8% | -5.7% | 229% | 85% |  |  |  |  |  |
| Class A2 | -13.9% | -15.1% | -15.6% | 186% | 79% |  |  |  |  |  |
| Class B | -13.9% | -17.0% | -15.0% | 174% | 79% | -23.2% | -34.0% | -33.2% | 111% | 92% |
| Class C | -13.4% | -14.3% | -13.1% | 136% | 77% | -18.7% | -26.9% | -26.3% | 101% | 90% |
| Class E |  |  |  |  |  | -52.5% | -61.6% | -59.6% | 78% | 76% |
| Overall | -12.0% | -13.1% | -12.7% | 174% | 79% | -29.0% | -38.5% | -37.5% | 98% | 87% |
| Class D | -12.2% | -14.6% | -14.6% | 109% | 56% | -17.7% | -25.9% | -27.7% | 76% | 90% |

| WP | Over BMS-2.1 - WP on/off - FadeWhite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -9.8% | -8.6% | -10.9% | 208% | 81% |  |  |  |  |  |
| Class A2 | -18.7% | -20.8% | -20.3% | 177% | 80% |  |  |  |  |  |
| Class B | -17.5% | -19.0% | -18.5% | 168% | 77% | -24.2% | -34.2% | -33.9% | 102% | 94% |
| Class C | -14.5% | -15.3% | -14.7% | 128% | 76% | -20.1% | -27.8% | -27.4% | 88% | 96% |
| Class E |  |  |  |  |  | -52.5% | -61.9% | -59.5% | 74% | 75% |
| Overall | -15.4% | -16.3% | -16.3% | 165% | 78% | -29.9% | -39.0% | -38.1% | 89% | 89% |
| Class D | -13.2% | -16.3% | -15.7% | 106% | 78% | -17.4% | -25.1% | -27.2% | 75% | 91% |

Table 1-2: BD-rate gains of GBI in BMS-2.1 .

| GBI | Over BMS-2.1 - GBI on/off - FadeBlack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.36% | -0.65% | 105% | 101% |  |  |  |  |  |
| Class A2 | -0.82% | -1.26% | -1.18% | 108% | 100% |  |  |  |  |  |
| Class B | -1.17% | -1.31% | -1.29% | 103% | 102% |  |  |  |  |  |
| Class C | -1.38% | -1.95% | -1.74% | 103% | 99% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.01% | -1.28% | -1.26% | 104% | 101% |  |  |  |  |  |
| Class D | -0.92% | -1.60% | -1.44% | 104% | 74% |  |  |  |  |  |

| GBI | Over BMS-2.1 - GBI on/off - FadeWhite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.17% | 0.07% | 93% | 102% |  |  |  |  |  |
| Class A2 | -0.44% | -0.71% | -0.79% | 98% | 100% |  |  |  |  |  |
| Class B | -1.51% | -1.87% | -2.23% | 98% | 101% |  |  |  |  |  |
| Class C | -1.00% | -1.46% | -1.30% | 107% | 101% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.88% | -1.19% | -1.23% | 99% | 101% |  |  |  |  |  |
| Class D | -0.77% | -1.36% | -0.95% | 101% | 103% |  |  |  |  |  |

For information, the performance of WP tool in BMS-2.1 is of same order as the performance of WP in HM16.19 (Table 1-3).

Table 1-3: BD-rate gains of WP in HM16.19 .

| WP | Over HM16.19 – WP on/off - FadeBlack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -7.1% | -7.3% | -9.5% | 255% | 105% |  |  |  |  |  |
| Class A2 | -18.1% | -22.1% | -21.7% | 231% | 96% |  |  |  |  |  |
| Class B | -17.3% | -23.2% | -20.8% | 216% | 94% | -30.4% | -43.0% | -40.6% | 160% | 92% |
| Class C | -16.1% | -17.7% | -17.7% | 204% | 81% | -24.1% | -32.6% | -30.5% | 159% | 92% |
| Class E |  |  |  |  |  | -64.4% | -69.7% | -68.0% | 141% | 85% |
| Overall | -15.1% | -18.3% | -17.8% | 223% | 93% | -36.8% | -46.2% | -44.1% | 155% | 90% |
| Class D | -16.9% | -19.6% | -19.9% | 174% | 47% | -23.4% | -31.7% | -34.6% | 143% | 94% |

| WP | Over HM16.19 – WP on/off - FadeWhite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -12.2% | -13.6% | -18.6% | 247% | 99% |  |  |  |  |  |
| Class A2 | -24.2% | -28.5% | -27.8% | 238% | 94% |  |  |  |  |  |
| Class B | -20.8% | -24.8% | -23.4% | 218% | 93% | -31.8% | -43.1% | -41.4% | 161% | 88% |
| Class C | -18.1% | -20.4% | -20.2% | 237% | 89% | -25.6% | -33.8% | -31.8% | 142% | 94% |
| Class E |  |  |  |  |  | -65.0% | -70.5% | -68.6% | 131% | 79% |
| Overall | -19.0% | -22.1% | -22.5% | 232% | 93% | -38.0% | -46.8% | -45.0% | 147% | 87% |
| Class D | -18.4% | -20.9% | -21.8% | 202% | 98% | -23.5% | -32.9% | -34.6% | 141% | 91% |

# Conclusion

It is reported that under BMS-2.1 configurations, the performance of WP tool for coding fade video sequences with fade is of same order as the performance of WP in HEVC (HM16.19) and that WP tool outperforms GBi tool in these conditions.

# References

- P.Bordes, “Weighted Prediction,” JVT Document, JVT-F265, Torino, IT, 14-22 July, 2011.

- Yoshihiro Kikuchi and Takeshi Chujoh,” Simplification of the Weighted Prediction and Verification result,” JVT Document, JVT-F077, Awaji, December, 2002.

- A.Fujibayashi, S.Kanumuri and F.Bossen,” TE12: Performance of Partition Based Illumination Compensation (PBIC),” JVT Document, JCTVC-C041, Guangzhou, October, 2010.