| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0206-v3 |
| --- | --- |

| Title: | CE12: report of CE12-3 and CE12-5 on in-loop refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. François, C. Chevance, F. Hiron (Technicolor) | Tel: Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

The contribution reports results from CE12-3 and CE12-5 relating to in-loop cross-component chroma refinement. CE12-3 focuses on HDR content, using HDR CTCs, and CE12-5 on SDR content, using SDR CTCs. In the tested implementation, the refinement is applied to the chroma samples after the deblocking filter, both in encoder and in decoder. The chroma samples are corrected using a luma-dependent scaling factor, the cb and cr scaling tables being signalled per slice using piece-wise linear models. The tool is also tested in combination with CE12-2 dealing with in-loop reshaping. In this case, refinement is applied after the inverse reshaping step in the encoder and in the decoder.

A performance summary in RA, fixed QP configuration, is provided for HDR PQ, HDR HLG and SDR content in the tables below, using metrics relevant for these three types of content. The tool benefits are reportedly mostly observed on chroma components of HDR content, with an average BD-rate gain of around 4% in (w)PSNR-U and of around 6-7% in (w)PSNR-V, and no impact on luma. It is also reported that the encoding and decoding runtime impact is negligible. It is reported that similar gains are observed on HDR PQ content when the tool is combined with the in-loop reshaping of CE12-2.

| RA, fixed QP | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| Average HDR-B (PQ) | -1.8% | 0.0% | 0.0% | -4.1% | -7.4% |

| RA, fixed QP | DE100 | PSNRL100 | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- |
| Average HDR-A (HLG) | -2.3% | 0.0% | 0.0% | -0.9% | -3.0% |

| RA, fixed QP | psnrY | psnrU | psnrV |
| --- | --- | --- | --- |
| Average SDR | 0.0% | -1.3% | -1.0% |

# Introduction

Dynamic range adaptation (DRA) was proposed in JVET-J0021 and JVET-J0022 for HDR category, and further studied in CE12. The current contribution aims at reporting results from different variants of this technology.

# Tool description

The in-loop cross-component chroma refinement process operates to the chroma signal (UR, VR) as follows:

- Urefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineU[ YRcoloc ] * ( UR – 2Bchr – 1 ))

- Vrefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineV[ YRcoloc ] * ( VR – 2Bchr – 1 ))

where YRcoloc is the co-located luma sample.

LUTrefineU, LUTrefineV are built from piece-wise linear (PWL) tables, signaled per slice. When tables are not signaled, no refinement applies to the current slice. Pivot points of the PWL are equi-distant, and the log2 of the PWL size (noted log2_S) is signaled in the slice header (the resulting actual number of points being equal to 2log2_S + 1).

In the proposed implementation, the process can be applied in different locations in the decoding process: after deblocking, after SAO, after ALF. In practice, in the reported experiments, the refinement process is applied after the deblocking filter.

At encoder side, the PWL pivot points are estimated to minimize the rate-distortion cost. In practice they are only derived for slices of temporal level 0. The refinement does not apply for the slices of higher temporal level.

A proposed syntax and decoder process is provided in the annex of this document.

# Results

The tools described above are implemented within the BMS-2.0.1 software, configured using the macros as VTM-2.0.1 (JEM_TOOLS macros deactivated). Reference are HDR anchors as defined in the HDR CTCs. Complete simulation results are provided in the attachments to this contribution and summary for all requested metrics are provided in this document.

In the reported experiments, Refinement is applied after the deblocking filter and prior to SAO. The refinement parameters are signalled for each slice of temporal level 0.

In the reported results below, BD-rate for HDR metrics (DE100 and PSNRL100) are reported. For PSNR, BD-rate for using normal PSNR are reported for HLG, and BD-rate for using wPSNR are reported for PQ.

## CE12-3 test results (in-loop refinement for HDR content)

Summary of the simulation results produced in CE12-3 (in-loop chroma refinement for HDR) are provided in Table 1 and Table 2 for AI and RA, fixed QP configuration.

Table 1. CE12-3 vs HDR anchor, AI fixed QP constraints

|  | DE100 | PSNRL100 | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- |
| DayStreet_300 | -1.8% | 0.0% | 0.0% | -0.6% | -2.1% |
| FlyingBirds_300 | -8.5% | 0.0% | 0.0% | -2.5% | -10.4% |
| PeopleInShoppingCenter_300 | -0.7% | 0.0% | 0.0% | -0.9% | -0.9% |
| SunsetBeach_300 | -1.3% | 0.1% | 0.0% | -1.0% | -4.1% |
| Average HDR-A | -3.1% | 0.0% | 0.0% | -1.3% | -4.4% |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | -0.5% | 0.0% | 0.0% | -1.8% | -1.7% |
| SunRise | -0.4% | 0.0% | 0.1% | -1.3% | 0.0% |
| ShowGirl2 | -0.7% | 0.1% | 0.1% | -7.3% | -8.1% |
| BalloonFestival | -0.3% | 0.1% | 0.1% | -2.3% | -1.7% |
| Hurdles | -11.6% | 0.1% | 0.1% | -9.5% | -20.8% |
| Starting | -0.3% | 0.1% | 0.1% | -3.9% | -9.1% |
| Cosmos1 | -3.0% | -0.2% | 0.1% | -17.1% | -37.6% |
| Average HDR-B | -2.4% | 0.0% | 0.1% | -6.2% | -11.3% |
| Enc Time[%] | 97% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

Table 2. CE12-3 vs HDR anchor, RA fixed QP constraints

|  | DE100 | PSNRL100 | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- |
| DayStreet_300 | -1.5% | 0.0% | 0.0% | -0.6% | -1.5% |
| FlyingBirds_300 | -8.2% | 0.0% | 0.0% | -2.4% | -8.6% |
| PeopleInShoppingCenter_300 | -0.7% | 0.0% | 0.0% | -0.9% | -0.9% |
| SunsetBeach_300 | 1.3% | 0.0% | 0.0% | 0.4% | -0.9% |
| Average HDR-A | -2.3% | 0.0% | 0.0% | -0.9% | -3.0% |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | -0.1% | 0.0% | 0.0% | -1.0% | -0.8% |
| SunRise | -0.2% | 0.0% | 0.1% | -0.4% | 0.2% |
| ShowGirl2 | -0.7% | 0.1% | 0.1% | -6.6% | -5.0% |
| BalloonFestival | 0.0% | 0.0% | 0.0% | -1.3% | -0.8% |
| Hurdles | -10.3% | 0.1% | 0.1% | -8.6% | -14.8% |
| Starting | -0.7% | 0.0% | 0.0% | -3.1% | -5.1% |
| Cosmos1 | -1.0% | -0.1% | 0.0% | -7.5% | -25.3% |
| Average HDR-B | -1.8% | 0.0% | 0.0% | -4.1% | -7.4% |
| Enc Time[%] | 103% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |

## CE12-2 test results (in-loop refinement on top of the in-loop reshaping for HDR)

The in-loop refinement has also been tested in CE12-2 (in-loop reshaping). In these results, anchor correspond to CE12-2 (in-loop reshaping for HDR, with ILFOPT=3). Test reported in Table 3 corresponds to using chroma refinement on top of the in-loop reshaping. Test reported in Table 4 corresponds to using luma and chroma refinement on top of the in-loop reshaping. Results are reported for RA, fixed QP configuration.

Table 3. CE12-2 with in-loop chroma refinement vs CE12-2, RA fixed QP constraints

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| Market | -0.3% | 0.0% | 0.0% | -1.2% | -0.9% |
| SunRise | -0.3% | 0.1% | 0.1% | -0.9% | 0.1% |
| ShowGirl2 | -0.4% | 0.0% | 0.0% | -5.8% | -5.6% |
| BalloonFestival | 0.0% | 0.1% | 0.0% | -1.6% | -1.4% |
| Hurdles | -9.5% | 0.0% | 0.0% | -6.4% | -14.6% |
| Starting | 0.0% | 0.1% | 0.1% | -3.1% | -5.2% |
| Cosmos1 | -0.9% | 0.0% | 0.1% | -5.7% | -20.7% |
| Average HDR-B | -1.6% | 0.0% | 0.0% | -3.5% | -6.9% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |

Table 4. CE12-2 with in-loop luma and chroma refinement vs CE12-2, RA fixed QP constraints

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| Market | -0.2% | 0.0% | 0.0% | -1.1% | -0.7% |
| SunRise | -0.2% | 0.1% | 0.1% | -0.8% | 0.1% |
| ShowGirl2 | -0.4% | 0.0% | 0.0% | -5.9% | -5.9% |
| BalloonFestival | -0.1% | 0.1% | 0.1% | -1.5% | -1.3% |
| Hurdles | -9.5% | 0.0% | 0.0% | -6.4% | -14.7% |
| Starting | 0.0% | 0.1% | 0.1% | -3.1% | -5.1% |
| Cosmos1 | -1.2% | 0.0% | -0.4% | -6.0% | -21.4% |
| Average HDR-B | -1.6% | 0.0% | 0.0% | -3.5% | -7.0% |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |

Table 5 reports results in AI fixed QP configuration, using luma and chroma refinement.

Table 5. CE12-2 with in-loop luma and chroma refinement vs CE12-2, AI fixed QP constraints

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| Market | 0.0% | 0.0% | 0.0% | -1.8% | -1.0% |
| SunRise | -0.1% | 0.1% | 0.0% | -0.5% | 0.1% |
| ShowGirl2 | -0.3% | 0.0% | 0.0% | -7.1% | -5.9% |
| BalloonFestival | -0.4% | 0.1% | 0.0% | -2.8% | -1.8% |
| Hurdles | -9.1% | 0.0% | 0.0% | -7.4% | -19.1% |
| Starting | 0.2% | 0.0% | 0.1% | -3.7% | -8.5% |
| Cosmos1 | -4.7% | -1.1% | -2.6% | -15.3% | -34.0% |
| Average HDR-B | -2.0% | -0.1% | -0.4% | -5.5% | -10.0% |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |

## CE12-5 test results (in-loop refinement for SDR content)

Summary of the simulation results produced in CE12-5 (in-loop chroma refinement for SDR) are provided in Table 6, Table 7 and Table 8 for AI, RA and LDB, fixed QP configuration.

Table 6. CE12-5 vs SDR anchor, AI fixed QP constraints

|  | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.0% | -0.8% | -0.6% | 102% | 100% |
| Class A2 | 0.0% | -0.4% | -0.2% | 101% | 100% |
| Class B | 0.1% | -1.9% | -2.0% | 101% | 100% |
| Class C | 0.0% | -1.0% | -0.6% | 100% | 100% |
| Class E | 0.1% | -0.8% | -0.7% | 101% | 100% |
| Overall | 0.0% | -1.1% | -0.9% | 101% | 100% |
| Class D | 0.0% | -0.2% | -0.1% | 100% | 100% |
| Class F (optional) | 0.1% | -1.6% | -1.8% | 100% | 100% |

Table 7. CE12-5 vs SDR anchor, RA fixed QP constraints

|  | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.0% | -0.8% | -0.3% | 101% | 100% |
| Class A2 | 0.0% | -0.6% | -0.2% | 100% | 101% |
| Class B | 0.1% | -2.3% | -2.1% | 103% | 101% |
| Class C | 0.0% | -0.9% | -0.6% | 100% | 101% |
| Overall | 0.0% | -1.3% | -1.0% | 101% | 101% |
| Class D | 0.0% | -0.5% | -0.3% | 101% | 100% |
| Class F (optional) | 0.0% | -1.1% | -1.1% | 101% | 101% |

Table 8. CE12-5 vs SDR anchor, LDB fixed QP constraints

|  | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.1% | -3.7% | -4.2% |  |  |
| Class C | 0.0% | -1.8% | -0.7% |  |  |
| Class E | 0.0% | -0.2% | -0.3% |  |  |
| Overall | 0.0% | -2.2% | -2.1% |  |  |
| Class D | 0.1% | -0.5% | -0.1% |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! |  |  |

# Conclusion

Reported results in the previous section tend to show that benefits from the cross-component chroma refinement is much higher on HDR content than on SDR content. For example, in RA configuration, the observed gain is around 4-7% in PSNR or wPSNR for the chroma components, as summarized in the tables below. The impact on the luma component is negligible. The encoding/decoding runtime remain in the range of 100% (considering the uncertainty level of the runtime variations estimation).

| RA, fixed QP | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| Average HDR-B (PQ) | -1.8% | 0.0% | 0.0% | -4.1% | -7.4% |

| RA, fixed QP | DE100 | PSNRL100 | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- |
| Average HDR-A (HLG) | -2.3% | 0.0% | 0.0% | -0.9% | -3.0% |

| RA, fixed QP | psnrY | psnrU | psnrV |
| --- | --- | --- | --- |
| Average SDR | 0.0% | -1.3% | -1.0% |

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Syntax

The refinement-related syntax is added in SPS and in slice header.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| refinement_enable_flag | u(1) |
| … |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( refinement_enable_flag ) { |  |
| refine_crosscomp_table_new_flag | u(1) |
| if ( refine_crosscomp_table_new_flag ) { |  |
| refine_table_log2_size | u(4) |
| for ( comp = 1; comp < 3; comp++ ) { |  |
| refine_comp_flag[ comp ] | u(1) |
| if ( refine_comp_flag[ comp ] ) { |  |
| for ( i = 0; i < refineTableSize; i++ ) |  |
| { |  |
| refine_table_comp_value[ comp – 1 ][ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … | ue(v) |
| } |  |

#### 8.x.x Coding tree block modification process

#### 8.x.x.1 General

Inputs to this process are:

–	the picture sample array recPictureY for the colour component 0,

–	the picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying the colour component index, cIdx being greater than 0

–	a pair of variables ( rx, ry ) specifying the coding tree block location,

–	the coding tree block width nCtbSw and height nCtbSh.

Output of this process is a modified picture sample array refinePicture for the colour component cIdx.

The array lutScaling of size ( 1 << BitDepthY ) is derived by invoking clause 8.x.x.2 with cIdx as input and the variable interval and the array lutScaling as output.

The location ( xCtb, yCtb ), specifying the top-left sample of the current coding tree block for the colour component cIdx relative to the top-left sample of the current picture component cIdx, is derived as follows:

( xCtb, yCtb ) = ( rx * nCtbSw, ry * nCtbSh )		(6-xxx)

The sample locations inside the current coding tree block are derived as follows:

( xSi, ySj ) = ( xCtb + i, yCtb + j )		(6-xxx)

( xYi, yYj ) = ( xSi * SubWidthC, ySj * SubHeightC )	(6-1)

The variables neutralVal, offset and shift are derivbed as follows:

–	neutralVal is set equal to 64

–	offset is set equal to ( 1 << ( BitDepthC – 1 ) )

–	shift is set equal to ( 6 + log2( interval ) ).

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, the following applies:

–	If refine_comp_flag[ cIdx – 1 ] is equal to 0, refinePicture[ xSi ][ ySj ] is not modified.

–	Otherwise, the following applies:

- The variable Yc is set equal to recPicture[ xSi << 1 ][ ySj << 1 ]

- refinePicture[ xSi ][ ySj ] is derived as follows

refinePicture[ xSi ][ ySj ] = ( offset * ( 1 << shift ) + ( 1 << ( shift – 1 ) + 
                                               ( recPicture[ xSi ][ ySj ] – offset ) * lutScaling[ Yc ] ) >> shift	(6-xxx)

#### 8.x.x.2 Scaling look-up-table derivation

Inputs to this process are:

–	a variable cIdx specifying the colour component index, cIdx being greater than 0

Output of this process is a variable interval and an array lutScaling of size ( 1 << BitDepthY ).

The variable neutralVal is set equal to 64.

The variable interval is set equal to ( 1 << BitDepthY ) / ( refineTableSize – 1 ).

The array ccLut of size refineTableSize is derived as follows:

–	For pt = 0..refineTableSize, ccLut[ pt ] is derived as follows:

ccLut[ pt ] = neutralVal + refine_table_comp_value[ cIdx – 1 ][ pt ] 	(6-xxx)

For pt = 0..( refineTableSize – 2 ), the following applies:

–	For i = ( pt * interval ).. ( ( pt + 1 ) * interval – 1 ), lutScaling[ i ] is derived as follows:

lutScaling[ i ] = interval * ccLut[ pt ] + ( i – pt * interval ) * ( ccLut[ pt + 1 ] – ccLut[ pt ] ) 	(6-xxx)