| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0211-v1 |
| --- | --- |

| Title: | CE13: Results on CE13.3.2, CE13.4.3 and CE13.7.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Johannes Sauer Max Bläser | Tel: Email: | +49241-80-27678 sauer@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document relates to the CE13 sub-experiments CE13.3.2, CE13.4.3 and CE13.7.7. It reports the results of geometry padding and adapted in-loop filters for 360° video as proposed initially in document JVET-J0023, implemented into the BMS-2.0.1 software. The VTM configuration was used for the anchor and experiments. 
CE13.3.2 investigates HEC with face based geometry padding of reference pictures. Compared to the BMS-2.0.1 reference, CE13.3.2 obtains average Y E2E WS-PSNR BD-rate loss of 0.28% for Random Access.
CE13.4.3 investigates HEC with in-loop filters using spherical neighbors. Compared to the BMS-2.0.1 reference, CE13.4.3 obtains average Y E2E WS-PSNR BD-rate loss of 1.20% for Random Access.
CE13.7.7 is the combination of CE13.3.2 and CE13.4.3. Compared to the BMS-2.0.1 reference, CE13.7.7 obtains average Y E2E WS-PSNR BD-rate loss of 0.24% for Random Access.
It is likely that the results are still affected by a bug in the software implementation.

# Introduction

The experiment description is given in JVET-K1033. The tools are implemented at picture level. The implementation differs from the one in JVET-J0023. It was re-implemented using 360Lib as a basis. Around each HEC face a padding area was added. Before loop filters are executed this area is filled with the samples of the spherical neighbors without reprojection. For geometry padding the same padding area is filled with reprojected pixels before the pictures is stored in the RPS. The reprojection is matching the HEC format. The used geometry type and face arrangement are signaled in the SPS. The software was further, modified such that Intra prediction works correctly across face boundaries, ignoring the added padding area. However, this does not work completely yet, which is the cause of the BD-rate loss.
ALF was disabled for CE13.3.2 as a still unresolved bug led to an encoder/decoder mismatch.

# Results

Coding results have been obtained according to the common test conditions for 360° video given in
JVET-K1012. The results for the three sub-experiments are:

|  | CE13.3.2 VS PHEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E2E WS-PSNR |  |  | E2E S-PSNR-NN |  |  | VP #1 PSNR |  |  | VP #2 PSNR |  |  | C WS-PSNR |  |  | C S-PSNR-NN |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | EncT | DecT |
| Class S1 | 0.16% | 6.03% | 4.75% | 0.19% | 5.96% | 4.82% | 2.27% | 3.72% | 2.32% | 2.91% | 3.87% | 2.38% | 0.95% | 8.79% | 7.11% | 1.01% | 8.85% | 7.07% | 99% | 349% |
| Class S2 | 0.46% | 2.23% | 1.50% | 0.47% | 2.21% | 1.44% | 0.92% | 1.61% | 1.62% | 2.08% | 0.45% | -0.09% | 1.92% | 4.72% | 3.38% | 1.88% | 4.84% | 3.38% | 100% | 287% |
| Overall | 0.28% | 4.51% | 3.45% | 0.30% | 4.46% | 3.47% | 1.73% | 2.87% | 2.04% | 2.58% | 2.51% | 1.39% | 1.34% | 7.16% | 5.62% | 1.36% | 7.25% | 5.60% | 99% | 322% |

|  | CE13.4.3 VS PHEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E2E WS-PSNR |  |  | E2E S-PSNR-NN |  |  | VP #1 PSNR |  |  | VP #2 PSNR |  |  | C WS-PSNR |  |  | C S-PSNR-NN |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | EncT | DecT |
| Class S1 | 0.71% | 6.34% | 5.15% | 0.76% | 6.36% | 5.15% | 2.59% | 3.99% | 2.64% | 3.43% | 4.33% | 2.70% | 1.54% | 9.18% | 7.44% | 1.60% | 9.24% | 7.42% | 101% | 99% |
| Class S2 | 1.94% | 4.12% | 3.43% | 1.90% | 4.10% | 3.25% | 2.17% | 3.31% | 2.86% | 3.42% | 1.74% | 1.51% | 3.44% | 6.70% | 5.45% | 3.39% | 6.63% | 5.24% | 101% | 117% |
| Overall | 1.20% | 5.45% | 4.46% | 1.22% | 5.46% | 4.39% | 2.42% | 3.72% | 2.73% | 3.43% | 3.30% | 2.22% | 2.30% | 8.19% | 6.64% | 2.31% | 8.20% | 6.55% | 101% | 106% |

|  | CE13.7.7 VS PHEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E2E WS-PSNR |  |  | E2E S-PSNR-NN |  |  | VP #1 PSNR |  |  | VP #2 PSNR |  |  | C WS-PSNR |  |  | C S-PSNR-NN |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | EncT | DecT |
| Class S1 | 0.12% | 5.76% | 4.57% | 0.16% | 5.79% | 4.51% | 2.19% | 3.48% | 2.12% | 2.93% | 3.67% | 2.13% | 0.93% | 8.61% | 6.79% | 1.01% | 8.64% | 6.76% | 101% | 364% |
| Class S2 | 0.42% | 2.20% | 1.42% | 0.41% | 2.21% | 1.23% | 0.90% | 1.69% | 1.65% | 2.05% | 0.51% | -0.38% | 1.87% | 4.51% | 3.29% | 1.85% | 4.64% | 3.01% | 100% | 262% |
| Overall | 0.24% | 4.34% | 3.31% | 0.26% | 4.36% | 3.20% | 1.67% | 2.77% | 1.94% | 2.57% | 2.41% | 1.13% | 1.31% | 6.97% | 5.39% | 1.34% | 7.04% | 5.26% | 100% | 319% |

The complexity measurements are not exact, all simulations were carried out on the RWTH compute cluster.

# Evil viewports

| Landing, original, Frame 233, QP37 | Landing, CE13.4.3, Frame 233, QP37 |
| --- | --- |
| Landing, CE13.3.2, Frame 233, QP37 | Landing, CE13.7.7, Frame 233, QP37 |
| Balboa, original, Frame 454, QP37 | Balboa, CE13.4.3, Frame 454, QP37 |
| Balboa, CE13.3.2, Frame 454, QP37 | Balboa, CE13.7.7, Frame 454, QP37 |

# Acknowledgement

Simulations were performed with computing resources granted by RWTH Aachen University under project rwth0333.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.