| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau SAR, CN, 3–12 Oct. 2018 | Document: JVET-L0220 |
| --- | --- |

| Title: | CE3 6.3.2: Intra mode coding with 6 MPM and remapping strategy for non-MPM signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen | Tel: Email: | biao.wang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a simple strategy to build a 6-MPM list using only the left and above neighboring blocks. Furthermore, an intra mode remapping strategy is proposed to further improve the coding gain. It remaps three most probably used intra modes in the non-MPM set to the smallest intra mode in non-MPM set. The smallest modes only needs 5 bits in truncated binary coding (while the remaining intra modes needs 6 bits), thus improved coding performance can be expected. Two variations of the proposal contribution are provided, one with the constraint on the line buffer for CUs along the CTU boundary and the other without. Experimental results of the proposed modification are as follows with almost no change in encoding and decoding running time:

Over VTM Anchor with constraint (AI, RA): − 0.34%, − 0.13% Luma BD rate saving.

Over VTM Anchor without constraint (AI, RA): − 0.38%, − 0.15% Luma BD rate saving.

# Introduction

This proposal tries to build a 6-MPM list with a simplified strategy. It only use left and above neighboring blocks, A and L as shown in Fig.1, to construct the MPM list. For the non-MPM intra modes, truncated binary coding with a remapping strategy is used. It exploited the fact that smallest modes in the non-MPM modes would use only 5 bits, while the most probably intra modes in the non-MPM set can use 6 bits. Therefore, a higher coding performance can be achieved with this remapping strategy.

# Background

VTM 2 uses a total of 67 intra prediction modes. The intra mode is signaled by dividing the available intra modes into two different lists: MPM modes list, and non-MPM modes list. The MPM modes list consists of 3 intra modes and the non-MPM modes list is comprised of the remaining 64 intra modes. The MPM mode list only considers two spatial neighbors A and L as shown in Fig. 1.

L

A

Fig. 1 Neighbour intra modes

The order of insertion of intra modes into the MPM list is similar as that in HEVC. The signalling of non-MPM list uses fixed length coding as 64 is aligned with power of 2. The binarization for the syntax element intra prediction mode is specified in Table 1.

| Intra prediction modes | MPM flag | Bin string |
| --- | --- | --- |
| MPM modes (3) | 1 | 0 |
|  |  | 10 |
|  |  | 110 |
| Non-MPM modes (64) | 0 | 6 bits fixed length code |

Table 1. Intra prediction mode coding in VTM2.0.1

| Intra prediction modes | MPM flag | Bin string |
| --- | --- | --- |
| MPM modes (6) | 1 | 0 |
|  |  | 10 |
|  |  | 110 |
|  |  | 1110 |
|  |  | 11110 |
|  |  | 11111 |
| Non-MPM modes (61) | 0 | Truncated binary code |

Table 2. Intra prediction mode coding in this proposal

# Proposed Method

In this proposal, the coding of MPM mode index and non-MPM modes are shown in Table 2. The MPM list is expanded to 6 intra modes and they are constructed based on the L and A neighboring blocks as shown in Fig.2.

L

A

Fig. 2 Neighbour intra modes

Correspondingly, non-MPM list shrinks to 61 modes. If the current block’s intra prediction mode falls into the MPM list, only an index of MPM list is needed and it is truncated unary coded. Otherwise, this intra-prediction mode is directly truncated binary coded. The order of insertion of intra modes into the MPM list is as follows:

- Insert “Left (L)”, “Above (A)”, “Planar”, “DC”.

- If there is no angular mode inserted, further insert Vertical, Horizonal, Vertical − 4, and Vertical + 4

- Otherwise if only one angular mode (labeled as Ang) has been inserted, further insert Ang − 1, and Ang+1. If Vertical mode is not equal to any of Ang − 1, Ang, and Ang + 1, then further insert Vertical, otherwise, insert Horizontal

- Otherwise (i.e., two angular modes have been inserted), use Left angular mode to derive Left − 1 and Left + 1.

  - If Left − 1 is equal to Above, further insert Left + 1 and Above − 1.

  - Otherwise if Left + 1 equals to Above, further insert Left − 1 and Above + 1

  - Otherwise (Above is not equals to either Left − 1 and Left + 1), further insert Left − 1 and Left + 1

The remapping strategy is applied when the intra mode of current block belongs to non-MPM set. Non-MPM modes use truncated binary code, where the smallest 3 modes in the non-MPM set would use only 5 bits, while the remaining 58 (61-3) intra modes would require 6 bits to represent. If three most probable most in the non-MPM list is derived and can be remapped to the smallest 3 modes, then 5 bits instead of 6 bits will be used.

The remapping procedure starts from constructing a coded word list, which is comprised of the smallest 3 modes in the non-MPM set. Afterwards, a 3-entry derived mode list is constructed, which represents the most probably intra mode the current block will use. It is derived based on the first two angular modes in the MPM list. Their neighboring angular modes are derived and inserted into the derived mode list.

At the encoder, if the current block’s intra mode is in the Non-MPM set, then it is firstly checked whether in the derived mode list and then code word list. If so, its corresponding intra mode in the other list is used by the encoder. Correspondingly, after an intra mode is derived at the decoder, it is firstly checked whether in the code word list and then the derived mode list. If so, its corresponding intra mode in the other list is used by the decoder.

In addition, line buffer is a concern for hardware implementation. A constraint is applied when the current CU is on the CTU boundary and its above block locates on another CTU. In such case, the above block is considered to be not available. This constrain can be switch on/off using a macro.

# Experimental Results

The table below shows the coding gain and processing time impact of the proposed solutions.

- VTM simulations

  - Anchor: VTM2.0.1

  - Test: VTM2.0.1 + Proposal

# Conclusion

The proposal reports the results for a simple 6-MPM construction method with non-MPM mode signaling using truncated binary coding. CABAC decoding is decouple from MPM list construction with by-pass coding. The proposal with line buffer constraint “ON” achieves a Luma coding gain of − 0.34% and − 0.13% on top of VTM 2.0.1 for AI and RA configurations, respectively, with no increase in encoding and decoding time.

# Reference

[1]“Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, A. Alshin et. al., doc. JVET-J0024, April 2018.

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu doc. JVET-K1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-K1023, September 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).