| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0230 |
| --- | --- |

| Title: | CE13: HEC with face row based geometry padding using projection with bilinear interpolation (Test 3.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 3.1.b. For inter prediction, geometry padding of reference pictures is used to pad each face row instead of using repetitive padding at the picture boundaries. Bilinear interpolation is applied for the luminance and chrominance components. Results based on end-to-end WS-PSNR show that HEC with intra prediction disabled across face discontinuities provides {Y, U, V} BD-rate savings of {-1.60%, -1.11%, -1.16%} and {-1.29%, -1.56%, -1.76%} over PHEC and HEC, respectively, for VTM configuration.

# Introduction

Face discontinuities in the frame packed picture may yield to seam artifacts in reconstructed viewports. One of the goals of CE13 is to study removing subjective artifacts at face boundaries using normative decoding tools. The tested coding tools include modifications to intra and inter prediction, as well as in-loop filtering and projection format adaptation. This contribution reports the coding performance of HEC with face row based geometry padding using projection with bilinear interpolation, as studied in CE13 test 3.1.b [1].

# Technical description

The inter prediction replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s). The geometry padding in Figure 1 demonstrates a pixel position qb outside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by the projection mapping functions. When the derived coordinate qa is fractional, bilinear interpolation is applied for the luminance and chrominance components.

Figure 1. Geometry padding process.

# Simulation results

The software is implemented based on BMS-2.0.1 (with fix BMS ticket #78 and #79) and 360Lib-7.0. The encoding was done under 360° video CTC [2]. The following table only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for HEC with face row based geometry padding using projection with bilinear interpolation over PHEC and HEC are reported in Table 1.

Table 1. BD-rate for HEC with face row based geometry padding using projection with bilinear interpolation over PHEC and HEC.

|  |  | PHEC |  |  | HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -2.83% | -1.79% | -2.20% | -2.86% | -2.76% | -3.30% |
|  | ChairLift | -2.79% | -2.37% | -2.35% | -2.67% | -2.60% | -2.62% |
|  | KiteFlite | -0.48% | 0.06% | 0.24% | -0.03% | -0.07% | -0.13% |
|  | Harbor | -0.01% | 1.43% | 1.71% | -0.04% | -0.08% | -0.16% |
|  | Trolley | -0.84% | 0.14% | -0.39% | -0.19% | -0.34% | -0.48% |
|  | GasLamp | -0.55% | 0.29% | 0.40% | -0.08% | -0.08% | -0.07% |
| Class S2 | Balboa | -2.30% | -2.89% | -2.71% | -2.18% | -2.93% | -3.16% |
|  | Broadway | -1.62% | -1.94% | -1.96% | -1.55% | -2.05% | -2.51% |
|  | Landing2 | -1.91% | -1.06% | -1.36% | -1.42% | -2.08% | -2.19% |
|  | BranCastle2 | -2.70% | -2.96% | -3.03% | -1.90% | -2.60% | -3.00% |
| Class S1 |  | -1.25% | -0.37% | -0.43% | -0.98% | -0.99% | -1.13% |
| Class S2 |  | -2.13% | -2.21% | -2.26% | -1.76% | -2.41% | -2.71% |
| Overall |  | -1.60% | -1.11% | -1.16% | -1.29% | -1.56% | -1.76% |
| Enc Time[%] |  | 102% |  |  | - |  |  |
| Dec Time[%] |  | 110% |  |  | 110% |  |  |

BD-rate savings over PHEC and HEC are around {-1.60%, -1.11%, -1.16%} and {-1.29%, -1.56%, -1.76%}, respectively. Bit rate savings are particularly noticeable for moving camera content, i.e., SkateboardInLot, ChairLift, Balboa, Broadway, Landing2, and BranCastle2. For these sequences, the gain is between -1.42% and -2.86% compared to HEC. For static camera content, i.e., KiteFlite, Trolley, and GasLamp, the gain is below -0.2% compared to HEC. The encoding complexity of HEC with face row based geometry padding using projection with bilinear interpolation is similar to that of PHEC. However, the decoding complexity is increased by 10% compared to PHEC and HEC due to geometry padding.

Results for HEC with face row based geometry padding using projection with bilinear interpolation (test 3.1.b) over HEC with face row based geometry padding using projection with Lanczos 3/2 interpolation (test 3.1.a) are reported in Table 2.

Table 2. BD-rate for HEC with face row based geometry padding using projection with bilinear interpolation (test 3.1.b) over HEC with face row based geometry padding using projection with Lanczos 3/2 interpolation (test 3.1.a).

|  |  | Test 3.1.b vs test 3.1.a |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.04% | -0.03% | -0.53% |
|  | ChairLift | 0.06% | 0.04% | 0.17% |
|  | KiteFlite | -0.01% | 0.00% | -0.07% |
|  | Harbor | -0.01% | 0.01% | -0.12% |
|  | Trolley | 0.01% | 0.00% | -0.11% |
|  | GasLamp | -0.01% | 0.02% | -0.01% |
| Class S2 | Balboa | 0.05% | -0.09% | -0.24% |
|  | Broadway | 0.04% | 0.02% | -0.05% |
|  | Landing2 | -0.01% | -0.07% | -0.13% |
|  | BranCastle2 | 0.02% | -0.01% | 0.06% |
| Class S1 |  | 0.00% | 0.01% | -0.11% |
| Class S2 |  | 0.02% | -0.04% | -0.09% |
| Overall |  | 0.01% | -0.01% | -0.10% |
| Enc Time[%] |  | - |  |  |
| Dec Time[%] |  | 91% |  |  |

Simulation results show that bilinear interpolation provides similar performance as Lanczos 3/2 interpolation while reducing decoding complexity by about 9%.

# Conclusion

This contribution reports simulation result of CE13 test 3.1.b, which evaluates HEC with face row based geometry padding using projection with bilinear interpolation. The results show BD-rate savings of {-1.60%, -1.11%, -1.16%} and {-1.29%, -1.56%, -1.76%} over PHEC and HEC, respectively, for VTM configuration.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).