| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0231 |
| --- | --- |

| Title: | CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 3.3. For inter prediction, horizontal geometry padding of reference pictures is used over conventional repetitive padding at the left and right boundaries for the padded equirectangular projection (PERP). Results based on end-to-end WS-PSNR show that PERP with horizontal geometry padding of reference pictures provides {Y, U, V} BD-rate savings of {-0.22%, -0.29%, -0.38%} over PERP for VTM configuration at a similar complexity as repetitive padding. More importantly, it is reported that horizontal geometry padding significantly reduces the seam artifact issue for the ChairLift sequence.

# Introduction

The PERP format was shown to effectively reduce the seam artifacts in reconstructed viewports that encompass the left and right boundaries of the ERP picture [1]. However, padding and blending may not be sufficient to completely resolve the seam issue, as a seam artifact is still visible for the ChairLift sequence, as reported in [2]. The seam artifact in ChairLift is particularly visible during video playback, as the video appears to flicker where face seams occur. One of the goals of CE13 is to study removing subjective artifacts at face boundaries using normative decoding tools. The tested coding tools include modifications to intra and inter prediction, as well as in-loop filtering and projection format adaptation. This contribution reports the coding performance of PERP with horizontal geometry padding of reference pictures, as studied in CE13 test 3.3 [3].

# Technical description

The horizontal geometry padding for PERP is the same as for ERP, with the exception that the offset should be based on the unpadded ERP width instead of the picture width to account for the size of the padded regions. Figure 1 depicts the horizontal geometry padding for PERP. If a reference block is outside the left (right) reference picture boundary, it is replaced with the “wrapped-around” reference block shifted to the right (left) by the ERP width. The traditional repetitive padding is still employed in the vertical direction. The blending of the left and right padded regions is kept out of loop, as a post-processing operation.

Figure 1. Horizontal geometry padding for PERP.

# Simulation results

The software is implemented based on BMS-2.0.1 (with fix BMS ticket #78 and #79) and 360Lib-7.0. The encoding was done under 360° video CTC [4]. The following table only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for horizontal geometry padding over conventional repetitive padding are reported in Table 1 for PERP.

Table 1. BD-rate for horizontal geometry padding over conventional repetitive padding for PERP.

|  |  | PERP |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.53% | -1.11% | -1.18% |
|  | ChairLift | -0.73% | -0.95% | -0.85% |
|  | KiteFlite | -0.02% | 0.03% | 0.03% |
|  | Harbor | -0.03% | -0.05% | -0.10% |
|  | Trolley | -0.12% | -0.13% | -0.18% |
|  | GasLamp | -0.01% | 0.02% | 0.01% |
| Class S2 | Balboa | -0.04% | 0.17% | 0.10% |
|  | Broadway | -0.04% | 0.05% | 0.02% |
|  | Landing2 | -0.14% | -0.26% | -0.81% |
|  | BranCastle2 | -0.52% | -0.70% | -0.81% |
| Class S1 |  | -0.24% | -0.37% | -0.38% |
| Class S2 |  | -0.18% | -0.19% | -0.37% |
| Overall |  | -0.22% | -0.29% | -0.38% |
| Enc Time[%] |  | 101% |  |  |
| Dec Time[%] |  | 98% |  |  |

BD-rate savings for horizontal geometry padding over conventional repetitive padding for PERP are around {-0.22%, -0.29%, -0.38%}. The maximum BD-rate savings is observed on sequence ChairLift and reaches {-0.73%, -0.95%, -0.85%}. The complexity of horizontal geometry padding is similar to that of repetitive padding.

Besides providing over 0.7% BD-rate saving for ChairLift, the most important highlight of horizontal geometry padding is that it significantly improves the subjective quality of the viewport video. With repetitive padding, a seam artifact is clearly visible when the viewport encompasses the left and right boundaries of the 360° video. Though less visible on still images, the seam artifact is particularly objectionable during video playback, as the video appears to flicker where face seams occur. The horizontal geometry padding method significantly reduces the seam artifact issue for the ChairLift sequence, and the seam artifact becomes barely noticeable to expert viewers.

# Conclusion

This contribution reports simulation result of CE13 test 3.3, which evaluates PERP with horizontal geometry padding of reference pictures. The results show BD-rate savings of {-0.22%, -0.29%, -0.38%} over PERP for VTM configuration. It is reported that horizontal geometry padding can significantly reduce the seam artifact for cases where PERP alone is not sufficient.

# References

- J. Boyce, Z. Deng, “EE4: Padded ERP (PERP) projection format”, JVET-G0098, July 2017, Torino, IT.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Geometry padding for PERP”, JVET-J0044, Apr. 2018, San Diego, USA.

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).