| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0239 |
| --- | --- |

| Title: | CE3-related: Enabling different chroma sample location types in CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In VTM-2.0, the cross-component linear model (CCLM) tool assumes chroma sample location type 0 when downsampling the reconstructed chroma samples. However, other chroma sample location types should be supported. For example, in ITU-R BT.2020, it is a requirement that the chroma sample location type 2 is used. This contribution investigates the impact of the CCLM default downsampling filter on coding efficiency when encoding an input video with a chroma sample location type that is different from type 0. Simulation results for 360° content generated using chroma sample location types 1, 2, and 3 show an average coding loss in the range of 1% to 2% on the chroma components when compared to when the 360° content is generated using chroma sample location type 0. To compensate for this coding loss, this contribution proposes to use different downsampling filters in CCLM mode based on the chroma sample location type of the input video. In this way, the coding loss can be completely avoided.

# Problem statement

## Chroma sample location type

Three different chroma formats are commonly used when encoding video sequences: 4:4:4, 4:2:2, and 4:2:0. In the 4:2:0 chroma format, the chroma planes are down-sampled by a factor 2 in both horizontal and vertical directions compared to the luma plane. The chroma sample location type defines the position of chroma samples with respect to luma samples. Figure 1 depicts the location of chroma samples in 4:2:0 chroma format for chroma sample types 0-3. In HEVC, the chroma sample location type may be signaled in the video usability information (VUI) parameters using the syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field. At the decoder side, the video player may exploit these syntax elements when converting the decoded picture format to the display format.

Figure 1. Location of chroma samples in 4:2:0 chroma format.

## Cross-component linear model prediction in VTM-2.0

To reduce the cross-component redundancy, a cross-component linear model (CCLM) prediction mode is used in the VTM-2.0 [3] for intra prediction. In the CCLM mode, the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

predCi, j=α·recL'i, j+ βpredCi,j=α·recL'i,j+β

where predCi, jpredCi,j represents the predicted chroma samples in a CU and recLi, jrecLi,j represents the downsampled reconstructed luma samples of the same CU. Parameters αα and ββ are derived by minimizing the regression error between the neighbouring reconstructed luma and chroma samples around the current block. Figure 2 shows the location of the left and above samples and the sample of the current block involved in the CCLM mode.

Figure 2. Locations of the samples used for the derivation of α and β.

To perform the cross-component prediction, the reconstructed luma samples need to be downsampled to match the sampling of the chroma signal. The downsampling filter used in CCLM mode is as follows:

| recL'i,j= recL2i-1,2j+2∙recL2i,2j+recL2i+1,2j+recL2i-1,2j+1+2∙recL2i,2j+1+recL2i+1,2j+1+4≫3 | (1) |
| --- | --- |

As shown in Figure 2 and Eq. (1), the CCLM prediction in VTM-2.0 assumes chroma sample location type 0, since it is the most commonly used in legacy content in ITU-R BT.601 or ITU-R BT.709 container. However, in ITU-R BT.2020, it is a requirement that the chroma sample location type 2 is used [1][2]. Thus, CCLM should be modified to support different chroma sample location types. This could be achieved by using different downsampling filters based on the chroma sample location type of the coded picture.

# Enabling different chroma sample location types in CCLM

To enable the CCLM tool to switch between different downsampling filters based on the chroma sample location type of the coded picture, the chroma sample location type should be signaled in the sequence parameter set (SPS) instead of the VUI parameters. Table 1 shows one example of how the chroma sample location type may be signaled in the SPS.

Table 1. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | ue(v) |
| … |  |
| lm_chroma_enabled_flag | u(1) |
| if( lm_chroma_enabled_flag && chroma_format_idc = = 1 ) { |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) |  |
| chroma_sample_loc_type | ue(v) |
| } |  |
| … |  |
| } |  |

chroma_loc_info_present_flag equal to 1 specifies that chroma_sample_loc_type is present. chroma_loc_info_present_flag equal to 0 specifies that chroma_sample_loc_type is not present.

When chroma_format_idc is not equal to 1, chroma_loc_info_present_flag should be equal to 0.

chroma_sample_loc_type specifies the location of chroma samples as follows:

- If chroma_format_idc is equal to 1 (4:2:0 chroma format), chroma_sample_loc_type specifies the location of chroma samples as shown in Figure 1.

- Otherwise (chroma_format_idc is not equal to 1), the values of the syntax element chroma_sample_loc_type shall be ignored.

The value of chroma_sample_loc_type shall be in the range of 0 to 3, inclusive. When the chroma_sample_loc_type is not present, the value of chroma_sample_loc_type is inferred to be equal to 0.

The CCLM tool would use a specific downsampling filter based on the value of chroma_sample_loc_type. For example, the downsampling filters specified in Eq. (2), (3), and (4) may be used for chroma sample location types 1, 2, and 3, respectively.

| recL'i,j= recL2i-1,2j+3∙recL2i,2j+3∙recL2i+1,2j+recL2i+2,2j+recL2i-1,2j+1+3∙recL2i,2j+1+3∙recL2i+1,2j+1+recL2i+2,2j+1+8≫4 | (2) |
| --- | --- |
| recL'i,j= recL2i-1,2j-1+2∙recL2i,2j-1+recL2i+1,2j-1+2∙recL2i-1,2j+4∙recL2i,2j+2∙recL2i+1,2j+recL2i-1,2j+1+2∙recL2i,2j+1+recL2i+1,2j+1+8≫4 | (3) |
| recL'i,j= recL2i,2j-1+recL2i+1,2j-1+2∙recL2i,2j+2∙recL2i+1,2j+recL2i,2j+1+recL2i+1,2j+1+4≫3 | (4) |

# Simulation results

The software is implemented based on BMS-2.0.1 and modified version of 360Lib-7.0 [4]. The encoding was done in the hybrid equi-angular cubemap (HEC) format (without padding) under 360° video CTC [5]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for chroma sample location type 1, 2, and 3 encoding are reported in Table 2, Table 3, and Table 4, respectively. The anchor uses chroma sample location type 0 encoding in VTM-2.0. The chroma sample location type is used by all projection format conversions and metrics calculations. The 360Lib software fixes proposed in [4] are included in the projection format conversion and metric calculation in this contribution.

Table 2. BD-rate for chroma sample location type 1 encoding compared to chroma sample location type 0 encoding.

|  |  | Default downsampling filter |  |  | Modified downsampling filter |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.04% | 0.37% | 0.26% | 0.05% | -0.07% | 0.22% |
|  | ChairLift | 0.05% | 2.94% | 0.60% | -0.04% | -0.35% | -0.17% |
|  | KiteFlite | 0.00% | 0.70% | 0.65% | -0.01% | -0.31% | 0.01% |
|  | Harbor | 0.05% | 0.20% | 0.44% | 0.02% | -0.22% | -0.26% |
|  | Trolley | 0.01% | 0.71% | 0.43% | 0.00% | -0.06% | 0.03% |
|  | GasLamp | 0.03% | 1.45% | 1.71% | -0.01% | -0.05% | -0.27% |
| Class S2 | Balboa | -0.03% | 0.85% | 1.24% | 0.00% | 0.20% | 0.16% |
|  | Broadway | 0.04% | 0.47% | 1.58% | 0.01% | -0.36% | 0.49% |
|  | Landing2 | 0.02% | 1.21% | 0.44% | 0.02% | 0.05% | 0.01% |
|  | BranCastle2 | 0.02% | 3.38% | 2.25% | 0.01% | -0.08% | -0.20% |
| Class S1 |  | 0.03% | 1.06% | 0.68% | 0.00% | -0.18% | -0.07% |
| Class S2 |  | 0.01% | 1.48% | 1.37% | 0.01% | -0.05% | 0.12% |
| Overall |  | 0.02% | 1.23% | 0.96% | 0.00% | -0.12% | 0.00% |

Table 3. BD-rate for chroma sample location type 2 encoding compared to chroma sample location type 0 encoding.

|  |  | Default downsampling filter |  |  | Modified downsampling filter |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.02% | 1.10% | 0.48% | 0.07% | -0.52% | -0.33% |
|  | ChairLift | 0.02% | 2.94% | -0.08% | -0.04% | -1.23% | -0.39% |
|  | KiteFlite | 0.00% | 0.36% | 1.03% | -0.01% | -0.52% | -0.31% |
|  | Harbor | 0.07% | 0.67% | 1.11% | 0.05% | -0.37% | -0.37% |
|  | Trolley | 0.01% | 0.76% | 0.34% | -0.02% | -0.17% | 0.00% |
|  | GasLamp | 0.02% | 0.98% | 1.48% | -0.01% | -0.60% | 0.12% |
| Class S2 | Balboa | -0.01% | 0.15% | 0.17% | -0.02% | -0.10% | -0.07% |
|  | Broadway | 0.02% | 0.64% | 1.05% | 0.00% | -0.23% | -0.06% |
|  | Landing2 | -0.01% | 0.24% | -0.43% | 0.01% | -0.52% | -0.34% |
|  | BranCastle2 | 0.01% | 2.68% | 1.57% | 0.00% | -0.87% | -0.87% |
| Class S1 |  | 0.02% | 1.14% | 0.72% | 0.01% | -0.57% | -0.21% |
| Class S2 |  | 0.00% | 0.93% | 0.59% | 0.00% | -0.43% | -0.33% |
| Overall |  | 0.01% | 1.05% | 0.67% | 0.00% | -0.52% | -0.26% |

Table 4. BD-rate for chroma sample location type 3 encoding compared to chroma sample location type 0 encoding.

|  |  | Default downsampling filter |  |  | Modified downsampling filter |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.12% | 0.97% | 0.56% | -0.05% | 0.30% | 0.08% |
|  | ChairLift | 0.06% | 5.78% | 0.74% | 0.00% | -0.32% | -0.09% |
|  | KiteFlite | 0.02% | 1.27% | 1.51% | 0.01% | -0.09% | 0.12% |
|  | Harbor | 0.02% | 1.08% | 1.73% | 0.03% | 0.06% | -0.33% |
|  | Trolley | 0.00% | 1.27% | 0.67% | 0.00% | 0.24% | 0.06% |
|  | GasLamp | 0.06% | 2.04% | 2.66% | 0.03% | -0.25% | 0.12% |
| Class S2 | Balboa | -0.02% | 0.79% | 1.58% | -0.02% | 0.30% | 0.20% |
|  | Broadway | 0.00% | 0.84% | 1.96% | 0.01% | -0.18% | 0.58% |
|  | Landing2 | 0.01% | 1.23% | 0.18% | 0.03% | 0.08% | -0.23% |
|  | BranCastle2 | 0.02% | 5.63% | 3.61% | 0.00% | -0.28% | -0.20% |
| Class S1 |  | 0.00% | 2.07% | 1.31% | 0.00% | -0.01% | 0.00% |
| Class S2 |  | 0.00% | 2.12% | 1.83% | 0.01% | -0.02% | 0.09% |
| Overall |  | 0.00% | 2.09% | 1.52% | 0.00% | -0.01% | 0.03% |

Simulation results show an average coding loss in the range of 1% to 2% on the chroma components when the CCLM default downsampling filter, which assumes chroma sample location type 0, is used for encoding an input video with a chroma sample location type that is different from type 0. The coding loss can even exceed 5.5% on some sequences. However, when the downsampling filter used in CCLM mode is modified according to the chroma sample location type of the input video, the coding loss can be completely avoided.

# Conclusion

This contribution proposes to use different downsampling filters in CCLM mode based on the chroma sample location type of the input video. It is reported that, if the input video has a chroma sample location type different from the one assumed in CCLM mode, an average coding loss in the range of 1% to 2% is observed on the chroma components. However, when the downsampling filter used in CCLM mode is switched based on the chroma sample location type of the input video, the coding loss can be completely avoided. It is therefore suggested to modify the next release of VTM to support other chroma sample location types (and appropriate downsampling filters) in CCLM mode.

# References

- ITU-T H.Sup15, “Conversion and coding practices for HDR/WCG Y'CbCr 4:2:0 video with PQ transfer characteristics”, January 2017.

- White Paper Blu-ray Disc™ Read-Only Format (Ultra HD Blu-ray™), “Audio Visual Application Format Specifications for BD-ROM Version 3.1”, August 2016, http://www.blu-raydisc.com/Assets/Downloadablefile/BD-ROM_Part3_V3.1_WhitePaper_160729_clean.pdf

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)”, JVET-K1002, July 2018, Ljubljana, SI.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Chroma sample location type support for 360Lib”, JVET-L0238, October 2018, Macao, CN.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).