| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0240 |
| --- | --- |

| Title: | CE2: Subsampled gradient calculation for highest temporal layer (Test 2.7.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rahul Vanam, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4800 rahul.vanam@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Technologies related to the Adaptive Loop Filter (ALF) are currently being studied in the Core Experiment 2 (CE2).  This proposal reports the test results of the ALF simplification in CE2 Test 2.7.1.  In this test, derivative calculation for the highest temporal layer pictures of RA configuration is simplified by computing derivatives only at the top-left sample position of each 2×2 luma block. It is reported that applying this simplification to the VTM-2.0.1 anchor yields an average decoding speedup of 2% with a small loss of 0.02% for the RA configuration, and no impact for other configurations.

# Introduction

The adaptive loop filter (ALF) was adopted into the VTM software during the 11th JVET meeting.  The ALF performs block classification of each 4×4 reconstructed luma block at both the encoder and decoder.  For each pixel in the 4×4 luma block, gradients are computed along the horizontal, vertical, diagonal, and anti-diagonal directions.  Core Experiment 2 [1] was established in the 11th JVET meeting for ALF related testing, and a scheme to simplify the gradient calculation in ALF was proposed in CE2 Test 2.4.1.2 [2].

# Proposed simplification

Let a 4×4 luma reconstructed block be composed of four 2×2 blocks as shown in Figure 1.  The proposed simplification involves calculating the derivative only at the top-left sample position (the gray circles in Figure 1) of each 2×2 block as follows

| - G2×2dir=gdiri0,j0 | (1) |
| --- | --- |

where dir is the direction such as horizontal, vertical, diagonal and anti-diagonal, and gdiri0,j0 is the derivative at the top-left sample position of each 2×2 luma block.

In the SIMD implementation, gradients are calculated for all samples belonging to alternating rows, for example, row0, row2, row4, etc. in Figure 1.  Among these samples, gradients belonging to only the top-left samples (gray circles) are used in block classification.

Figure 1. Block classification for a 4×4 block.  Gray circles represent pixels for which derivatives are calculated.

# Experimental results

VTM-2.0.1 is used as the software basis in our experiments as specified by [1], and we use test sequences, configurations, and test conditions specified by JVET-K1010 [3].  In our test, the proposed simplification is applied only to the highest temporal layer of VTM-2.0.1 anchor. Summary result for RA is provided in Table 1, while there is no impact on AI and LB configurations.  Detailed results for AI, RA, and LB configurations are provided in the accompanying Excel sheet.

Table 1. RA results of CE2.7.1 over VTM-2.0.1 anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.01% | 0.00% | 100% | 98% |
| Class A2 | 0.02% | 0.01% | 0.01% | 100% | 97% |
| Class B | 0.03% | -0.01% | -0.01% | 100% | 98% |
| Class C | 0.01% | 0.00% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.00% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Class F (optional) | 0.00% | -0.01% | 0.00% | 100% | 99% |

# Computational complexity analysis

The computational complexity of the proposed simplification is obtained using the procedure in JVET-K0521[4], and the results are summarized in Table 2 for both the anchor and test.  We provide the computational complexity of the highest temporal layer for our approach, since the complexity of our approach is the same as the anchor for lower temporal layers.  For gradient calculation of an 8×8 block, the anchor takes 1824 additions, 576 comparisons, and 144 shift operations, while our approach takes 528 (288+240) additions, 144 comparisons, and 36 shift operations.  For remaining computations associated with block classification, our approach has the same complexity as the anchor, which is 8 additions, 28 comparisons, 8 multiplications, and 20 shift operations.  Therefore, the total computations for our approach are 536 additions, 172 comparisons, 8 multiplications, and 56 shift operations, and computations per sample are 8.375 additions, 2.6875 comparisons, 0.125 multiplications, and 0.875 shift operations.  Similarly, for the anchor, the total computations are 1832 additions, 604 comparisons, 8 multiplications, and 164 shift operations, and computations per sample are 28.625 additions, 9.4375 comparisons, 0.125 multiplications, and 2.5625 shift operations.

Table 2. Computational complexity analysis.

| Test | Operations of classification process per luma sample | Operations of filtering process per luma sample | Operations of filtering process per chroma sample | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) | Memory for storing filters | Summary of operations per sample |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor (VTM-2.0.1) | Add: 28.625 Comp: 9.4375 Mul: 0.125 Shift: 2.5625 | Add: 25 Comp: 4 Mul: 13 Shift: 1 | Add: 13 Comp: 1 Mul: 7 Shift: 1 | 4×4, slice. slice | 6, 4 | 0 | Add: 66.625 Comp: 14.4375 Mul: 20.125 Shift: 4.5625 |
| CE2.7.1 | Highest temporal layer: Add: 8.375 Comp: 2.6875 Mul: 0.125 Shift: 0.875 | Same as anchor | Same as anchor | Same as anchor | Same as anchor | Same as anchor | Highest temporal layer: Add: 46.375 Comp: 7.6875 Mul: 20.125 Shift: 2.875 |

# Conclusions

This contribution describes the ALF simplification scheme in CE2 Test CE2.7.1.  The derivative calculation for pictures belonging to the highest temporal layer of RA configuration is simplified by computing derivatives only at the top-left sample position of each 2×2 luma block.  Applying this simplification to the VTM-2.0.1 anchor yields an average decoding speedup of 2% with a small loss of 0.02% for the RA configuration.

# References

- V. Seregin, C.-Y. Chen, “Description of Core Experiment 2 (CE2): Adaptive Loop Filter,” JVET-K1022, 11th meeting, July 2018, Ljubljana, SI.

- R. Vanam, Y. He, Y. Ye, “CE2: Adaptive Loop Filter Simplification (Test 2.4.1.2),” JVET-K0327, 11th meeting, July 2018, Ljubljana, SI.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th Meeting, July 2018, Ljubljana, SI.

- L. Zhang, “BoG Report on ALF,” JVET-K0521, 11th Meeting, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).