In 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_reshaper_enabled_flag | u(1) |
| if( sps_reshaper_enabled_flag ) { |  |
| sps_reshaper_signal_type | u(2) |
| sps_reshaper_chromaAdj_flag | u(1) |
| sps_reshaper_model_present_flag | u(1) |
| if( sps_reshaper_model_present_flag) |  |
| sps_reshaper_model( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

In 7.3.3.1 General Slice Header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( sps_reshaper_enabled_flag) { |  |
| slice_reshaper_model_present_flag | u(1) |
| if( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if( slice_reshaper_enable_flag) { |  |
| if( sps_reshaper_chromaAdj_flag ) |  |
| slice_reshaper_chromaAdj_flag | u(1) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

Add a new syntax table SPS reshaper model :

| sps_reshaper_model( ) { | Descriptor |
| --- | --- |
| full_range_input_flag | u(1) |
| dQP_model_scale_int_prec | ue(v) |
| if( dQP_model_scale_int_prec > 0 ) |  |
| dQP_model_scale_int | u(v) |
| dQP_model_scale_frac_prec_minus16 | ue(v) |
| dQP_model_scale_frac | u(v) |
| if( dQPModelScaleAbs ) |  |
| dQP_model_scale_sign_flag | u(1) |
| dQP_model_offset_int_prec_minus3 | ue(v) |
| dQP_model_offset_int | u(v) |
| dQP_model_offset_frac_prec_minus1 | ue(v) |
| dQP_model_offset_frac | u(v) |
| if( dQPModelOffsetAbs ) |  |
| dQP_model_offset_sign_flag | u(1) |
| dQP_model_abs_prec_minus3 | ue(v) |
| dQP_model_max_abs | u(v) |
| if( dQP_model_max_abs) |  |
| dQP_model_max_sign_flag | u(1) |
| dQP_model_min_abs | u(v) |
| if( dQP_model_min_abs) |  |
| dQP_model_min_sign_flag | u(1) |
| } |  |

Add a new syntax table slice reshaper model:

| slice_reshaper_model () { | Descriptor |
| --- | --- |
| reshape_model_profile_type | u(4) |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_num_cw_minus1 | ue(v) |
| for ( i = 0; i < reshape_model_num_cw_minus1 + 1; i++) { |  |
| reshape_model_delta_abs_CW [ i ] | u(5) |
| if( reshaper_model_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| for ( i = reshape_model_min_bin_idx; i < = reshape_model_max_bin_idx; i++ ) |  |
| reshaper_model_bin_profile_delta[ i ] | ue(v) |
| } |  |

In General sequence parameter set RBSP semantics, add the following semantics:

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type equal to 0 specifies SDR (gamma) transfer characteristics; sps_reshaper_signal_type equal to 1 specifies PQ transfer characteristics; sps_reshaper_signal_type equal to 2 specifies HLG transfer characteristics; and sps_reshaper_signal_type equal to 3 is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved value of sps_reshaper_signal_type.  When sps_reshaper_signal_type is not present, it is inferred to be equal to 0.

sps_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is used in CVS. sps_reshaper_chromaAdj equal to 0 specifies that chroma residue scaling is not used in CVS. When sps_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

sps_reshaper_model_present_flag equal to 1 specifies sps_reshaper_model() is present in SPS. sps_reshaper_model_present_flag equal to 0 specifies sps_reshaper_model() is not present in SPS. When sps_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

Add sps_reshaper_model( ) syntax

full_range_input_flag equal to 0 specifies a standard dynamic range input video signal. full_range_input_flag equal to 0 specifies a full range input video signal.  When full_range_input_flag is not present, it is inferred to be equal to 0.

dQP_model_scale_int_prec specifies the number of bits used for the representation of dQP_model_scale_int.

dQP_model_scale_int specifies the integer value of dQP model scale. When dQP_model_scale_int is not present, it is inferred to be equal to 0.

dQP_model_scale_frac_prec_minus16 plus 16 specifies the number of bits used for the representation of dQP_model_scale_frac.

dQP_model_scale_frac specifies the fractional value of the dQP model scale.

The variable dQPModelScaleAbs is derived as:

dQPModelScaleAbs = dQP_model_scale_int << ( dQP_model_scale_frac_prec_minus16 + 16 ) + dQP_model_scale_frac

dQP_model_scale_sign_flag specifies the sign of dQP model scale as follows:

–	If dQP_model_scale_sign_flag is equal to 0, the corresponding variable dQPModelScale has a positive value.

–	Otherwise (dQP_mode_scale_sign_flag is equal to 1), the corresponding variable dQPModelScale has a negative value.

When dQP_model_scale_sign is not present, it is inferred to be equal to 0.

The variable dQPModelScale = dQPModelScaleAbs * ( 1 – 2 * dQP_model_scale_sign_flag );

dQP_model_offset_int_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_offset_int.

dQP_model_offset_int specifies the integer value of dQP model offset.

dQP_model_ offset _frac_prec_minus1 plus 1 specifies the number of bits used for the representation of dQP_model_ offset_frac.

dQP_model_ offset_frac specifies the fractional value of the dQP model offset.

The variable dQPModelOffsetAbs is derived as:

dQPModelOffsetAbs = dQP_model_offset_int << (dQP_model_offset_frac_prec_minus1 + 1) + dQP_model_offset_frac

dQP_model_offset_sign_flag specifies the sign of dQP model offset as follows:

–	If dQP_model_offset_sign_flag is equal to 0, the corresponding dQPModelOffset has a positive value.

–	Otherwise (dQP_model_offset_sign_flag is equal to 1), the corresponding dQPModelOffset has a negative value.

When dQP_model_offset_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelOffset = dQPModelOffsetAbs * ( 1 – 2 * dQP_model_offset_sign_flag);

dQP_model_abs_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_max_abs and dQP_model_min_abs.

dQP_model_max_abs specifies the absolute value of the maximum dQP value.

dQP_model_max_sign_flag specifies the sign of the maximum dQP value as follows:

–	If dQP_model_max_sign_flag is equal to 0, the corresponding dQPModelMax has a positive value.

–	Otherwise (dQP_model_max_sign_flag is equal to 1), the corresponding dQPModelMax has a negative value.

When dQP_model_max_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMax = dQP_model_max_abs * ( 1 – 2 * dPQ_model_max_sign_flag ).

dQP_model_min_abs specifies the specifies the absolute value of the minimal dQP value.

dQP_model_min_sign_flag specifies the sign of the minimal dQP value as follows:

–	If dQP_model_min_sign_flag is equal to 0, the corresponding dQPModelMin has a positive value.

–	Otherwise ( dQP_model_min_sign_flag is equal to 1 ), the corresponding dQPModelMin has a negative value.

When dQP_model_min_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMin = dQP_model_min_abs * ( 1 – 2 * dQP_model_min_sign_flag ).

In slice header syntax, add the following semantics

slice_reshaper_model_present_flag equal to 1 specifies slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 specifies slice_reshaper_model() is not present in slice header. When slice_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

slice_reshaper_enabled_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enabled_flag equal to 0 specifies that reshaper is not enabled for the current slice. When slice_reshaper_enable_flag is not present, it is inferred to be equal to 0.

slice_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is enabled for the current slice. slice_reshaper_chromaAdj_flag equal to 0 specifies that chroma residue scaling is not enabled for the current slice. When slice_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

Add slice_reshaper_model( ) syntax

reshape_model_profile_type specifies the profile type to be used in the reshaper construction process.

reshape_model_min_bin_idx specifies the minimum bin index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to 31, inclusive.

reshape_model_delta_max_bin_idx is set equal to the maximum allowed bin index MAX_BIN_IDX minus the maximum bin index to be used in the reshaper construction process. MAX_BIN_IDX is set to equal to 31. reshape_model_max_bin_idx then equal to MAX_BIN_IDX – reshape_model_delta_max_bin_idx.

reshape_model_num_cw_minus1 plus 1 specifies the number of codewords to be signalled.

reshape_model_delta_abs_CW[ i ] specifies the i-th absolute delta codeword value.

reshape_model_delta_sign_CW_flag[ i ] specifies the sign of the i-th delta codeword as follows:

–	 If reshape_model_delta_sign_CW_flag[ i ] is equal to 0, the corresponding variable RspModelDeltaCW[ i ] has a positive value.

–	Otherwise ( reshape_model_delta_sign_CW_flag[ i ] is not equal to 0 ), the corresponding variable RspModelDeltaCW[ i ] has a negative value.

When reshape_model_delta_sign_CW_flag[ i ] is not present, it is inferred to be equal to 0.

The variable RspModelDeltaCW[ i ] = (1 - 2* reshape_model_delta_sign_CW [ i ]) * reshape_model_delta_abs_CW [ i ];

The variable OrgCW is set equal to 1 << ( BitDepthY  – 5 );

The variable RspModelCW[ i ] = OrgCW + RspModelDeltaCW[ i ].

reshape_model_bin_profile_delta[ i ]  specifies the delta value to be used to adjust the profile of the i-th bin in the reshaper construction process.

The variable RspCW[ i ] for ith bin is derived as following:

–	If reshape_model_bin_profile_delta[ i ] is equal to 0, RspCW[ i ] = OrgCW + 1.

–	Otherwise (reshape_model_bin_profile_delta[ i ] is not equal to 0), RspCW[ i ] = RspModelCW[ j – 1 ] when  reshape_model_bin_profile_delta[ i ] is equal to j.

Add the following in Picture reconstruction process

### 8.4.5	Picture reconstruction process

Inputs to this process are:

–	a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

–	the variables nCurrSw and nCurrSh specifying the width and height, respectively, of the current block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nCurrSw)x(nCurrSh) array predSamples specifying the predicted samples of the current block,

–	an (nCurrSw)x(nCurrSh) array resSamples specifying the residual samples of the current block.

Depending on the value of the colour component cIdx, the following assignments are made:

–	If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y.

–	Otherwise, if cIdx is equal to 1, recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C.

–	Otherwise (cIdx is equal to 2), recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C.

When the value of slice_reshaper_enabled_flag is equal to 1, the (nCurrSw)x(nCurrSh) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as the mapping process specified in clause 8.4.5.1. Otherwise, the (nCurrSw)x(nCurrSh) block of the reconstructed sample array recSamples at location ( xCurr, yCurr ) is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.4.5.1	Picture reconstruction with mapping process

This clause specifies picture reconstruction with mapping process. The two luma mapping LUT : FwdLUT and InvLUT, and chroma mapping LUT ChromaAdjLUT generation is specified in clause 8.4.5.1.1. The picture reconstructon with mapping process for luma sample value is specified in 8.4.5.1.2. The picture reconstructon with mapping process for chroma sample value is specified in 8.4.5.1.3.

        - Luma and chroma mapping LUT generation

The output of this process is the array FwdLUT[ ], InvLUT[ ], and chromaAdjLUT[ ].

Let the variable lutSize = ( 1 << BitDepthY ). The variable binNum = 32 and binLen = lutSize / binNum.

The variable fullRangeFlag = ( full_range_input_flag || (reshape_model_min_bin_idx = = 0 & & reshape_model_max_bin_idx = = 31 )

The variable rangeLow and rangeHigh is derived as follows:

–	If fullRangeFlag is equal to 1,

rangeLow = 0
rangeHigh = ( ( 1 << BitDepthY ) – 1 )

–	Otherwise (fullRangeFlag is equal to 0),

rangeLow = ( ( 1 << ( BitDepthY – 8 ) ) * 16 )
rangeHigh = ( ( 1 << ( BitDepthY – 8 ) ) * 235 )

When sps_reshaper_model_present_flag is equal to 1, FwdLUT[ ] and chromaAdjLUT[ ] is generated as follows:

Define function calcEXP2 as following:

function exp2Val = calcEXP2( int32_t val ) {
int32_t i, f, r, s, exp2Val;
r = 0x00000e20;
i = (val + 0x8000) & (~0xffff);
f = val - i;
s = ( ( 15 < < 16 ) – i ) > > 16;
r = ( r * f + 0x3e1cc333 ) > > 17;
r = ( r * f + 0x58bd46a6 ) > > 16;
r = r * f + 0x7ffde4a3;
exp2Val = (uint32_t)r >> s;

The variables p1, p2, totShift, maxDQPFP, minDQPFP, slopeLUT[ ], hpLUT[ ], tQPFP, addVal, valQP, dQPDIV6_FP, maxY are derived as follows:

p1 = dQP_model_scale_frac_prec_minus16 + 16
p2 = dQP_model_offset_frac_prec_minus1 + 1
totShift = p1 + p2
maxDQPFP = dQPModelMax * ( 1 < <totShift )
minDQPFP = dQPModelMin * ( 1 < <totShift )
slopeLUT[ lutSize ] = { 0 }
hpLUT[ lutSize ] = { 0 }
for( i = 1; i < lutSize ; i++ ) {
	tQPFP = (dQPModelScale * i) * ( 1 < < p2 ) + offsetFP * ( 1 < < p1 )
	tQPFP = Clip3( minDQPFP, maxDQPFP, tQPFP )

	if( sps_reshaper_chromaAdj_flag) {
		addVal = (Abs(tQPFP) = = ( ( Abs( tQPFP ) > > total_shift ) < < total_shift ) )
			 ? 0 : ( Sign( tQPFP ) < 0 ) ? – 1 : ( i < 567 ) ? 0 : 1
		valQP = Clip3(dQPModelMin, dQPModelMax, Sign( tQPFP ) * ( Abs( tQPFP ) > > totShift ) + addVal )
		dQPDIV6_FP = Sign(valQP) * ((((Abs(valQP) < < 16) + 3 ) / 6 )
		ChromaAdjLUT[i] = calcEXP2(dQPDIV6_FP)
	}
	dQPDIV6_FP = ( Abs(tQPFP) + 3 ) / 6;
	dQPDIV6_FP = Sign(tQPFP) * ((dQPDIV6_FP + ( 1 < < ( totShift – 17 ) ) ) > > ( totShift – 16 ) )
	slopeLUT[i] = calcEXP2(dQPDIV6_FP)
}

if (full_range_input_flag = = 0) {
	for( i = 0; i <rangeLow; i++ )
		slopeLUT[ i ] = 0;
	for( i = rangeHigh; i <lutSize; i++ )
		slopeLUT[ i ] = 0;
	}
for( i = 0; i < lutSize - 1; i++) {
	hpLUT[ i + 1 ] = hpLUT[ i ] + slopeLUT[ i ]
}
maxY = (hpLUT[ lutSize – 1 ] + ( 1 < < 7 ) ) > > 8
for( i = 0; i < lutSize; i++ ) 
	FwdLUT[ i ] = ( ( ( hpLUT[ i ] > > 8) * ( lutSize – 1 ) + ( maxY > > 1 ) ) / maxY);

When slice_reshaper_model_present_flag is equal to 1, FwdLUT[ ] and chromaAdjLUT[ ] is generated as follows:

The variables fpPREC, sfPREC, sFactor, tCW, binsLUT[ ], iStart, iEnd,scaleFP  are derived as follows:

fpPREC = 14
sfPREC = 4
sFactor = ( 1 < < sfPREC )

for( i = 0; i < binNum; i++) {
	tCW = ( RspCW[ i ] * (1 << fpPREC ) + ( 1 < < (Log2(binLen) – 1  ) ) > > Log2( binLen )
	for( j = 0; j < binLen; j++)
		binsLUT[ i * binLen + j] = tCW;
}
for( i = 1; i < lutSize; i++ )
	binsLUT[ i ] = binsLUT[ i - 1 ] +binsLUT [ i ]
for( i = 0; i < lutSize; i++) {
	tCW = Clip3(0, lutSize - 1, (binsLUT[i] * sFactor + ( 1 < < ( fpPREC + sfPREC – 1 ) ) ) > > ( fpPREC + sfPREC ) )
	FwdLUT[i] = tCW
}

//chromaAdjLUT generation
The array scaleLUT[ ] is defined as followings:

scaleLUT[ 2 * binNum ] = { 8192, 8192, 8192, 8192, 8192, 8192, 8192, 16384, 16384, 16384, 16384, 24576, 24576, 24576, 24576, 32768, 32768, 32768, 32768, 40960, 40960, 40960, 40960, 49152, 49152, 49152, 49152, 57344, 57344, 57344, 65536, 65536, 65536, 73728, 73728, 73728, 81920, 81920, 81920, 81920, 90112, 90112, 90112, 90112, 98304, 98304, 98304, 98304, 106496, 106496, 106496, 106496, 114688, 114688, 114688, 114688, 122880, 122880, 122880, 122880, 131072, 131072, 131072, 131072 };  // corresponding to codeword 1:64 for 10bit case

for( i = 0; i <binNum; i++ ){
	iStart = i * binLen
	iEnd = ( i + 1 ) * binLen - 1
	binsLUT[ i ] = Clip3( 1, 64, FwdLUT[ iEnd ] – FwdLUT[iStart] )
}

for( i = 0; i <binNum; i++ ){ 
	if ((i < reshape_model_min_bin_idx) || (i > reshape_model_max_bin_idx))
		scaleFP = ( 1 < < 16 )
	else {
		if( reshape_model_bin_profile_delta[i] = = 0 )
 			scaleFP = ( 1 < < 16 );

else
			scaleFP = scaleLUT[ binsLUT[ i ] – 1 ]
	}
	for( j = 0; j < binLen; j++) {
		ii = i * binLen + j
		ChromaAdjLUT[ii] = scaleFP
	}
}

The InvLUT[ ] is generated as follows:

The variables low, high, first, last are derived as follows:

low = FwdLUT[ 0 ]
high = FwdLUT[ lutSize - 1 ]
for( i = 1; i < lutSize ; i++ )
	if( FwdLUT[ 0 ] < FwDLUT[ i ] ) {
		first = i - 1;
		break;
	}
for( i = lutSize - 2; i >= 0; i-- )
	if( FwdLUT[ lutSize - 1] > FwdLUT[ i ] ) {
		last = i + 1;
		break;
	}

for( i = 0; i < lutSize; i++ ) {
	if( i <= low )
		InvLUT[ i ] = first;
	else if( i > = high )
		InvLUT[ i ] = last;
	else
		for( j = 0; j < lutSize - 1; j++ )
			if( FwdLUT[ j ] > = i ) {
				InvLUT[ i ] = j;
				break;
			}
	InvLUT[ i ] = clipRange(InvLUT[ i ], rangeLow, rangeHigh);
}

        - Picture reconstructon with mapping process for luma sample value

The output for this process is reconstructed luma sample array recLumaSamples.

The recLumaSamples is derived as follows:

–	If slice_type ! = I, the followign applies:

–	If tu_cbf_luma[ xCurr ][ yCurr ] equal to 1

recLumaSamples[ xCurr + i ][ yCurr + j ] = Clip1Y( InvLUT[ FwdLUT[predSamples[ i ][ j ] ] + resSamples[ i ][ j ] ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	Otherwise (cbf_luma[ xCurr ][ yCurr ] equal to 0)

recLumaSamples[ xCurr + i ][ yCurr + j ] = Clip1Y(predSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (slice_type = = I)

recLumaSamples[ xCurr + i ][ yCurr + j ] = Clip1Y( InvLUT[predSamples[ i ][ j ] + resSamples[ i ][ j ] ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

### 8.4.5.1.3	Picture reconstructon with mapping process for chroma sample value

The output for this process is reconstructed chroma sample array recChromaSamples.The variable varScale is derived as follows:

- If  slice_type = = I

varScale =chromaLUT[ InvLUT[Clip1Y( ( ij recLumaSamples[ (xCurr << 2 ) + i ][ (yCurr << 2) + j ]
 + nCurrSw * nCurrSh *2) / ( nCurrSw * nCurrSh *4 ) ) ] ]	(8-xxx)
	with i = 0..nCurrSw *2 − 1, j = 0..nCurrSh *2 – 1

- Otherwise ( slice_type ! = I )

varScale =chromaLUT[ Clip1Y( ( ij predLumaSample[ (xCurr << 2 ) + i ][ (yCurr << 2) + j ]
 + nCurrSw * nCurrSh *2) / ( nCurrSw * nCurrSh *4 ) ) ]	(8-xxx)
	with i = 0..nCurrSw *2 − 1, j = 0..nCurrSh *2 – 1

The recChromaSamples is derived as follows:

–	 If tu_cbf_cIdx [ xCurr ][ yCurr ] equal to 1, the followign applies:

recChromaSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1 ( predSamples[ i ][ j ] + Sign( resSamples[ i ][ j ] )
 * ( (Abs( resSamples[ i ][ j ] ) < < 16 ) + ( varScale >> 1)  /  varScale ) )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

–	 Otherwise (tu_cbf_cIdx[ xCurr ][ yCurr ] equal to 0)

recChromaSamples[ xCurr + i ][ yCurr + j ] = ClipCidx1(predSamples[ i ][ j ] )	(8-xxx)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1