| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0257 |
| --- | --- |

| Title: | CE4-related: Mismatch between text specification and reference software on clipping the positions of collocated blocks for alternative temporal motion vector prediction (ATMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

When performing alternative temporal motion vector prediction (ATMVP) in VTM-2.0.1, a motion offset is obtained from the motion vector of one of spatial neighboring blocks at the coding unit (CU) level and applied to derive the sub-CU-level motion information from collocated picture. To limit implementation complexity, it is guaranteed that only motion field of size (1 CTU + 4 columns) need to be accessed in the collocated picture. This is accomplished by clipping the collocated block position to be within this restricted region (1 CTU + 4 columns) when needed. In this contribution, it is reported that there is a mismatch between the VVC draft 2 and the test model software VTM-2.0.1 when clipping is performed to obtain the collocated block positions. Further, fixes are proposed to the test model software to align the clipping process to that of the draft specification.

In the VVC draft 2, the motion offset is directly applied to calculate the collocated block position for each sub-block in the collocated picture; if the collocated block is outside of the constrained region, then its position is clipped to the closest boundary of the constrained region. However, in the VTM-2.0.1 software, the motion offset is firstly clipped at the CU level to ensure that all the shifted sample positions of the CU (i.e., after the offset is applied) are inside the constrained region of the collocated picture; then, the same clipped offset is applied to all the sub-blocks within the CU to fetch the motion information from the collocated picture. In this document, it is proposed to change the software to align with the specification draft. Simulation results show that compared to the VTM-2.0.1 anchors, the proposed alignment provides 0.03% average luma BD-rate savings for both RA and LD without any impact to encoding and decoding time.

# Proposal

At the 11th JVET meeting, alternative temporal motion vector prediction (ATMVP) was adopted to the VVC draft 2 [1]. As per the text specification, the main process of the ATMVP can be summarized as follows:

- One motion offset is obtained from the MV of the first spatial merge candidate that refers to the collocated picture as reference picture.

- For each sub-block of the current CU,

- The motion offset is applied to determine the position of its collocated block inside the collocated picture.

- If the collocated block position is outside the constrained region, it is clipped to the closest boundary of the region.

- The motion information of the collocated block (after clipping if applicable)in the collocated picture is used to derive the motion information of the sub-block.

On the other hand, in the VTM-2.0.1 [2], the order of the ATMVP operations are summarized as:

- One motion offset is obtained from the MV of the first spatial merge candidate that refers to the collocated picture as reference picture.

- Clipping is applied to the motion offset, if needed, to make sure that all the shifted positions of the samples within the CU (i.e., after applying the motion offset) are inside the constrained region in the collocated picture.

- For each sub-block of the current CU,

- The clipped offset is applied to determine the position of its collocated block inside the collocated position.

- The motion information of the block that covers the collocated block in the collocated picture is used to derive the motion information of the sub-block.

As can be seen from the above, there is a mismatch between the VVC draft 2 and VTM-2.0.1 on when to clip the positions of the collocated blocks for the ATMVP derivation. Specifically, in the specification text, the clipping is applied to the location of each collocated block at sub-block-level whereas in the reference software the clipping is applied to the offset at CU-level. Figure 1 compares the difference between the two clipping methods.

Figure 1 Illustration of the clipping methods of collocated block positions in (left) VVC draft 2 and (right) VTM-2.0.1.

As the clipping method in VTM-2.0.1 clips the offset according to the CU size, the range of valid collocated positions derived by the method for one sub-block could be smaller than that derived by the clipping method in VVC draft 2. Without loss of generality, assume that the ATMVP sub-block size is 4×4 and the coordinate of the i-th sub-block inside the current CU is xsubi, ysubi. Then, by the clipping method in VTM-2.0.1, the range of valid offset values for the sub-block, i.e., [horMin, horMax] and [verMin, verMax], is calculated as

| horMax=minPicWidth, xctu+Wctu+4-xcu+Wcu horMin=max0, xctu-xcu verMax=minPicHeight, yctu+Hctu-ycu+Hcu verMin=max0, yctu-ycu | (1) |
| --- | --- |

where xcu, ycu and Wcu×Hcu are the coordinates and the size of the current CU, xctu, yctu and Wctu×Hctu are the coordinates and the size of the CTU that contains the CU, PicWidth and PicHeight are the width and the height of the picture. In comparison, the range of the valid offset values based on the clipping method in the specification draft is calculated as

| horMax=minPicWidth, xctu+Wctu+4-xsubi+4 horMin=max0, xctu-xsubi verMax=minPicHeight, yctu+Hctu-ysubi+4 verMin=max0, yctu-ysubi | (2) |
| --- | --- |

where xsubi, ysubi are the coordinates of the i-th sub-block.

To solve the mismatch, this document proposes to change the reference software to be aligned with the specification text, where the clipping is applied at the sub-block level. Software patch is included in the package for this fix.

# Simulation results

Simulations are carried out based on the BMS-2.0.1 reference software [3] based on the VTM configuration and follow the common test conditions as specified in [4].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.06% | 0.05% | 101% | 102% |
| Class A2 | -0.06% | -0.05% | -0.01% | 101% | 101% |
| Class B | 0.00% | -0.09% | -0.08% | 101% | 101% |
| Class C | 0.03% | 0.12% | -0.12% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.05% | 101% | 102% |
| Class D | 0.07% | -0.01% | 0.09% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.17% | -0.37% | 101% | 102% |
| Class C | 0.02% | 0.16% | -0.24% | 100% | 103% |
| Class E | -0.11% | -0.47% | 0.23% | 101% | 100% |
| Overall | -0.03% | -0.14% | -0.17% | 101% | 102% |
| Class D | 0.01% | 0.28% | -0.26% | 100% | 101% |

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001, July 2018, Ljubljana, Slovenia.

VTM-2.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1.

BMS-2.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1.

J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, the 11th JVET meeting, July 2018, Ljubljana, Slovenia.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).