| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0258 |
| --- | --- |

| Title: | CE4.1.7: Shape dependent control point selection for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Xiaoyu Xiu, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Affine motion model was adopted in VTM-2.0. It includes both 4-parameter and 6-parameter affine models. Shape dependent control point selection for 4-parameter affine mode was proposed at 10th meeting and it was included in CE4 tests. The control points for 4-parameter affine coding block are selected depending on the coding block shape. Experimental results show that compared to VTM-2.0.1 anchor, the shape dependent control point selection for 4-parameter affine coding achieves average Y BD-rate savings of -0.05% and -0.06% for RA and LD, respectively.

# Introduction

At the 11th JVET meeting, 4-parameter and 6-parameter affine motion models were adopted in VTM-2.0 reference software [2]. Shape dependent control point selection for 4-parameter affine mode was proposed in JVET-K0335 [1]. As shown in Figure 1 (a), if the block is coded with 4-parameter affine model and its block width is no smaller than the block height, the control points are defined at top-left and top-right; As shown in Figure 1 (b), if the block width is smaller than the block height, the control points are defined at top-left and bottom-left.

(a)                                                               (b)

Figure 1. Shape dependent control point selection: (a) CP in horizontal direction; (b) CP in vertical direction

The control point distance is used to select the affine merge candidate and order the inherited affine MV predictors. The control point distance is calculated as:

| d=max⁡(block_width, block_height);if 4-parameter affine modelblock_width+block_height;if 6-parameter affine model | (1) |
| --- | --- |

The affine merge candidate with maximum control point distance from five spatial neighbors is selected for affine merge mode. The inherited affine MV predictors are also ordered in a descending order using control point distance.

# Simulation results

This CE test was implemented based on the BMS-2.0.1 reference software and tested using the VTM configuration. The test condition and evaluation criteria that are used in CTC [3] are applied. There are three changes in this CE test: (a) applying shape dependent control point selection for explicit 4-parameter affine coding mode; (b) applying control point distance defined by Eq.(1) to select affine merge candidate; (c) ordering inherited affine MV predictor with control point distance defined by Eq.(1). Two tests are performed: CE4.1.7a includes three changes; CE4.1.7b includes change (a). Table 1 lists the CE4.1.7a simulation results compared to VTM-2.0.1 anchor. Table 2 lists the CE4.1.7b simulation results compared to VTM-2.0.1 anchor.

Table 1. Test_4.1.7a compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.02% | -0.01% | 99% | 100% |
| Class A2 | -0.06% | 0.03% | -0.07% | 99% | 100% |
| Class B | -0.06% | 0.01% | -0.06% | 100% | 100% |
| Class C | -0.03% | 0.12% | 0.06% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.04% | -0.02% | 100% | 99% |
| Class D | -0.04% | -0.09% | -0.04% | 101% | 99% |
| Class F (optional) | -0.07% | -0.09% | -0.14% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.16% | -0.09% | 97% | 100% |
| Class C | -0.01% | -0.11% | 0.01% | 97% | 100% |
| Class E | -0.15% | 0.05% | -0.76% | 98% | 100% |
| Overall | -0.06% | -0.09% | -0.23% | 97% | 100% |
| Class D | -0.05% | 0.56% | -0.18% | 100% | 100% |
| Class F (optional) | -0.12% | 0.49% | 0.03% | 98% | 99% |

Table 2. Test_4.1.7b compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | -0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | -0.06% | 100% | 101% |
| Class B | -0.03% | 0.05% | -0.09% | 100% | 100% |
| Class C | -0.05% | 0.11% | 0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.07% | -0.04% | 100% | 100% |
| Class D | -0.03% | -0.06% | -0.06% | 100% | 100% |
| Class F (optional) | -0.02% | -0.08% | -0.08% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.03% | -0.32% | 100% | 100% |
| Class C | 0.02% | -0.10% | -0.17% | 100% | 100% |
| Class E | -0.22% | 0.26% | -0.55% | 100% | 99% |
| Overall | -0.06% | 0.02% | -0.33% | 100% | 100% |
| Class D | -0.12% | 0.36% | -0.64% | 100% | 98% |
| Class F (optional) | -0.16% | 0.60% | 0.33% | 100% | 100% |

# References

Y. He, X. Xiu, Y. Yan, “CE4-related: Shape dependent control point selection for affine mode,” JVET-K0335, July 2018.

VTM-2.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.1.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, July 2018.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).