| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0261-v1 |
| --- | --- |

| Title: | CE6 – Set of Transforms (Tests 6.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mischa Siekmann, Christian Bartnik, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-440 mischa.siekmann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

As part of CE6 a unified signaling and selection scheme for primary and secondary transforms is tested together with two different sets of secondary transforms. The first test comprises 4x4 HyGT-based NSSTs only, while the second test also allows 8x8 NSSTs, which are implemented as Hierarchical Structured Matrix Transforms (HSMTs) [JVET-J0062]. The transform selection is done by introducing an intra mode and block size depending mapping from the Multi Transform Selection (MTS) flag and index to a pair of primary and secondary transforms. Allowing 4x4 NSSTs only (Test 3.1-B), gives average Luma BD-rate gains of 0.75%, 0.36% and 0.23% for AI, RA and LD coding, respectively. With 8x8 NSSTs (Test-3.1-C) the average Luma BD-rate gains increase to 1.67%, 0.95% and 0.45% for AI, RA and LD coding, respectively. While the encoder runtime could be slightly decreased (~97%) for both tests in AI configuration, the decoder runtime and the encoder runtimes for all other configurations remain about the same as the reference VTM 2.0.1.

It was also found that the fast search mode of MTS has a higher impact on the BD rate when MTS also comprises secondary transforms. While disabling the fast mode for AI configuration in VTM 2.0.1 gives an average BD gain of about 0.07% at an encoder runtime of 124%, the average Luma BD rate gains (AI) of Test B and Test C increase from 0.75% to 0.93% and 1.67% to 1.92%, respectively, at an encoder runtime of 123%.

# CE test description

In VTM 2.0 MTS was included which adaptively chooses the transform used for coding intra predicted Luma residuals. The fix set of candidates comprises the following 5 separable transforms (vertical, horizontal): { (DCT-2,DCT-2), (DST-7,DST-7), (DCT-8,DST-7), (DST-7,DCT-8), (DCT8-DCT-8) }.

We propose to adaptively change the set of transform candidates depending on the intra mode and block size. In doing so, each candidate is also defined as a pair of primary and secondary transform, where the identity transform can serve as secondary transform, this is, no secondary transform is applied in that case.

The actual candidate derivation is done by a table lookup. Each candidate is represented by 4 bit, 2 bit specify the primary transform as in index into the set: {(DCT-2,DCT-2), (DST-7,DST-7), (DCT-8,DST-7), (DST-7,DCT-8) }and the remaining 2 bit specify the NSST index. For each block size in {4,8,16,32}x{4,8,16,32} 13 sets of 5 transform candidates are defined.

Tests B and C differ in the types of applied secondary transforms. While in Test B, the secondary 4x4 – NSSTs as defined in BMS 2.0 are used for all primary transform sizes in {4,8,16,32}x{4,8,16,32}, in Test C they are only applied for blocks which have at least one dimension of 4. The remaining sizes use secondary transforms of size 8x8. The 8x8-NSSTs are implemented as HSMTs [4].

Note, for chroma residuals (DCT-2,DCT-2) without any secondary transform is used.

# Test results

According to the CE6 description in JVET-K1026 [1] and the common test conditions in JVET-K1010 [3], simulation were performed for intra-only (AI), random-access (RA) and low-delay B (LB) configuration with BMS-2.0.1 software in VTM configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | Off | AVX2 | GCC 7.2.1 | Linux |

## Test 6.3.1-B - Set of transforms – comprising a subset of MTS primary and 4x4 HyGT-NSSTs as defined in BMS.

Table 2 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.35% | -0.04% | -0.15% | 97% | 99% |
| Class A2 | -0.41% | -0.17% | -0.25% | 98% | 100% |
| Class B | -0.57% | -0.47% | -0.42% | 98% | 99% |
| Class C | -1.27% | -1.11% | -1.05% | 94% | 103% |
| Class E | -1.11% | -0.75% | -0.82% | 97% | 101% |
| Overall | -0.75% | -0.54% | -0.55% | 97% | 101% |
| Class D | -1.23% | -0.94% | -1.34% | 94% | 104% |
| Class F (optional) | -1.07% | -0.91% | -0.92% | 96% | 102% |

Table 3 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.25% | -0.04% | -0.15% | 99% | 101% |
| Class A2 | -0.25% | -0.08% | -0.18% | 99% | 99% |
| Class B | -0.32% | -0.20% | -0.14% | 100% | 101% |
| Class C | -0.58% | -0.20% | -0.55% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.15% | -0.26% | 99% | 101% |
| Class D | -0.55% | -0.38% | -0.34% | 100% | 102% |
| Class F (optional) | -0.51% | -0.25% | -0.40% | 100% | 100% |

Table 4 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.07% | -0.06% | 101% | 102% |
| Class C | -0.21% | -0.05% | -0.57% | 100% | 102% |
| Class E | -0.51% | -0.09% | -0.24% | 102% | 103% |
| Overall | -0.23% | -0.01% | -0.27% | 101% | 103% |
| Class D | -0.09% | 0.23% | -0.62% | 101% | 102% |
| Class F (optional) | -0.31% | 0.26% | -0.70% | 101% | 103% |

## Test 6.3.1-C - Set of transforms – 8x8 HSMT-NSST.

Table 7 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.56% | -0.77% | -0.99% | 96% | 99% |
| Class A2 | -1.07% | -0.58% | -0.61% | 97% | 100% |
| Class B | -1.37% | -0.95% | -0.93% | 98% | 100% |
| Class C | -2.32% | -1.41% | -1.35% | 96% | 102% |
| Class E | -2.02% | -1.35% | -1.29% | 98% | 100% |
| Overall | -1.67% | -1.03% | -1.04% | 97% | 100% |
| Class D | -2.02% | -1.38% | -1.70% | 97% | 103% |
| Class F (optional) | -1.86% | -1.24% | -1.27% | 97% | 102% |

Table 8 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.90% | -0.21% | -0.39% | 97% | 101% |
| Class A2 | -0.65% | 0.11% | -0.19% | 99% | 98% |
| Class B | -0.90% | -0.44% | -0.19% | 99% | 99% |
| Class C | -1.28% | -0.37% | -0.41% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.95% | -0.27% | -0.29% | 99% | 100% |
| Class D | -0.92% | -0.61% | -0.13% | 100% | 100% |
| Class F (optional) | -1.03% | -0.43% | -0.49% | 100% | 100% |

Table 9 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.30% | -0.40% | 100% | 103% |
| Class C | -0.52% | -0.21% | -0.27% | 100% | 103% |
| Class E | -0.54% | -0.83% | -0.77% | 100% | 101% |
| Overall | -0.45% | -0.40% | -0.45% | 100% | 102% |
| Class D | -0.32% | -0.38% | -0.29% | 100% | 100% |
| Class F (optional) | -0.45% | 0.26% | -0.50% | 100% | 102% |

## Disabling Fast Encoder Search of Intra AMT (MTS)

Table 2 Result for VTM 2.0.1 with AI configuration and AMT-Fast disabled

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.10% | -0.02% | -0.05% | 127% | 101% |
| Class A2 | -0.06% | 0.04% | -0.01% | 123% | 101% |
| Class B | -0.05% | 0.07% | 0.08% | 123% | 101% |
| Class C | -0.08% | 0.07% | 0.18% | 119% | 101% |
| Class E | -0.10% | 0.11% | -0.01% | 132% | 101% |
| Overall | -0.07% | 0.06% | 0.05% | 124% | 101% |
| Class D | -0.05% | 0.18% | -0.10% | 115% | 101% |
| Class F (optional) | -0.02% | -0.06% | -0.04% | 122% | 100% |

Table 3 Result for Test-B with VTM AI configuration and AMT-Fast disabled

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.64% | -0.09% | -0.33% | 126% | 100% |
| Class A2 | -0.54% | -0.31% | -0.21% | 123% | 102% |
| Class B | -0.73% | -0.45% | -0.37% | 123% | 100% |
| Class C | -1.41% | -1.05% | -1.02% | 114% | 103% |
| Class E | -1.31% | -0.76% | -0.85% | 131% | 102% |
| Overall | -0.93% | -0.55% | -0.56% | 123% | 101% |
| Class D | -1.32% | -1.00% | -1.09% | 110% | 106% |
| Class F (optional) | -1.17% | -0.92% | -0.95% | 119% | 102% |

Table 4 Result for Test-C with VTM AI configuration and AMT-Fast disabled

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.92% | -0.89% | -1.22% | 124% | 99% |
| Class A2 | -1.28% | -0.59% | -0.64% | 122% | 101% |
| Class B | -1.60% | -0.88% | -0.90% | 124% | 99% |
| Class C | -2.51% | -1.46% | -1.41% | 116% | 104% |
| Class E | -2.34% | -1.33% | -1.30% | 131% | 101% |
| Overall | -1.92% | -1.04% | -1.09% | 123% | 101% |
| Class D | -2.15% | -1.33% | -1.52% | 114% | 103% |
| Class F (optional) | -2.04% | -1.26% | -1.29% | 121% | 102% |

# Complexity metrics

In test 6.3.1-B the transform candidate lists are populated with transforms defined in BMS-2.0.1. Hence, the complexity metrics in terms of memory requirements for storing and applying the transform cores and the number of needed arithmetic operations are the same as in BMS-2.0.1 and are summarized below. The table of candidate lists requires 4bit per candidate (i.e. [shape x quantized intra modes x #candidates] 16x13x5x4 bits = 520 bytes in total).

## Primary transforms

Transform types: DCT-2, DCT-8, DST-7

Allowed combinations [vertical, horizontal]: [DCT-2, DCT-2], [DST-7, DST-7], [DCT-8, DST-7], [DST-7, DCT8]

Transform block sizes: WxH with W,H in { 4, 8, 16, 32}

## HyGT-4x4 secondary transform as in BMS-2.0.1

| Type | HyGT – 4x4 |
| --- | --- |
| Number of passes: | 8 (2 rounds, 4 layers each) |
| Multiplications per pass: | 32 |
| Additions per pass: | 32 |
| Bit-shifts per pass: | 16 |
| Parameterized permutation layer: | 1 |
| Multiplications: | 256 |
| Additions: | 256 |
| Bit-shifts: | 145 |
| Parameter: | 64 x Givens-angles (8bit), 1 x permutation (16x4 bit) |
| Precision of arithmetic operations | +15 bits (rel. to precision of primary coefficients) |
| Precision intermediate data | +5 bits (rel. to precision of primary coefficients) |
| Storing single transform core: | 576 bits |
| Number of transforms: | 103 |
| Storing transform cores: | 59,328 bits |

## HSMT-8x8 secondary transform (Test-C only)

| Type | HSMT–8x8 |
| --- | --- |
| Number of passes: | 1 |
| Parameterized permutation layer: | 1 |
| Multiplications: | 24x64 = 1536 |
| Additions: | 24x64 = 1536 |
| Bit-shifts: | 24x8 = 192 |
| Parameter: | 24 x 64 coefficients (9bit), 1 x permutation (64x6 bit) |
| Precision of arithmetic operations |  |
| Precision intermediate data |  |
| Storing single transform core: | 14,208 bits |
| Number of transforms: | 37 |
| Storing transform cores: | 525,696 bits |

# References

- A. Said, X. Zhao “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-K1026, Ljubljana, Slovenia, July 2018.

- M. Siekmann, B. Stallenberger, C. Bartnik, J. Pfaff, D. Marpe, H. Schwarz, T. Wiegand „Set of Transforms“, JVET-J0040, San Diego, USA, April 20018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, V. Seregin, M. Karczewicz, “Non-Separable Secondary Transform Implementations with Reduced Memory via Hierarchically Structured Matrix-based Transforms“, JVET-J0062, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).