| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12Oct. 2018 | Document: JVET-L0263-v2 |
| --- | --- |

| Title: | CE6-1.8: MTS with DCT-2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Leannec, Edouard Francois (Technicolor) | Tel: Email: | karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports results from CE6-1.8. In this contribution, a new set of MTS transforms is proposed. The set contains DCT-2 with two additional transform: either DST-7 and DCT-8 (Test#1) or DST-4 and DST-4 (Test#2). A mapping table between the intra prediction mode and prediction block size to the transform pair is also defined. Reported performance for Test#1 compared to the anchor are of -0.05%, -0.02% and -0.04% BD-rate variations for AI, RA and LDB configurations, respectively. Reported performance for Test#2 compared to the anchor are of 0.03%, 0.08% and -0.03% BD-rate variations for AI, RA and LDB configurations, respectively. Solution of test#2 is reportedly a design simplification of MTS, with negligible impact on the coding performance. Test#2 is also reported to give a BD-rate variation of -0.20%, -0.10% and -0.08% in AI, RA and LDB configurations, compared to CE6-1.7a consisting in replacing DCT-8/DST-7 of MTS by DCT-4/DST-4.

# Proposed method

In a previous contribution [1], a design of MTS with mapping table was introduced. However, [1] considers three transforms in addition to DCT-2, namely: DST-4, DST-7 and DCT-8. A mapping between the intra prediction mode, transform block size and transform set is described as described in the pseudo-code below:

nSzIdx =min(3,min(g_aucLog2[iWidth]-2, g_aucLog2[iHeight]-2));

nMdIdx = ucMode>DIA_IDX ?(NUM_LUMA_MODE + 1 - ucMode) : ucMode;

nTrSet = g_aucIpmToTrSet[nSzIdx][nMdIdx];

nTrIdxVer = g_aucTrSet[nTrSet][ucTrIdx][ucMode> DIA_IDX ? 1 : 0];

nTrIdxHor = g_aucTrSet[nTrSet][ucTrIdx][ucMode> DIA_IDX ? 0 : 1];

where the mapping table is defined as g_aucTrSet, indexed by a mode- and size- dependent index (nTrSet), in order to choose among the 7 possible sets of pairs, a transform index (ucTrIdx) to choose among the 4 pairs of the selected set, and the horizontal/vertical index (0/1), as follows:

g_aucTrSet[7][4][2] =

{

{ {DST4,DST4}, { DST7,DST7}, {	DST4,DCT8}, {	DCT8,DST4} }, //SST1

{ {DST7,DST7}, { DST7,DCT2}, {	DCT2,DST7}, {	DCT2,DCT8} }  //SST2

{ {DST7,DST7}, { DST7,DCT8}, {	DCT8,DST7}, {	DCT2,DST7} }  //SST3

{ {DST4,DST4}, { DST4,DCT2}, {	DCT8,DST4}, {	DCT2,DST7} }  //SST4

{ {DST4,DST7}, { DST7,DCT2}, {	DCT8,DST7}, {	DCT2,DST7} }  //SST5

{ {DST7,DST7}, { DST7,DCT2}, { 	DCT8,DST7}, {	DCT2,DST7} }  //SST6

{ {DST7,DST7}, { DST7,DCT2}, {	DCT2,DST7}, {	DCT2,DST7} }  //SST7

}

In the current proposal, to comply with the current design of VTM2 [3], the design of [1] is modified as follows. The proposed MTS contains only two transforms, DST-7 and DCT-8. DCT-2 is also considered in the set of transforms, and the mapping table is defined as follows:

g_aucTrSet[7][4][2] =

{

{ {DST7, DST7 },{ DST7, DCT2 },{ DST7, DCT8 },{ DCT8, DST7 } },  //SST1

{ {DST7, DST7 },{ DST7, DCT2 },{ DCT2, DST7 },{ DCT2, DCT8 } },  //SST2

{ {DST7, DST7 },{ DST7, DCT8 },{ DCT8, DST7 },{ DCT2, DST7 } },  //SST3

{ {DST7, DST7 },{ DST7, DCT2 },{ DCT8, DST7 },{ DCT2, DST7 } },  //SST4

{ {DST7, DST7 },{ DST7, DCT2 },{ DCT8, DST7 },{ DCT2, DST7 } },  //SST5

{ {DST7, DST7 },{ DST7, DCT2 },{ DCT8, DST7 },{ DCT2, DST7 } },  //SST6

{ {DST7, DST7 },{ DST7, DCT2 },{ DCT2, DST7 },{ DCT2, DST7 } },  //SST7

}

Alternatively to DST-7 and DCT-8, DST-4 and DCT-4 can be advantageously used as N-point DST-4 and DCT-4 can be both inherited from 2N-point DCT-2. Therefore, no extra memory/implementation is required for these transforms. DST-4 and DCT-4 can be employed by modifying the aucTrSetto:

g_aucTrSet[7][4][2] =

{

{ {DST4, DST4 },{ DST4, DCT2 },{ DST4, DCT8 },{ DCT4, DST4 } },  //SST1

{ {DST4, DST4 },{ DST4, DCT2 },{ DCT2, DST4 },{ DCT2, DCT4 } },  //SST2

{ {DST4, DST4 },{ DST4, DCT4 },{ DCT4, DST4 },{ DCT2, DST4 } },  //SST3

{ {DST4, DST4 },{ DST4, DCT2 },{ DCT4, DST4 },{ DCT2, DST4 } },  //SST4

{ {DST4, DST4 },{ DST4, DCT2 },{ DCT4, DST4 },{ DCT2, DST4 } },  //SST5

{ {DST4, DST4 },{ DST4, DCT2 },{ DCT4, DST4 },{ DCT2, DST4 } },  //SST6

{ {DST4, DST4 },{ DST4, DCT2 },{ DCT2, DST4 },{ DCT2, DST4 } },  //SST7

}

# Experimental Results

Two tests are performed. The first test (Test#1) corresponds to a transform mapping table containing DCT-2, DST-7 and DCT-8.

The second test (Test#2) corresponds to a transform mapping table containing DCT-2, DST-4 and DCT-4.

Using the common testing conditions of VTM software [3], and the default VTM2.0.1 software as an anchor, the following results are obtained for the AI and RA configurations.

Note that the encoding and decoding runtime initially delivered in the CE6 branch were not properly estimated due to the usage of inhomogeneous cores. The values reported in the current document are estimated with an homogeneous computer grid.

## Test#1 - DCT-2, DST-7 and DCT-8

Table 1.BD-rate results of Test#1 in AI configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.10% | 0.05% | 0.04% | 102% | 101% |
| Class A2 | -0.04% | 0.06% | 0.02% | 100% | 99% |
| Class B | -0.05% | 0.18% | 0.14% | 101% | 99% |
| Class C | -0.01% | 0.09% | 0.24% | 101% | 99% |
| Class E | -0.07% | 0.04% | 0.05% | 103% | 102% |
| Overall | -0.05% | 0.09% | 0.11% | 101% | 100% |
| Class D | -0.05% | 0.17% | 0.00% | 101% | 100% |

Table 2.BD-rate results of Test#1 in RA configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.07% | 0.15% | 0.14% | 101% | 99% |
| Class A2 | -0.05% | -0.11% | -0.04% | 101% | 101% |
| Class B | -0.01% | 0.06% | 0.01% | 101% | 102% |
| Class C | 0.01% | 0.23% | 0.14% | 99% | 98% |
| Overall | -0.02% | 0.09% | 0.06% | 100% | 100% |
| Class D | -0.03% | -0.14% | 0.32% | 98% | 98% |

Table 3. BD-rate results of Test#1 in LDB configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.14% | 0.08% | 101% | 102% |
| Class C | 0.07% | -0.02% | -0.32% | 98% | 97% |
| Class E | -0.24% | 0.47% | -0.77% | 107% | 103% |
| Overall | -0.04% | 0.05% | -0.27% | 101% | 101% |
| Class D | 0.05% | 0.41% | 0.17% | 98% | 103% |

We have also experimented the impact when MTS is activated for inter coding. For this, the following settings are used:

EMT = 3

EMTFast = 3

The results on RA configuration are provided below:

Table 4. BD-rate results of Test#1 in RA configuration (EMT=3, EMTFast=3).

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.04% | 0.02% | 0.10% |
| Class A2 | -0.04% | -0.08% | 0.03% |
| Class B | 0.00% | 0.01% | -0.03% |
| Class C | 0.02% | 0.08% | 0.11% |
| Class E |  |  |  |
| Overall | -0.01% | 0.01% | 0.05% |
| Class D | -0.03% | -0.09% | 0.48% |

## Test#2 - DCT-2, DST-4 and DCT-4

Table 5. BD-rate results of Test#2 in AI configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.42% | 0.69% | 0.70% | 102% | 98% |
| Class A2 | 0.27% | 0.26% | 0.21% | 99% | 97% |
| Class B | 0.08% | 0.17% | 0.13% | 100% | 97% |
| Class C | -0.32% | -0.25% | -0.03% | 100% | 99% |
| Class E | -0.21% | -0.08% | -0.26% | 102% | 100% |
| Overall | 0.03% | 0.14% | 0.14% | 101% | 98% |
| Class D | -0.46% | -0.21% | -0.39% | 101% | 100% |

Table 6. BD-rate results of Test#2 in RA configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.19% | 0.36% | 0.48% | 102% | 100% |
| Class A2 | 0.18% | 0.27% | 0.29% | 100% | 101% |
| Class B | 0.12% | 0.09% | 0.15% | 101% | 102% |
| Class C | -0.12% | 0.03% | 0.06% | 98% | 99% |
| Overall | 0.08% | 0.16% | 0.22% | 100% | 101% |
| Class D | -0.13% | -0.38% | -0.02% | 98% | 98% |

Table 7. BD-rate results of Test#2 in LDB configuration.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.08% | -0.02% | 100% | 101% |
| Class C | -0.04% | -0.13% | -0.27% | 97% | 97% |
| Class E | -0.20% | 0.18% | 0.33% | 105% | 99% |
| Overall | -0.03% | -0.03% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.24% | 0.48% | 98% | 102% |

Similarly, the impact when MTS is activated for inter coding is also studied as in Sec. Error! Reference source not found..  The results on RA configuration are provided below:

Table 8 BD-rate results of Test#2 in RA configuration (EMT=3, EMTFast=3).

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.20% | 0.10% | 0.33% |
| Class A2 | 0.21% | 0.25% | 0.27% |
| Class B | 0.09% | -0.02% | -0.06% |
| Class C | -0.22% | -0.17% | -0.02% |
| Class E |  |  |  |
| Overall | 0.05% | 0.02% | 0.09% |
| Class D | -0.33% | -0.43% | -0.01% |

# Comments

The results of Test#1 show a little average improvement over the current design of MTS in VTM2. Test#2, where the type IV transforms are used, results in a small average loss. This loss is acceptable when compared to the large simplification of the MTS design thanks to the factorization of transforms from the DCT-2.

Additionally, it is also important to show the improvement when the proposed method is compared to the direct replacement of the MTS transform by the type IV transforms, which is proposed in the core experiment about transform coding (CE6-1.7a [2]). To do so, the results of CE6-1.7a are considered as the anchor, and the Test#2 results as the test data. The following results are obtained:

Table 9. BD-rate results of Test#2 in AI configuration using CE6-1.7a as anchor.

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.25% | 0.05% | 0.05% |
| Class A2 | -0.20% | 0.10% | 0.09% |
| Class B | -0.24% | 0.11% | 0.07% |
| Class C | -0.14% | -0.06% | 0.07% |
| Class E | -0.19% | 0.06% | -0.05% |
| Overall | -0.20% | 0.05% | 0.05% |
| Class D | -0.18% | 0.06% | 0.06% |

Table 10. BD-rate results of Test#2 in RA configuration using CE6-1.7a as anchor.

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.11% | 0.00% | 0.08% |
| Class A2 | -0.06% | -0.03% | 0.05% |
| Class B | -0.12% | -0.08% | 0.01% |
| Class C | -0.10% | -0.08% | -0.11% |
| Overall | -0.10% | -0.06% | 0.00% |
| Class D | -0.04% | -0.24% | -0.30% |

Table 11. BD-rate results of Test#2 in LDB configuration using CE6-1.7a as anchor.

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -0.02% | -0.14% | -0.32% |
| Class C | -0.02% | -0.01% | 0.03% |
| Class E | -0.28% | -0.08% | 0.89% |
| Overall | -0.08% | -0.08% | 0.10% |
| Class D | 0.03% | -0.41% | 0.70% |

As can be seen, the proposed method highly improves the performance. The method provides a much performant way of using the type IV transforms for MTS design.

# Conclusion

In this contribution, a method for designing MTS is presented. It is based on defining a table that maps the intra prediction mode and transform size to the MTS pairs. It shows a little improvement over the current MTS design. More importantly, when the type IV transforms are used, the method is much performant than the direct replacement of DST-7/DCT-8 by DST-4/DCT-4 without a mapping table.

# Reference

- Karam Naser, Fabrice Le Leannec and Edouard Francois, “CE6: Report of CE6.1.3 (Transform reduction in AMT), with further reduction via DST-4 inheritance” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0264,

- Karam Naser, Fabrice Le Leannec, Edouard Francois, Kiyofumi Abe, Tadamasa Toma, “CE 6-1.7a: AMT simplification by reusing DCT-2 partial butterfly - Change AMT transform to DST-4/DCT-4” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0264,

- J. Chen, Y. Ye, S. Kim., “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).