#### General

…

The derivation process for motion vector arrays from affine control point motion vectors as specified in subclause 8.3.3.7 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma prediction block height cbHeight, the number of control point motion vectors numCpMv, the control point motion vectors cpMvL1[ cpIdx ], cpMvL0[ cpIdx ] with cpIdx being 0..2, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ], the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) − 1, ySbIdx = 0 .. ( cbHeight >> 2 ) − 1, and the interweaved motion vector array mvLXIW[xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) , ySbIdx = 0 .. ( cbHeight >> 2 ) , X being 0 and 1 as output.

…

### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvL0[ cpIdx ] and cpMvL1[ cpIdx ], with cpIdx = 0..numCpMv − 1,

- the prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) − 1, ySbIdx = 0 .. ( cbHeight >> 2 ) − 1, X being 0 and 1,

- the luma subblock motion vector array mvLXIW[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) , ySbIdx = 0 .. ( cbHeight >> 2 ) , X being 0 and 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..( cbWidth >> 2 ) − 1, ySbIdx = 0 .. ( cbHeight >> 2 ) − 1, X being 0 and 1.

….

- For xSbIdx = 0..( cbWidth >> 2 ) − 1 and ySbIdx = 0..( cbHeight >> 2 ) − 1, the following applies:

  - The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-338)

yPosCb = 2 + ( ySbIdx << 2 )		(8-339)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-340)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-341)

  - The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

- For xSbIdx = 0..( cbWidth >> 2 ) and ySbIdx = 0..( cbHeight >> 2 ) , the following applies:

  - The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = Clip3(1, cbWidth -1, 4 + ( ( xSbIdx -1) << 2 )))	(8-xxx)

yPosCb = Clip3(1, cbHeight -1, 4 +( ( ySbIdx -1) << 2 )))	(8-xxx)

mvLXIW[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-xxx)

mvLXIW[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-xxx)

  - The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLXIW[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLXIW[ xSbIdx ][ ySbIdx ] as output.

…

#### General

…

If MotionModelIdc[ xCb ][ yCb ] is not equal to 0, and PredFlagL0[ xCb ][ yCb ]+ PredFlagL1[ xCb ][ yCb ]=1, the prediction samples inside the current luma coding block, predSamplesL[ xL + xSb ][ yL + ySb ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the interweaved prediction process specified in clause 8.3.4.5 with ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture, the luma coding block width cbWidth, the luma coding block height cbHeight and the sample arrays predSamplesL0L if predFlagL0 is equal to 1, or predSamplesL1L if predFlagL1 is equal to 1, and the variables predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 as inputs.

#### Reference picture selection process

…

#### 8.3.4.5 Interweaved prediction process

This process is invoked when decoding coding unit when CuPredMode[ xCb ][ yCb ] is not equal to MODE_INTRA, and MotionModelIdc[ xCb ][ yCb ] is not equal to 0, and PredFlagL0[ xCb ][ yCb ]+ PredFlagL1[ xCb ][ yCb ]=1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the luma interweaved motion vectors in 1/16 fractional-sample accuracy mvL0IW[ xSbIdx ][ ySbIdx ] and mvL1IW[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX, and ySbIdx = 0 .. numSbY,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1.

- an (cbWidth)x(cbHeight) array predSamplesLXL of luma prediction samples,

Outputs of this process are:

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

Let predSamplesIWL be a (cbWidth)x(cbHeight) arrays of predicted luma sample values.

Let xSbBegin = 1, xSbEnd = numSbX -1, ySbBegin = 0, ySbEnd = numSbY when cbWidth is equal or greater than cbHeight; Otherwise, xSbBegin = 0, xSbEnd = numSbX, ySbBegin = 1, ySbEnd = numSbY-1.

Copy all samples in predSamplesL to predSamplesIWL.

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = xSbBegin .. xSbEnd, and ySbIdx = ySbBegin .. ySbEnd, the following applies:

  - The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + Clip3( 0, cbWidth -2, 2+(xSbIdx – 1)<<2), yCb + Clip3( 0, cbHeight -2, 2+(ySbIdx – 1)<<2) )	(8-xxx)

  - The width and the height of the current luma coding subblock are derived as follows:

sbWidth = ( xSbIdx == 0 || xSbIdx == numSbX? 2: 4)

sbHeight = ( ySbIdx == 0 || ySbIdx == numSbY? 2: 4)

  - For X being each of 0 and 1, when predFlagLX is equal to 1, the following applies:

The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples is derived by invoking the process specified in clause 8.3.4.2 with refIdxLX as input.

The array predSamplesIWL are derived by invoking the fractional sample interpolation process specified in clause 8.3.4.3 with the luma location ( xSb, ySb ), the luma coding subblock width sbWidth, the luma coding subblock height sbHeight, the motion vectors mvLXIW[ xSb ][ ySb ], and the reference arrays refPicLXL as inputs.

For each luma sample location ( xS, yS ) with xS = xCb .. xCb+cbWidth -1, and yS = 0 .. yCb+ cbHeight -1, the following applies:

Weighting values w0 and w1 are derived as follows:

- w0 = ((xS%4 == 1 || xS%4 == 2) && (yS%4 == 1 || yS%4 == 2))? 3 : 1;

- w1 = (((xS+2)%4 == 1 || (xS+2)%4 == 2) && ((yS+2)%4 == 1 || (yS+2)%4 == 2))? 3 : 1;

- If w0 is equal to w1, set w0 and w1 to be 2.

predSamplesL[xS][yS] = (w0* predSamplesLXL [xS][yS]  + w1*predSamplesIWL[xS][yS]+2)>>2.