| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0269-v2 |
| --- | --- |

| Title: | CE10: Interweaved Prediction for Affine Motion Compensation (Test 10.5.1 and Test 10.5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With the affine motion compensation (AMC) in the VTM, a coding-block is divided into 4×4 sub-blocks, and each sub-block is assigned with individual motion vectors derived by one affine model. In this contribution, an interweaved prediction approach is proposed for AMC. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. The interweaved prediction is only applied on the luma-component for affine-coded blocks with uni-prediction. Two tests are conducted in this contribution. In CE Test 10.5.1, interweaved prediction is applied on the whole block. In CE Test 10.5.2, interweaved prediction is only applied on the central part of the block. Simulation results reportedly show 0.40% and 0.35% BD-rate savings for CE Test 10.5.1 and Test 10.5.2, respectively, under Random Access (RA) configurations.

# Introduction

Fig. 1. 4*4 sub-blocks in a 16*16 AMC-coded block.

With affine motion compensation (AMC) in the VTM, a coding-block is divided into sub-blocks as small as 4×4, each of which is assigned with an individual motion vector (MV) derived by the affine model, as shown in Fig. 1. With the 4-parameter or 6-parameter affine model, a MV is derived from MVs of the two or three control points.

AMC faces a dilemma on the size of sub-blocks. With smaller sub-blocks, AMC can achieve a better coding performance but suffers a higher complexity burden.

# Interweaved Prediction

To address the dilemma of AMC, interweaved prediction as shown in Fig. 2 is proposed to achieve a finer granularity of MVs without increasing the complexity too much.

First, a coding block is divided into sub-blocks with two different dividing patterns. The first dividing pattern is the same as that in VTM as pattern 0 shown in Fig. 2, while the second dividing pattern also divides the coding-block into 4*4 sub-blocks but with a 2*2 offset as pattern 1 shown in Fig. 2.

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each sub-block in a dividing pattern is derived from CPMVs by the affine model.

The final prediction is calculated as a weighted-sum of the two auxiliary predictions, formulated as:

| . | (1) |
| --- | --- |

As shown in Fig. 3, an auxiliary prediction sample located at the center of a sub-block is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a sub-block is associated with a weighting value 1.

In this contribution, the proposed interweaved prediction is only applied on the luma-component for affine-coded blocks with uni-prediction.

Two tests are conducted in CE10:

- Test 10.5.1: Interweaved prediction is applied on the whole block.

- Test 10.5.2: Interweaved prediction is only applied on the central part of the block. The width and height of current block is denoted as W and H.

  - If W >= H, interweaved prediction is not applied on the left and right 2 * H regions, as shown in Fig. 4 (a).

  - Otherwise, interweaved prediction is not applied on the top and right W * 2 regions, as shown in Fig. 4 (b).

Fig. 2. Interweaved prediction procedure.

Fig. 3. Weighting values in a sub-block.

| (a) | (b) |
| --- | --- |

Fig. 4 Interweaved prediction is not applied on the shaded regions.

To investigate the bandwidth requirement of the proposed method, the numbers of samples required in the MC procedure are summarized in Table 1. Test 10.5.1 will increases the worst-case bandwidth when H is equal to 8 and W is smaller than 64; Test 10.5.1 does not increase the worst-case bandwidth in any case.

Table 1: Samples required in MC

| Block Size | Affine with bi-prediction | Test 10.5.1 | Test 10.5.2 |
| --- | --- | --- | --- |
| 8*8 | 2*(11*11*4+5*5*4*2) =1368 | (11*11*4+5*5*4*2)+ 11*11+11*9*2+9*11*2+9*9*4 =1525 | (11*11*4+5*5*4*2)+ 11*11+11*9*2 =1003 |
| 16*8 | 2*(11*11*8+5*5*8*2) =2736 | (11*11*8+5*5*8*2)+11*11*3+11*9*6+9*11*2+9*9*4=2847 | (11*11*8+5*5*8*2)+ 11*11*3+11*9*6 =2325 |
| 32*8 | 2*(11*11*16+5*5*16*2) =5472 | (11*11*16+5*5*16*2)+ 11*11*7+11*9*14+9*11*2+9*9*4 =5491 | (11*11*16+5*5*16*2)+ 11*11*7+11*9*14 =4969 |
| 64*8 | 2*(11*11*32+5*5*32*2) =10944 | (11*11*32+5*5*32*2)+ 11*11*15+11*9*30+9*11*2+9*9*4 =10779 | (11*11*32+5*5*32*2)+ 11*11*15+11*9*30 =10257 |
| 128*8 | 2*(11*11*64+5*5*64*2) =21888 | (11*11*64+5*5*64*2)+ 11*11*31+11*9*62+9*11*2+9*9*4 =21355 | (11*11*64+5*5*64*2)+11*11*31+11*9*62 =20833 |
| 16*16 | 2*(11*11*16+5*5*16*2) =5472 | (11*11*16+5*5*16*2)+ 11*11*9+11*9*6+9*11*6+9*9*4 =5337 | (11*11*16+5*5*16*2)+ 11*11*9+11*9*6 =4419 |

# Experimental Results

The proposed enhanced interweaved prediction is implemented on BMS-2.0.1, and results of Random Access (RA) and Low-delay B (LB) configuration for Test 10.5.1 and Test 10.5.2 are summarized in Table 2 and Table 3, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 2: Restuls for Test 10.5.1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.06% | -0.05% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.77% | -0.18% | -0.16% | 102% | 102% |  |  |  |  |  |
| Class B | -0.50% | -0.24% | -0.18% | 101% | 102% | -0.15% | -0.22% | -0.43% | 103% | 104% |
| Class C | -0.21% | 0.06% | -0.05% | 101% | 102% | -0.12% | -0.06% | -0.27% | 103% | 103% |
| Class E |  |  |  |  |  | -0.42% | -0.55% | -0.09% | 103% | 99% |
| Overall | -0.40% | -0.09% | -0.12% | 101% | 102% | -0.21% | -0.25% | -0.29% | 103% | 102% |

Table 3: Results of Test 10.5.2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | -0.01% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.69% | -0.15% | -0.19% | 101% | 102% |  |  |  |  |  |
| Class B | -0.42% | -0.24% | -0.27% | 101% | 101% | -0.16% | -0.18% | -0.39% | 102% | 101% |
| Class C | -0.19% | 0.10% | -0.04% | 102% | 102% | -0.17% | -0.15% | -0.28% | 101% | 97% |
| Class E |  |  |  |  |  | -0.50% | -0.15% | -0.13% | 104% | 97% |
| Overall | -0.35% | -0.08% | -0.14% | 101% | 101% | -0.25% | -0.16% | -0.29% | 102% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, Jul. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).