| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0274-v2 |
| --- | --- |

| Title: | CE7: Transform coefficient coding with reduced number of regular-coded bins (tests 7.1.3a, 7.1.3b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Tung Nguyen, Detlev Marpe, Thomas Wiegand Marta Karczewicz, Muhammed Coban, Jie Dong | Tel: Email: | +49 30 31002 226 heiko.schwarz@hhi.fraunhofer.de tung.nguyen@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de martak@qti.qualcomm.com mcoban@qti.qualcomm.com jiedong@qti.qualcomm.com |
| Source: | Fraunhofer HHI and Qualcomm Inc. |  |  |

_____________________________

version 2 changes:

- completed results; added an analysis for the bin-to-bit ratio / padding bytes

- added regular bin count restriction for chroma 2x2

- updated CABAC initialization tables for sig_flag, par_flag, gtX_flag’s for version B

# Abstract

In this document, two alternative entropy coding methods for transform coefficient levels are described. The two methods have in common that (a) the maximum number of regular coded bins is reduced relative to VTM-2, (b) for each transform coefficient, the local template of neighboring already coded data is evaluated only once, and (c) the absolute levels for a part of the transform coefficients of a subblock can be completely coded in bypass mode. The maximum number of regular-coded bins per subblock (not including the coded_subblock_flag and syntax elements coded before a coded_subblock_flag) is equal to 36 for the first method (version A) and equal to 32 for the second method (version B), while in VTM-2, this maximum number is equal to 64.  While version A uses the same concept for coding par_flag and gt1_flag as VTM-2, in version B, the coding order and the meaning of par_flag and gt1_flag is modified.  This change yields smaller bin—to-bit ratios, but introduces additional bin-to-bin dependencies.

For the common test conditions, the following average Bjøntegaard delta rates are reported:

version A (CE-7.1.3):	  0.03% AI,   0.03% RA,    0.06% LB;
	version B (CE-7.1.3b):	−0.18% AI, −0.07% RA, −0.15% LB.

For a configuration, in which dependent quantization and RDOQ are disabled (referred to as low-complexity encoding configuration in the CE description), the following Bjøntegaard delta rates are reported:

version A (CE-7.1.3):	  0.02% AI,   0.02% RA,    0.01% LB;
	version B (CE-7.1.3b):	  0.04% AI,   0.08% RA,    0.00% LB.

A part of the coding gains for version B and common test conditions reportedly result from the effect that no cabac_zero_word’s are inserted for version B. When excluded this effect by removing the cabac_zero_word’s from all bitstream (including the VTM-2 bitstreams), the following average Bjøntegaard delta rates are reported for the common test conditions

version A (CE-7.1.3):	  0.05% AI,   0.06% RA,    0.08% LB;
	version B (CE-7.1.3b):	−0.03% AI,   0.00% RA, −0.09% LB.

In version 2 of this document, additional restrictions for 2x2 chroma subblocks are imposed and, for version B, new CABAC initialization tables for the syntax elements sig_flag, par_flag, rem_abs_gtX_flag’s are used. With these modifications, the following average Bjøntegaard delta rates are reported for the high-complexity encoder configuration (CTC, dependent quantization enabled):

version A (CE-7.1.3):	  0.05% AI,   0.05% RA,    0.08% LB;
	version B (CE-7.1.3b):	−0.20% AI, −0.14% RA,    (TBD)% LB.

For the low-complexity encoder configuration (dependent quantization and RDOQ disabled, but sign data hiding enabled), the following average Bjøntegaard delta rates are reported:

version A (CE-7.1.3):	  0.03% AI,   0.02% RA,    (TBD)% LB;
	version B (CE-7.1.3b):	  0.01% AI,   0.03% RA,    (TBD)% LB.

# Introduction

At the last JVET meeting, dependent quantization and a corresponding coefficient coding (as proposed in JVET-K0072 [3]) was adopted to the VTM [2].  A disadvantage of the adopted approach is that the maximum number of regular-coded bins (i.e., bins that are coded using an adaptive context model) is significantly increased relative to HEVC [5].  In VTM-2.0.1, up to 64 bins per subblock can be coded in the regular mode of the arithmetic coding engine (16 sig_flag’s, 16 par_flag’s, 16 gt1_flag’s, and 16 gt2_flag’s), whereas in HEVC, the maximum number of regular-coded bins is 25 (16 sig_flag’s, 8 gt1_flag’s, and 1 gt2_flag).

In this contribution, we propose modifications to the transform coefficient coding in VTM-2, which significantly reduce the maximum number of regular-coded bins.  The concept of dependent quantization is not changed, only the entropy coding of the transform coefficient levels (i.e., the quantization indexes).  The proposed entropy coding can be used for both quantization indexes for dependent scalar quantization and quantization indexes for conventional scalar quantization.

The contribution describes two alternative approaches for the entropy coding of transform coefficient levels. The two concepts differ in the order in which par_flag and gt1_flag are coded in the first pass for a subblock. Details for both approaches are described in the following sections.

# Proposed Transform Coefficient Coding

## CE-7.1.3 (version A)

Relative to VTM-2 [2], the first proposed approach for entropy coding of transform coefficient levels (tested in CE-7.1.3) contains the following modifications:

- the number of gt2_flag’s per subblock is limited to a maximum value of 2;

- the number of bins in the first pass for a subblock (sig_flag, par_flag, and gt1_flag) is limited to a maximum number of 34;

- the coefficients for which no sig_flag is coded in the first pass (due to reaching the limit of regular-coded bins) are completely coded in the bypass mode of the arithmetic coding engine using a Golomb-Rice code; the Rice parameter is derived based on the sum of absolute levels in a local template around the current transform coefficient level (the same template as in VTM-2 is used); the mapping of quantization index 0 depends on the state (for dependent quantization) and the sum of absolute levels in a local template;

- the Rice parameter for coding the remainder (for coefficients that are not completely coded in bypass mode) is derived similar to HEVC without referring to neighboring transform coefficient levels.

In comparison to the transform coefficient coding in VTM-2, the proposed version A has the following advantages:

- It is guaranteed that no more than 36 regular-coded bins (34 bins in the first pass and 2 bins in the second pass) have to be encoded or decoded for a subblock;

- For each transform coefficient, the local template has to be evaluated only once (either for determining sumAbs1 and numSig (see VTM-2) for deriving the contexts for the sig_flag, par_flag, gt1_flag, and gt2_flag or for determining sumAbs used for deriving the Rice parameter and the reordering index (see below) for the absolute levels completely coded in bypass mode).

Modified Subblock Syntax

The following pseudo code illustrates the proposed coefficient coding for 4x4 subblocks.  The Boolean variable firstSubblock specifies whether the current subblock is the first subblock in coding order (inside the transform block). The variable firstSigScanIdx specifies the scan index that corresponds to the position of the first significant transform coefficient in the transform block (the one for which the x and y location is explicitly stransmitted). minSubblockScanIdx and maxSubblockScanIdx represent the minimum and maximum values of the scan indexes for the current subblock.

regBins = 34 //maximum number of regular coded bins in first pass
gt2Bins = 2  //maximum number of gt2 flags

startScanIdx   = ( firstSubblock ? firstSigScanIdx : minSubblockScanIdx )
endScanIdx     = maxSubblockScanIdx
startIdxBypass = maxSubblockScanIdx + 1

// first pass (regular coded bins)
for( k = startScanIdx; k <= endScanIdx && regBins >= 3; k++ ) {
  coeff[ k ] = 0
  if( sig_flag cannot be inferred to be equal to 1 ) {
    decode sig_flag[ k ]
    regBins = regBins - 1
  }
  if( sig_flag != 0 ) {
    decode par_flag[ k ]
    decode gt1_flag[ k ]
    regBins = regBins – 2
    coeff[k] = 1 + par_flag[k] + 2 * gt1_flag[k]
  }
  if( regBins < 3 )
    startIdxBypass = k + 1
}

// second pass (regular coded bins)
startIdx2 = startIdxBypass
for( k = startScanIdx; k < startIdxBypass && gt2Bins > 0; k++ ) {
  if( gt1_flag[ k ] != 0 ) {
    decode gt2_flag[ k ]
    gt2Bins = gt2Bins – 1
    coeff[k] = coeff[k] + 2 * gt2_flag[k]
  }
  if( gt2Bins == 0 )
    startIdx2 = k + 1
}

// third pass (bypass coding of remainder)
for( k = startScanIdx; k < startIdx2; k++ ) {
  if( gt2_flag[ k ] != 0 ) {
    decode remainder[k]
    coeff[k] = coeff[k] + 2 * remainder[k]
  }
}
for( k = startIdx2; k < startIdxBypass; k++ ) {
  if( gt1_flag[ k ] != 0 ) {
    decode remainder[k]
    coeff[k] = coeff[k] + 2 * remainder[k]
  }
}

// fourth pass (bypass coding of complete coefficients)
for( k = startIdxBypass; k <= endScanIdx; k++ ) {
  decode absLevel[k]
  coeff[k] = absLevel[k]
}

// fifth pass (bypass coding of sign bins)
for( k = startScanIdx; k <= endScanIdx; k++ ) {
  if( coeff[ k ] != 0 )  {
    decode sign[k]
    if( sign[k] == 1 )
      coeff[k] = -coeff[k]
  }
}

Context modelling

The contexts for sig_flag[k], par_flag[k], gt1_flag[k], and gt2_flag[k] are derived in exactly the same way as in VTM-2.

The Rice parameter ricePar for coding the non-binary syntax element remainder[] is derived similar to HEVC:

- At the start of each subblock, ricePar is set equal to 0;

- After coding a syntax element remainder, the Rice parameter ricePar is modified as follows:
If ricePar is less than 3 and the last coded value of remainder is greater than ( (3 << ricePar) – 1 ), ricePar is incremented by 1.

For coding the non-binary syntax element absLevel[k], i.e., the absolute quantization indexes that are completely bypass-coded, the following applies:

- The sum of absolute values sumAbs in a local template is determined (the same template as in VTM-2 is used);

- The variables ricePar and posZero are determined by a table look-up according to

ricePar   = riceParTable[ min( 31, sumAbs ) ]
posZero = posZeroTable[ max( 0, state – 1 ) ][ min( 31, sumAbs ) ]

where the variable state represent the state for dependent quantization (it is equal to 0 when dependent quantization is disabled) and the tables riceParTable[] and posZeroTable[][] are given by

riceParTable[32] = {
  0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3
};

posZeroTable[3][32] = {
  {0,0,0,0,0,1,2,2,2,2,2,2,4,4,4,4,4,4,4,4,4,4,4,8,8,8,8,8,8,8,8,8},
  {1,1,1,1,2,3,4,4,4,6,6,6,8,8,8,8,8,8,12,12,12,12,12,12,12,12,16,16,16,16,16,16},
  {1,1,2,2,2,3,4,4,4,6,6,6,8,8,8,8,8,8,12,12,12,12,12,12,12,16,16,16,16,16,16,16}
};

- The intermediate variable codeValue is derived as follows:

  - If absLevel[k] is equal to 0, codeValue is set equal to posZero;

  - Otherwise, if absLevel[k] is less than or equal to posZero, codeValue is set equal to absLevel[k] – 1;

  - Otherwise (absLevel[k] is greater than posZero), codeValue is set equal to absLevel[k].

- The value of codeValue is coded using a Golomb-Rice code with Rice parameter ricePar.

At the decoder side, the non-binary syntax elements absLevel[k] are decoded as follows:

- The sum of absolute values sumAbs in a local template is determined as described above;

- The variables ricePar and posZero are determined as described above;

- The intermediate value codeValue is decoded using a Golomb-Rice code with Rice parameter ricePar;

- The value of absLevel[k] is derived as follows:

  - If codeValue is equal to posZero, absLevel[k] is set equal to 0;

  - Otherwise, if codeValue is less than posZero, absLevel[k] is set equal to codeValue + 1;

  - Otherwise (codeValue is greater than or equal to posZero), absLevel[k] is set equal to codeValue.

## CE-7.1.3 (version B with modified order of par_flag and gt1_flag)

Relative to VTM-2 [2], the second proposed approach for entropy coding of transform coefficient levels (tested in CE-7.1.3) contains the following modifications:

- the number of gt2_flag’s per subblock is limited to a maximum value of 4;

- the number of bins in the first pass for a subblock (sig_flag, par_flag, and gt1_flag) is limited to a maximum number of 28;

- the coefficients for which no sig_flag is coded in the first pass (due to reaching the limit of regular-coded bins) are completely coded in the bypass mode of the arithmetic coding engine using a Golomb-Rice code; the Rice parameter is derived based on the sum of absolute levels in a local template around the current transform coefficient level (the same template as in VTM-2 is used); the mapping of quantization index 0 depends on the state (for dependent quantization) and the sum of absolute levels in a local template;

- the Rice parameter for coding the remainder (for coefficients that are not completely coded in bypass mode) is derived similar to HEVC without referring to neighboring transform coefficient levels;

- the order of the flags par_flag and gt1_flag is swapped; that means, if sig_flag is equal to 1, first the gt1_flag is coded (which specifies whether the absolute level is greater than 1); if gt1_flag is equal to 1, the par_flag is additionally coded (it specifies the parity of the absolute level minus 2).

In comparison to the transform coefficient coding in VTM-2, the proposed version B has the following advantages:

- It is guaranteed that no more than 32 regular-coded bins (28 bins in the first pass and 4 bins in the second pass) have to be encoded or decoded for a subblock;

- For each transform coefficient, the local template has to be evaluated only once (either for determining sumAbs1 and numSig (see VTM-2) for deriving the contexts for the sig_flag, par_flag, gt1_flag, and gt2_flag or for determining sumAbs used for deriving the Rice parameter and the reordering index (see below) for the absolute levels completely coded in bypass mode);

- Due to the swapping of the order of par_flag and gt1_flag, the bin-to-bit ratio is reduced.

Modified Subblock Syntax

The following pseudo code illustrates the proposed coefficient coding for 4x4 subblocks.  The variables firstSubblock, firstSigScanIdx, minSubblockScanIdx and maxSubblockScanIdx have the same meaning as in sec. 2.1.

regBins = 28
gt2Bins = 4

startScanIdx   = ( firstSubblock ? firstSigScanIdx : minSubblockScanIdx )
endScanIdx     = maxSubblockScanIdx
startIdxBypass = maxSubblockScanIdx + 1

// first pass (regular coded bins)
for( k = startScanIdx; k <= endScanIdx && regBins >= 3; k++ )  {
  coeff[ k ] = 0
  if( sig_flag cannot be inferred to be equal to 1 ) {
    decode sig_flag[ k ]
    regBins = regBins - 1
  }
  if( sig_flag != 0 ) {
    decode gt1_flag[ k ]
    regBins = regBins – 1
    coeff[k] = 1 + gt1_flag[k]
    if( gt1_flag[k] != 0 ) {
      decode par_flag[ k ]
      regBins = regBins – 1
      coeff[k] = coeff[k] + par_flag[k]
    }
  }
  if( regBins < 3 )
    startIdxBypass = k + 1
}

// second pass (regular coded bins)
startIdx2 = startIdxBypass
for( k = startScanIdx; k < startIdxBypass && gt2Bins > 0; k++ ) {
  if( gt1_flag[ k ] != 0 ) {
    decode gt2_flag[ k ]
    gt2Bins = gt2Bins – 1
    coeff[k] = coeff[k] + 2 * gt2_flag[k]
  }
  if( gt2Bins == 0 )
    startIdx2 = k + 1
}

// third pass (bypass coding of remainder)
for( k = startScanIdx; k < startIdx2; k++ )  {
  if( gt2_flag[ k ] != 0 )  {
    decode remainder[k]
    coeff[k] = coeff[k] + 2 * remainder[k]
  }
}
for( k = startIdx2; k < startIdxBypass; k++ ) {
  if( gt1_flag[ k ] != 0 ) {
    decode remainder[k]
    coeff[k] = coeff[k] + 2 * remainder[k]
  }
}

// fourth pass (bypass coding of complete coefficients)
for( k = startIdxBypass; k <= endScanIdx; k++ ) {
  decode absLevel[k]
  coeff[k] = absLevel[k]
}

// fifth pass (bypass coding of sign bins)
for( k = startScanIdx; k <= endScanIdx; k++ ) {
  if( coeff[ k ] != 0 ) {
    decode sign[k]
    if( sign[k] == 1 )
      coeff[k] = -coeff[k]
  }
}

Context modelling

The contexts for sig_flag[k], par_flag[k], gt1_flag[k], and gt2_flag[k] are derived in exactly the same way as in VTM-2.

The Rice parameter ricePar for coding the non-binary syntax element remainder[] is derived as described in sec. 2.1.

The non-binary syntax elements absLevel[k] are coded as described in sec. 2.1.

## Difference between version A and version B

The main difference between version A (test CE-7.1.3) and version B (test CE-7.1.3b) is that, in version B, the order of the par_flag and gt1_flag are swapped and their meaning changed accordingly.  This has the following consequences:

- Version A (sec. 2.1) has less bin-to-bin dependencies than version B (sec. 2.2) in the first pass.  Similar to VTM-2, in version A, a sig_flag does not depend on a preceding gt1_flag and a gt1_flag does not depend on the preceding par_flag.  Whereas in version B, the presence of a par_flag does depend on the value of the preceding gt1_flag.

- Version B (sec. 2.2) results in smaller bin-to-bit ratios than version A (sec. 2.1).  In typical coding scenarios, most non-zero transform coefficient levels have an absolute value equal to 1.  In VTM-2 and version A, for transform coefficient levels with an absolute value of 1, three context-coded bins are transmitted, sig_flag equal to 1, par_flag equal to 0, and gt1_flag equal to 0. In contrast to that, in version B, only two context-coded bins are transmitted, sig_flag equal to 1 and gt1_flag equal to 0.  While the coding efficiency is nearly identical (see results below), the bin-to-bit ratio for version A (and VTM-2) is typically significantly larger than for version B.
As a consequence, with the current restriction on the maximum bin-to-bit ratio (which is inherited from AVC and HEVC), a higher percentage of cabac_zero_word’s is observed for version A than for version B.

## Chroma 2x2 subblock coding

Additional restrictions on number of coded regular bins per subblock are imposed on 2x2 chroma subblocks. Relative to CE7.1.3 version A and B methods described above, chroma 2x2 subblock coding contains the following modifications:

- the number of gt2_flag’s per subblock is limited to a maximum value of 2;

- the number of bins in the first pass for a subblock (sig_flag, par_flag, and gt1_flag) is limited to a maximum number of 6.

## CABAC initialization tables for Version B

In version B, the order of the syntax elements par_flag and gt1_flag was swapped relative to VTM-2.  As a consequence, these syntax elements have a different meaning and different statistics.  This aspect is taken into account by updating the CABAC initialization tables for sig_flag, par_flag. gt1_flag and gt2_flag for the binarization used in version B.

# Simulation Results

## Coding efficiency

According to the CE7 description [1], the proposed approaches for the transform coefficient coding were tested for two configurations:

- High-complexity encoder configuration:  This configuration is identical to the JVET common test conditions (CTC) and includes dependent quantization;

- Low-complexity encoder configuration:  In this configuration, both dependent quantization and RDOQ are disabled; all other aspects are the same as in the CTC.

Table 1 summarizes the average test results for the high-complexity encoder configuration and Table 2 summarizes the average test results for the low-complexity encoder configuration.

Table 1: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in high-complexity encoding mode (CTC, dependent quantization enabled).  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.11% | 0.18% | 109% | 100% | -0.14% | 0.00% | 0.11% | 108% | 101% |
| Class A2 | -0.04% | 0.04% | 0.02% | 110% | 101% | -1.04% | -0.63% | -0.63% | 109% | 102% |
| Class B | 0.03% | 0.18% | 0.14% | 110% | 99% | -0.04% | 0.04% | 0.12% | 110% | 99% |
| Class C | 0.05% | 0.17% | 0.32% | 111% | 100% | 0.04% | 0.08% | 0.19% | 110% | 101% |
| Class E | 0.03% | 0.12% | 0.07% | 109% | 100% | 0.13% | 0.08% | 0.11% | 109% | 101% |
| Overall | 0.03% | 0.13% | 0.15% | 110% | 100% | -0.18% | -0.06% | 0.00% | 109% | 101% |
| Class D | -0.03% | 0.28% | 0.15% | 111% | 98% | 0.04% | 0.27% | -0.03% | 111% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.23% | 0.21% | 104% | 94% | -0.06% | 0.03% | 0.13% | 103% | 93% |
| Class A2 | -0.01% | -0.06% | -0.04% | 105% | 101% | -0.21% | -0.19% | -0.19% | 104% | 101% |
| Class B | 0.03% | 0.09% | 0.14% | 104% | 102% | -0.10% | -0.10% | 0.07% | 104% | 103% |
| Class C | 0.05% | 0.13% | 0.30% | 105% | 100% | 0.06% | 0.18% | 0.31% | 105% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.03% | 0.10% | 0.16% | 105% | 100% | -0.07% | -0.02% | 0.09% | 104% | 100% |
| Class D | 0.04% | -0.23% | 0.17% | 105% | 99% | 0.10% | -0.09% | 0.49% | 105% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.12% | 0.19% | -0.17% | 105% | 105% | -0.25% | -0.14% | -0.33% | 104% | 103% |
| Class C | 0.16% | 0.20% | -0.13% | 105% | 104% | -0.06% | -0.32% | -0.28% | 104% | 104% |
| Class E | -0.17% | -0.02% | -0.18% | 105% | 108% | -0.09% | -0.43% | -0.51% | 103% | 105% |
| Overall | 0.06% | 0.14% | -0.16% | 105% | 105% | -0.15% | -0.27% | -0.36% | 104% | 104% |
| Class D | 0.15% | 0.70% | -0.14% | 105% | 102% | -0.06% | -0.15% | -0.01% | 104% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 2: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in low-complexity encoding mode (dependent quantization and RDOQ disabled).  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (low complexity encoding, QP=37,32,27,22) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.00% | 101% | 100% | -0.01% | 0.05% | -0.01% | 101% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 101% | 102% | -0.01% | 0.05% | 0.01% | 101% | 102% |
| Class B | 0.01% | 0.07% | 0.07% | 100% | 100% | 0.02% | 0.09% | 0.03% | 101% | 103% |
| Class C | 0.01% | 0.10% | 0.06% | 101% | 100% | 0.05% | 0.15% | 0.18% | 101% | 101% |
| Class E | 0.05% | 0.01% | 0.11% | 100% | 99% | 0.15% | 0.04% | 0.36% | 100% | 100% |
| Overall | 0.02% | 0.05% | 0.05% | 101% | 100% | 0.04% | 0.08% | 0.11% | 101% | 101% |
| Class D | 0.00% | 0.06% | 0.13% | 101% | 98% | 0.11% | 0.04% | 0.15% | 101% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (low complexity encoding, QP=37,32,27,22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.10% | 0.00% | 100% | 100% | 0.07% | 0.03% | 0.06% | 100% | 100% |
| Class A2 | 0.01% | -0.01% | -0.14% | 100% | 101% | 0.02% | 0.09% | -0.12% | 100% | 103% |
| Class B | 0.02% | 0.20% | 0.05% | 101% | 101% | 0.08% | 0.22% | -0.11% | 100% | 101% |
| Class C | 0.02% | 0.02% | -0.06% | 100% | 98% | 0.11% | -0.05% | -0.04% | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.09% | -0.03% | 100% | 100% | 0.08% | 0.08% | -0.06% | 100% | 101% |
| Class D | 0.00% | 0.01% | -0.08% | 100% | 97% | 0.15% | 0.12% | 0.19% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (low complexity encoding, QP=37,32,27,22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.03% | 0.18% | 101% | 104% | -0.01% | 0.10% | 0.41% | 101% | 104% |
| Class C | 0.03% | 0.07% | 0.06% | 100% | 104% | 0.05% | -0.09% | 0.16% | 99% | 104% |
| Class E | -0.03% | 0.26% | -1.33% | 101% | 101% | -0.05% | -0.15% | -0.39% | 100% | 101% |
| Overall | 0.01% | 0.08% | -0.24% | 101% | 103% | 0.00% | -0.03% | 0.12% | 100% | 103% |
| Class D | 0.00% | 0.25% | -0.27% | 102% | 102% | 0.06% | 0.17% | 0.19% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

When analyzing the bitstreams for the high-complexity encoder configuration, it can be observed that several bitstreams (QP=22) for the VTM-2 reference and version A contain cabac_zero_words, while none of the bitstreams for version B contains any cabac_zero_word.  In order to count out this effect, Table 3 shows the CTC (high-complexity encoding) results for the case that all cabac_zero_words are removed from both the VTM bitstreams and the bitstreams of the tested versions.

cabac_zero_word’s are not present in any bitstreams for the low-complexity encoder configuration.

Table 3: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in high-complexity encoding mode (dependent quantization enabled as specified in CTC) when all present cabac_zero_word’s are removed from the reference bitstreams and the bitstreams for the tested versions.  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (high complexity encoding, QP=37,32,27,22, without CZW) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.11% | 0.18% | 109% | 99% | -0.14% | 0.00% | 0.11% | 108% | 104% |
| Class A2 | 0.04% | 0.10% | 0.07% | 110% | 99% | -0.14% | -0.07% | -0.03% | 109% | 103% |
| Class B | 0.03% | 0.18% | 0.14% | 110% | 99% | -0.04% | 0.04% | 0.12% | 110% | 102% |
| Class C | 0.05% | 0.17% | 0.32% | 111% | 102% | 0.04% | 0.08% | 0.19% | 110% | 98% |
| Class E | 0.03% | 0.12% | 0.07% | 109% | 100% | 0.13% | 0.08% | 0.11% | 109% | 103% |
| Overall | 0.05% | 0.14% | 0.16% | 110% | 100% | -0.03% | 0.03% | 0.10% | 109% | 102% |
| Class D | -0.03% | 0.28% | 0.15% | 111% | 99% | 0.04% | 0.27% | -0.03% | 111% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (high complexity encoding, QP=37,32,27,22, without CZW) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.23% | 0.21% | 104% | 100% | -0.06% | 0.03% | 0.13% | 103% | 103% |
| Class A2 | 0.06% | 0.01% | 0.03% | 105% | 101% | -0.02% | 0.02% | 0.03% | 104% | 103% |
| Class B | 0.07% | 0.13% | 0.19% | 104% | 98% | 0.01% | 0.01% | 0.18% | 104% | 101% |
| Class C | 0.05% | 0.13% | 0.30% | 105% | 100% | 0.06% | 0.19% | 0.31% | 105% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.06% | 0.13% | 0.19% | 105% | 100% | 0.00% | 0.06% | 0.17% | 104% | 100% |
| Class D | 0.04% | -0.23% | 0.17% | 105% | 100% | 0.10% | -0.09% | 0.49% | 105% | 99% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (high complexity encoding, QP=37,32,27,22, without CZW) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.16% | 0.22% | -0.14% | 105% | 101% | -0.13% | -0.03% | -0.22% | 104% | 102% |
| Class C | 0.17% | 0.22% | -0.12% | 105% | 100% | -0.05% | -0.30% | -0.26% | 104% | 102% |
| Class E | -0.17% | -0.02% | -0.18% | 105% | 102% | -0.09% | -0.43% | -0.51% | 103% | 104% |
| Overall | 0.08% | 0.16% | -0.14% | 105% | 101% | -0.09% | -0.22% | -0.31% | 104% | 102% |
| Class D | 0.15% | 0.70% | -0.13% | 105% | 96% | -0.06% | -0.15% | -0.01% | 104% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Finally, Table 4 and Table 5 show average results for low QP values (QP=22, 17, 12, 7) and the high-complexity configuration. For the results in Table 4, cabac_zero_word’s were inserted according to the AVC/HEVC criterion (implemented in VTM-2). For the results in Table 5, the cabac_zero_word’s were removed from all bitstreams.

Table 4: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in high-complexity encoding mode (dependent quantization enabled as specified in CTC) for low QP values (22, 17, 12, 7).  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (high complexity encoding, QP=22,17,12,7) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.52% | -0.50% | 112% | 100% | -2.86% | -2.40% | -2.50% | 112% | 100% |
| Class A2 | -1.64% | -1.17% | -1.26% | 113% | 99% | -3.99% | -2.88% | -3.19% | 112% | 100% |
| Class B | -0.54% | -0.38% | -0.31% | 112% | 98% | -0.98% | -0.80% | -0.64% | 112% | 99% |
| Class C | -0.56% | -0.38% | -0.36% | 111% | 96% | -0.61% | -0.54% | -0.50% | 111% | 97% |
| Class E | -0.33% | -0.16% | -0.17% | 112% | 97% | -0.53% | -0.47% | -0.53% | 111% | 98% |
| Overall | -0.74% | -0.50% | -0.49% | 112% | 98% | -1.64% | -1.30% | -1.33% | 112% | 98% |
| Class D | -0.19% | 0.01% | -0.02% | 111% | 94% | -0.07% | 0.05% | 0.07% | 111% | 96% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (high complexity encoding, QP=22,17,12,7) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.30% | -0.21% | 109% | 100% | -3.54% | -2.98% | -2.76% | 108% | 98% |
| Class A2 | -1.78% | -1.51% | -1.56% | 109% | 99% | -6.99% | -6.47% | -6.55% | 108% | 99% |
| Class B | -1.21% | -0.83% | -0.84% | 107% | 99% | -3.93% | -3.26% | -3.02% | 107% | 99% |
| Class C | -1.14% | -0.98% | -0.79% | 106% | 96% | -3.92% | -3.89% | -2.96% | 106% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.20% | -0.90% | -0.84% | 108% | 99% | -4.46% | -4.01% | -3.66% | 107% | 98% |
| Class D | -0.80% | -0.71% | -0.46% | 106% | 92% | -2.25% | -2.23% | -1.77% | 105% | 93% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (high complexity encoding, QP=22,17,12,7) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.21% | -0.80% | -0.78% | 106% | 97% | -3.70% | -3.14% | -2.88% | 106% | 98% |
| Class C | -1.44% | -1.25% | -1.07% | 105% | 100% | -5.50% | -5.42% | -4.74% | 105% | 101% |
| Class E | -0.51% | -0.19% | -0.32% | 105% | 98% | -1.85% | -1.92% | -1.90% | 105% | 98% |
| Overall | -1.11% | -0.80% | -0.76% | 105% | 98% | -3.84% | -3.60% | -3.26% | 105% | 99% |
| Class D | -1.17% | -1.00% | -0.89% | 104% | 92% | -3.42% | -3.35% | -3.24% | 104% | 94% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 5: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in high-complexity encoding mode (dependent quantization and RDOQ disabled) for low QP values (22, 17, 12, 7) when all present cabac_zero_word’s are removed from the reference bitstreams and the bitstreams for the tested versions.  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (high complexity encoding, QP=22,17,12,7, without CZW) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | 0.63% | 0.64% | 112% | 98% | 0.06% | 0.25% | 0.24% | 112% | 98% |
| Class A2 | 0.39% | 0.68% | 0.67% | 113% | 98% | 0.17% | 0.41% | 0.39% | 112% | 98% |
| Class B | 0.07% | 0.21% | 0.22% | 112% | 97% | -0.02% | 0.07% | 0.10% | 112% | 95% |
| Class C | -0.10% | 0.16% | 0.13% | 111% | 98% | -0.08% | 0.08% | 0.06% | 111% | 95% |
| Class E | 0.13% | 0.30% | 0.34% | 112% | 97% | -0.07% | 0.00% | -0.02% | 111% | 95% |
| Overall | 0.14% | 0.36% | 0.37% | 112% | 98% | 0.00% | 0.15% | 0.14% | 112% | 96% |
| Class D | -0.19% | 0.01% | -0.02% | 111% | 95% | -0.07% | 0.05% | 0.07% | 111% | 93% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (high complexity encoding, QP=22,17,12,7, without CZW) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.59% | 1.00% | 0.99% | 109% | 99% | -0.17% | 0.21% | 0.25% | 108% | 101% |
| Class A2 | 0.70% | 1.18% | 1.16% | 109% | 98% | -0.06% | 0.23% | 0.23% | 108% | 98% |
| Class B | 0.44% | 0.71% | 0.70% | 107% | 99% | -0.04% | 0.11% | 0.20% | 107% | 96% |
| Class C | 0.28% | 0.49% | 0.41% | 106% | 97% | -0.04% | 0.10% | 0.15% | 106% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.48% | 0.81% | 0.77% | 108% | 98% | -0.07% | 0.15% | 0.20% | 107% | 97% |
| Class D | 0.23% | 0.46% | 0.48% | 106% | 94% | 0.13% | 0.30% | 0.36% | 105% | 91% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (high complexity encoding, QP=22,17,12,7, without CZW) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.50% | 0.81% | 0.80% | 105% | 99% | 0.01% | 0.22% | 0.29% | 106% | 99% |
| Class C | 0.35% | 0.63% | 0.59% | 105% | 100% | -0.03% | 0.10% | 0.14% | 104% | 100% |
| Class E | 0.50% | 1.02% | 0.85% | 105% | 99% | -0.36% | -0.04% | -0.07% | 105% | 97% |
| Overall | 0.45% | 0.80% | 0.74% | 105% | 97% | -0.10% | 0.12% | 0.15% | 105% | 100% |
| Class D | 0.32% | 0.60% | 0.60% | 104% | 95% | 0.07% | 0.19% | 0.19% | 104% | 94% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Coding efficiency results with chroma 2x2 subblock restrictions

In version 2 of this document, two aspects are modified relative to the versions tested in the CE:

- additional restriction are imposed for 2x2 chroma subblocks;

- CABAC initialization tables for sig_flag, par_flag, gtxX_flag’s are updated.

Table 6 summarizes the average test results for the high-complexity encoder configuration and Table 7 summarizes the average test results for the low-complexity encoder configuration.

Table 6: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in high-complexity encoding mode (CTC, dependent quantization enabled) with chroma 2x2 restrictions and, for version B, updated CABAC initialization tables.  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.07% | 0.18% | 112% | 108% | -0.13% | -0.05% | 0.08% | 108% | 102% |
| Class A2 | 0.01% | 0.04% | 0.04% | 113% | 107% | -1.02% | -0.61% | -0.66% | 109% | 103% |
| Class B | 0.03% | 0.18% | 0.22% | 113% | 103% | -0.06% | 0.01% | 0.08% | 110% | 101% |
| Class C | 0.07% | 0.19% | 0.39% | 114% | 102% | -0.01% | 0.05% | 0.23% | 111% | 102% |
| Class E | 0.05% | 0.10% | 0.13% | 113% | 105% | 0.05% | -0.10% | -0.07% | 109% | 101% |
| Overall | 0.05% | 0.13% | 0.21% | 113% | 104% | -0.20% | -0.11% | -0.03% | 109% | 102% |
| Class D | 0.04% | 0.23% | 0.11% | 113% | 103% | 0.00% | 0.06% | -0.03% | 111% | 100% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.14% | 0.19% | 106% | 103% | -0.10% | 0.07% | 0.08% | 104% | 96% |
| Class A2 | 0.01% | 0.14% | 0.07% | 107% | 103% | -0.24% | -0.10% | -0.15% | 104% | 105% |
| Class B | 0.03% | 0.02% | 0.07% | 105% | 101% | -0.18% | -0.18% | -0.11% | 105% | 106% |
| Class C | 0.07% | 0.07% | 0.19% | 107% | 100% | -0.03% | 0.12% | -0.04% | 106% | 106% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | 0.08% | 0.13% | 106% | 101% | -0.14% | -0.03% | -0.06% | 105% | 104% |
| Class D | 0.08% | -0.09% | 0.30% | 107% | 99% | -0.02% | -0.27% | 0.17% | 105% | 103% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (high complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.13% | 0.35% | 0.20% | 106% | 102% |  |  |  |  |  |
| Class C | 0.13% | 0.46% | 0.18% | 106% | 101% | -0.15% | -0.17% | -0.17% | 105% | 106% |
| Class E | -0.08% | 0.17% | -0.36% | 104% | 101% | -0.33% | -0.12% | -0.79% | 103% | 104% |
| Overall | 0.08% | 0.34% | 0.05% | 105% | 101% |  |  |  |  |  |
| Class D | 0.16% | 0.20% | -0.30% | 105% | 101% | -0.19% | 0.41% | 0.04% | 105% | 102% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 7: Average Bjøntegaard delta rate savings of the proposed coefficient coding relative to VTM-2.0.1 in low-complexity encoding mode (dependent quantization and RDOQ disabled) with chroma 2x2 restrictions and, for version B, updated CABAC initialization tables.  The table summarizes the results for both versions (A and B) tested in CE-7.1.3.

|  | All Intra (low complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.16% | -0.07% | 101% | 107% | -0.03% | 0.05% | -0.06% | 100% | 100% |
| Class A2 | 0.02% | 0.09% | 0.09% | 101% | 105% | -0.02% | 0.04% | -0.01% | 101% | 102% |
| Class B | 0.01% | 0.09% | 0.05% | 100% | 100% | 0.01% | 0.03% | 0.02% | 101% | 101% |
| Class C | 0.02% | 0.18% | 0.11% | 100% | 99% | 0.01% | 0.13% | 0.06% | 100% | 100% |
| Class E | 0.05% | 0.07% | 0.19% | 100% | 102% | 0.06% | -0.04% | 0.24% | 100% | 99% |
| Overall | 0.03% | 0.12% | 0.07% | 100% | 102% | 0.01% | 0.05% | 0.05% | 101% | 101% |
| Class D | 0.00% | 0.24% | 0.15% | 100% | 99% | 0.02% | 0.11% | 0.00% | 100% | 98% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access (low complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | -0.02% | 103% | 102% | 0.03% | -0.06% | 0.02% | 101% | 103% |
| Class A2 | -0.01% | 0.15% | 0.03% | 105% | 102% | -0.01% | -0.03% | -0.12% | 101% | 102% |
| Class B | 0.03% | 0.26% | -0.19% | 104% | 100% | 0.03% | 0.15% | -0.13% | 101% | 103% |
| Class C | 0.00% | -0.09% | 0.05% | 105% | 101% | 0.04% | 0.02% | -0.03% | 101% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.12% | -0.05% | 104% | 101% | 0.03% | 0.04% | -0.07% | 101% | 102% |
| Class D | -0.01% | -0.15% | 0.06% | 107% | 101% | -0.01% | -0.14% | -0.09% | 99% | 98% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B (low complexity encoding, QP=37, 32, 27, 22) |  |  |  |  |  |  |  |  |  |
|  | CE-7.1.3 (version A) |  |  |  |  | CE-7.1.3 (version B) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.03% | -0.17% | -0.07% | 100% | 98% | -0.09% | 0.00% | 0.19% | 104% | 107% |
| Class E | 0.06% | -0.20% | -1.17% | 100% | 96% | -0.04% | -0.32% | -0.46% | 102% | 107% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.94% | 0.08% | 101% | 104% | -0.10% | 0.62% | -0.27% | 100% | 98% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

## Analysis of bin-to-bit ratio / padding

One of the advantages of test 7.1.3b compared to test 7.1.3a and the VTM-2 is that, due to the changed binarization (swapping the order and meaning of par_flag and gt1_flag), the bin-to-bit ratio is significantly decreased.  For demonstrating the effect, we run simulations similar to that in JVET-L0276 [6].

We tested an extended QP range of {37, 32, 27, 22, 17, 12, 7} and measured the amount of padding bytes for both the bitstreams for test 7.1.3a and test 7.1.3b.  Except for the additional QP values, the common test conditions (CTC) are used.  The results in summarized in Figure 1.

Figure 1:  Percentage of padding bytes (cabac_zero_word’s) in bitstreams for test 7.1.3a and 7.1.3b for the common test conditions and QP values of 37, 32, 27, 22, 17, 12, 7.  The diagrams show (a) the average for a QP value, (b) the highest percentage for a bitstream (for the QP value) and (c) the highest percentage for an individual picture (for the QP value).

The following data are reported in Figure 1 (see also JVET-L0276):

- average percentage of padding bytes for all bitstream for a configuration (All Intra, Random Access, Low Delay B) and a QP value (base QP specified in configuration file);

- percentage of padding bytes in the worst bitstream (i.e., the bitstream with the highest percentage of padding bytes) for a configuration and QP value;

- percentage of padding bytes in the worst coded picture (i.e., the coded picture with the highest percentage of padding bytes) for a configuration and QP value.

The results demonstrate that the bin-to-bit ratio is significantly reduced.  When using the approach of test 7.1.3b, no padding data are included for the common test conditions.  The worst bitstreams for the extended QP range (random access / QP 17) contains only 0.8% of padding bytes.

# Conclusion

In this contribution, two alternative methods for reducing the maximum number of regular-coded bins are described.  Common to both methods is that

- the number of regular coded bins in the first pass (sig_flag, par_flag, gt1_flag) is limited to a pre-defined value (34 for version A, 28 for version B);

- the number of regular coded bins in the second pass (gt2_flag) is limited to a pre-defined value (2 for version A, 4 for version B);

- the Rice parameter for coding the remainder is derived without evaluating the already coded absolute values in a local template;

- the absolute values for coefficients for which no sig_flag is coded in the first pass are completely coded in bypass mode using a Golomb-Rice code with reordering; the Rice parameter and the reordering parameter are derived using a local template;

- for each transform coefficient, the local neighborhood is evaluated only once (either for the flags coded in the first pass or the absolute value coded in bypass mode).

The two proposed versions differ in the following aspect:

- Version A uses the same coding order of par_flag and gt1_flag as VTM-2;

- In version B, the order (and meaning) of par_flag and gt1_flag is modified.  As a consequence, the bin-to-bit ratio is generally reduced, but additional bin-to-bin dependencies are introduced.

# References

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-K1027, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” JVET-K1001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative Entropy Coding for Dependent Quantization,” JVET-K0072, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- J. Dong, M. Coban, M. Karczewicz, “CE7-Related: TCQ with High Throughput Coefficient Coding,” JVET-K0319, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- ITU-T and ISO/IEC, “High Efficiency Video Coding,” ITU-T Rec. H.265 | ISO/IEC 23008-8, version 5, Feb. 2018.

- H. Schwarz, T. Nguyen, “CE7-related: Analysis of padding bytes for VTM-2,” JVET-L0276, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).