| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, MO, 03–12 Oct. 2018 | Document: JVET-L0275-v1 |
| --- | --- |

| Title: | CE3: Intra reference sample interpolation filter (Test 3.1.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This contributions presents results of Core Experiment test CE3-3.1.4. Experimental results reportedly show -0.45% of luma BD-rate savings in all intra configuration and -0.17% of luma BD-rate savings over the BMS-2.0.1 software with the vtm configuration file (the suggested CE3 anchor).

# Introduction

This proposal reports test results for the experiment defined in [1]. The purpose of the test is to use the simplest design to achieve maximum coding performance and to prevent interference of WAIP, PDPC with the introduced 4-tap filters. The test comprises several modifications of the basis software that are described further. Intra prediction process by only directional intra modes is modified by this proposal. Non-directional intra prediction process is unmodified.

# Proposed techniques

This test proposes to modify the MDIS conditions in accordance with the aspect ratio of a block. The following operations are performed to get the MDIS conditions:

- An offset is determined and stored in dirOffset variable. The sign of this value is positive, when width of a block is greater than height of a block. Otherwise, dirOffset value is set negative. Magnitude of dirOffset is the same for the same aspect ratios. The following code snippet gives the description in a self-explanatory manner:

- static const char thrDirModeTable[4] = { 3, 5, 6, 7 }; //4 values of 3-bit length, i.e. this LUT requires 12 bits in total

- const int log2AspRatio = log2w - log2h;

- int dirOffset = 0, minLog2Length = log2w;

- if (log2AspRatio)

- {

- CHECK(abs(log2AspRatio) - 1 > 4, "Aspect ratio not supported");

- int arArg = abs(log2AspRatio) - 1;

- if (arArg > 3) arArg = 3;

- if (log2AspRatio < 0)

- {

- dirOffset += thrDirModeTable[arArg];

- minLog2Length = log2w;

- }

- else

- {

- dirOffset -= thrDirModeTable[arArg];

- minLog2Length = log2h;

- }

- }

- Based on dirOffset variable, adjustment of the MDIS threshold value is calculated (deltaMode variable):

- auto deltaMode = (dirMode>HOR_IDX && dirMode< VER_IDX) && ((dirMode>(DIA_IDX + dirOffset) && (log2AspRatio < 0)) || (dirMode<(DIA_IDX + dirOffset) && (log2AspRatio > 0))) ?

- (abs(log2AspRatio) >= 2 && minLog2Length <= 4 ? dirOffset : 0) : 0;

From the condition above, it could be noticed that modification is performed only for negative prediction angles (intra prediction mode is between HOR_IDX and VER_IDX). Another set of conditions “((dirMode>(DIA_IDX + dirOffset) && (log2AspRatio < 0)) || (dirMode<(DIA_IDX + dirOffset) && (log2AspRatio > 0)))” limit the intra prediction mode to use reference samples from the longer side of the block with additional offset (“dirOffset”) from the diagonal mode VDIA_IDX that depends on the aspect ratio.

The blocks affected by modification should have a shorter side length to be longer or equal to 16 samples and aspect ratio should be greater than 1:2.

- Finally, the distance to the closest orthogonal prediction mode (HOR_IDX or VER_IDX) is compared with the threshold value that is shifted along with dirOffset:

return (diff > m_aucIntraFilter[chType][log2Size]+deltaMode);

This test suggests usage of simplified 6-bit 4-tap filters, the selection between these filters are performed according to the MDIS condition for directional intra prediction modes that provide non-fractional displacements, i.e. to all the directional modes excluding the following: 2, HOR_IDX, DIA_IDX, VER_IDX, 66.

Depending on the intra prediction mode, the following reference samples processing is performed:

- The directional intra-prediction mode is classified into one of the following groups:

  - vertical or horizontal modes (HOR_IDX, VER_IDX),

  - diagonal modes that represent angles which are multiple of 45 degree (2, DIA_IDX, VDIA_IDX),

  - remaining directional modes;

- If the directional intra-prediction mode is classified as belonging to group A, then then no filters are applied to reference samples to generate predicted samples;

- Otherwise, if a mode falls into group B, then a [1, 2, 1] reference sample filter may be applied (depending on the MDIS condition) to reference samples to further copy these filtered values into an intra predictor according to the selected direction, but no interpolation filters are applied;

- Otherwise, if a mode is classified as belonging to group C, then only an intra reference sample interpolation filter is applied to reference samples to generate a predicted sample that falls into a fractional or integer position between reference samples according to a selected direction (no reference sample filtering is performed).

Filter coefficients used by this test are given in Table 1. Instead of “Cubic” filter used in BMS2.0.1 the test proposes to reuse 4-tap filter that is used for subpixel motion compensation of chroma component (“MC chroma filter”). Coefficients of “MC chroma filter” are not modified as compared to BMS2.0.1.

## Table 1. 6-bit coefficients of 4-tap filters used in the test

| Subpixel offset | MC chroma filter |  |  |  | Gauss filter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | c0 | c1 | c2 | c3 | c0 | c1 | c2 | c3 |
| 0 (integer) | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | -1 | 63 | 2 | 0 | 15 | 29 | 17 | 3 |
| 2 | -2 | 62 | 4 | 0 | 14 | 29 | 18 | 3 |
| 3 | -2 | 60 | 7 | -1 | 14 | 29 | 18 | 3 |
| 4 | -2 | 58 | 10 | -2 | 14 | 28 | 18 | 4 |
| 5 | -3 | 57 | 12 | -2 | 13 | 28 | 19 | 4 |
| 6 | -4 | 56 | 14 | -2 | 12 | 28 | 20 | 4 |
| 7 | -4 | 55 | 15 | -2 | 12 | 27 | 20 | 5 |
| 8 | -4 | 54 | 16 | -2 | 11 | 27 | 21 | 5 |
| 9 | -5 | 53 | 18 | -2 | 11 | 27 | 21 | 5 |
| 10 | -6 | 52 | 20 | -2 | 10 | 26 | 22 | 6 |
| 11 | -6 | 49 | 24 | -3 | 10 | 26 | 22 | 6 |
| 12 | -6 | 46 | 28 | -4 | 9 | 26 | 23 | 6 |
| 13 | -5 | 44 | 29 | -4 | 9 | 26 | 23 | 6 |
| 14 | -4 | 42 | 30 | -4 | 8 | 25 | 24 | 7 |
| 15 | -4 | 39 | 33 | -4 | 8 | 25 | 24 | 7 |
| 16 (half-pel) | -4 | 36 | 36 | -4 | 7 | 25 | 25 | 7 |
| 17 | -4 | 33 | 39 | -4 | 7 | 24 | 25 | 8 |
| 18 | -4 | 30 | 42 | -4 | 7 | 24 | 25 | 8 |
| 19 | -4 | 29 | 44 | -5 | 6 | 23 | 26 | 9 |
| 20 | -4 | 28 | 46 | -6 | 6 | 23 | 26 | 9 |
| 21 | -3 | 24 | 49 | -6 | 6 | 22 | 26 | 10 |
| 22 | -2 | 20 | 52 | -6 | 6 | 22 | 26 | 10 |
| 23 | -2 | 18 | 53 | -5 | 5 | 21 | 27 | 11 |
| 24 | -2 | 16 | 54 | -4 | 5 | 21 | 27 | 11 |
| 25 | -2 | 15 | 55 | -4 | 5 | 20 | 27 | 12 |
| 26 | -2 | 14 | 56 | -4 | 4 | 20 | 28 | 12 |
| 27 | -2 | 12 | 57 | -3 | 4 | 19 | 28 | 13 |
| 28 | -2 | 10 | 58 | -2 | 4 | 18 | 28 | 14 |
| 29 | -1 | 7 | 60 | -2 | 3 | 18 | 29 | 14 |
| 30 | 0 | 4 | 62 | -2 | 3 | 18 | 29 | 14 |
| 31 | 0 | 2 | 63 | -1 | 3 | 17 | 29 | 15 |

# Results

Experimental results were obtained using test conditions [2]. The proposed test is implemented on the basis of the CE3 anchor, i.e. BMS-2.0.1 software with vtm configuration file. Table 2 reportedly shows simulation results for AI and RA configuration over the CE3 anchor.

## Table 2. RD performance of CE3-3.1.4 on top of the CE3 anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.24% | -0.20% | 103% | 104% |
| Class A2 | -0.15% | -0.15% | -0.14% | 103% | 103% |
| Class B | -0.42% | -0.48% | -0.50% | 102% | 103% |
| Class C | -0.79% | -1.05% | -0.99% | 101% | 102% |
| Class E | -0.67% | -0.83% | -0.90% | 102% | 104% |
| Overall | -0.45% | -0.57% | -0.57% | 102% | 103% |
| Class D | -0.54% | -0.47% | -0.86% | 101% | 102% |
| Class F (optional) | -0.21% | -0.58% | -0.45% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.11% | 0.05% | 100% | 103% |
| Class A2 | -0.07% | -0.05% | -0.07% | 101% | 104% |
| Class B | -0.20% | -0.21% | -0.14% | 100% | 100% |
| Class C | -0.28% | -0.12% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.13% | -0.07% | 100% | 101% |
| Class D | -0.25% | -0.30% | 0.04% | 100% | 99% |
| Class F (optional) | 0.11% | 0.10% | 0.03% | 100% | 99% |

Testing CE3-3.1.4 without modification of MDIS conditions gives the following results (Table 3).

## Table 3. RD performance of CE3-3.1.4 without changing the MDIS conditions on top of the CE3 anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.16% | 102% | 103% |
| Class A2 | -0.15% | -0.19% | -0.23% | 102% | 103% |
| Class B | -0.42% | -0.47% | -0.49% | 102% | 103% |
| Class C | -0.78% | -0.96% | -0.98% | 101% | 102% |
| Class E | -0.67% | -0.84% | -0.88% | 101% | 103% |
| Overall | -0.45% | -0.54% | -0.57% | 102% | 103% |
| Class D | -0.53% | -0.51% | -0.83% | 101% | 102% |
| Class F (optional) | -0.20% | -0.56% | -0.41% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.10% | -0.08% | 101% | 103% |
| Class A2 | -0.09% | -0.07% | -0.11% | 102% | 104% |
| Class B | -0.19% | -0.06% | -0.04% | 101% | 102% |
| Class C | -0.29% | -0.12% | -0.10% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.09% | -0.08% | 101% | 103% |
| Class D | -0.24% | -0.16% | -0.01% | 101% | 103% |
| Class F (optional) | 0.14% | 0.02% | 0.13% | 101% | 101% |

Comparison of simulation results of CE3-3.1.4 without changing MDIS conditions and CE3-3.1.4 is shown in Table 4.

## Table 4. RD performance of CE3-3.1.4 without changing the MDIS conditions on top of CE3-3.1.4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-3.1.4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.04% | 99% | 99% |
| Class A2 | 0.00% | -0.03% | -0.08% | 99% | 100% |
| Class B | -0.01% | 0.01% | 0.01% | 99% | 100% |
| Class C | 0.01% | 0.09% | 0.01% | 99% | 100% |
| Class E | 0.00% | -0.01% | 0.02% | 99% | 100% |
| Overall | 0.00% | 0.03% | 0.00% | 99% | 100% |
| Class D | 0.01% | -0.05% | 0.03% | 99% | 99% |
| Class F (optional) | 0.00% | 0.02% | 0.05% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE3-3.1.4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.13% | 101% | 101% |
| Class A2 | -0.03% | -0.02% | -0.04% | 101% | 100% |
| Class B | 0.01% | 0.15% | 0.10% | 101% | 103% |
| Class C | -0.02% | 0.00% | -0.03% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | -0.01% | 101% | 102% |
| Class D | 0.01% | 0.14% | -0.05% | 101% | 103% |
| Class F (optional) | 0.03% | -0.08% | 0.11% | 101% | 102% |

# Conclusion

It is proposed to consider adoption of CE3-3.1.4 into the VVC Working Draft 3 and the VTM3.0 software.

# References

- G. Van der Auwera, J. Heo, A. Filippov, JVET-K1023 “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).