| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0276-v1 |
| --- | --- |

| Title: | CE7-related: Analysis of padding bytes for VTM-2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Tung Nguyen | Tel: Email: | +49 30 31002 226 heiko.schwarz@hhi.fraunhofer.de tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

It has been reported on the JVET reflector (bug tracker #89) that the current VTM-2.0.1 software generates a lot of padding data for some configurations.  In this document, an analysis of the actual amount of padding data (cabac_zero_word’s) for the common test conditions and an extended QP range (from 37 to 7) is presented.

# Introduction

The VTM-2 software includes a constraint on the maximum bin-to-bit ratio that is inherited from AVC and HEVC.  The constraint specifies that the number of bins in all VCL NAL units of a coded picture shall be less than or equal to a threshold maxBins given by

maxBins = (32 ÷ 3) * NumBytesInVclNalUnits + (RawMinCuBits * PicSizeInMinCbsY) ÷ 32,		(1)

where NumBytesInVlcNalUnits is the sum of the values of NumBytesInNalUnits for all VCL NAL units of a coded picture, RawMinCuBits specifies the number of raw data bits for the smallest coding unit size, and PicSizeInMinCbsY specifies the picture sizes in units of the smallest coding unit size. Hence, the term (RawMinCuBits * PicSizeInMinCbsY) can be interpreted as the number of raw data bits for the coded picture.

If the number of actually coded bins exceeds the threshold maxBins specified above, the encoder adds cabac_zero_words until NumBytesInVclNalUnits becomes large enough so that the above criterion is fulfilled.

# Analysis and Results

In the following, we analyze the amount of padding bytes (i.e., cabac_zero_word’s) that are contains in bitstreams generated with VTM-2.0.1. We tested an extended QP range of {37, 32, 27, 22, 17, 12, 7}. The following number are reported:

- average percentage of padding bytes for all bitstream for a configuration (All Intra, Random Access, Low Delay B) and a QP value (base QP specified in configuration file);

- percentage of padding bytes in the worst bitstream (i.e., the bitstream with the highest percentage of padding bytes) for a configuration and QP value;

- percentage of padding bytes in the worst coded picture (i.e., the coded picture with the highest percentage of padding bytes) for a configuration and QP value.

The results for the common test conditions and the extended QP range (QP=37, 32, 27, 22, 17, 12, 7) are summarized in the diagram of Figure 1. For QP values of 37, 32, 27 padding bytes are virtually not present (with exception of some individual frames for the low delay configuration).  However for smaller QP values, the percentage of padding bytes can become very high: About 7.5% on average for Random access and QP=17, about 15% for the worst sequence (random access, QP=17), and about 23.3% for the worst coded picture (random access, QP=17).

Figure 2 shows the amount of padding bytes, when the criterion in (1) is modified to

maxBins = (36 ÷ 3) * NumBytesInVclNalUnits + (RawMinCuBits * PicSizeInMinCbsY) ÷ 32,

i.e., the bin-to-bit ratio specified by the first term is increased by 12.5% (from 32/3 to 36/3).

Figure 3 shows the amount of padding bytes, when the criterion in (1) is modified to

maxBins = (40 ÷ 3) * NumBytesInVclNalUnits + (RawMinCuBits * PicSizeInMinCbsY) ÷ 32,

i.e., the bin-to-bit ratio specified by the first term is increased by 25% (from 32/3 to 40/3). In that case, the worst bitstream (random access, QP=17) includes only 0.5% padding data. By further increasing the maximum bin-to-bit ratio specified by the first term to (42/3), none of the bitstreams would contain any padding bytes.

Figure 1:  Percentage of padding bytes (cabac_zero_word’s) in bitstreams generated by VTM-2.0.1. for the common test conditions and QP values of 37, 32, 27, 22, 17, 12, 7.  The diagrams show (a) the average for a QP value, (b) the highest percentage for a bitstream (for the QP value) and (c) the highest percentage for a individual picture (for the QP value).

Figure 2:  Percentage of padding bytes (cabac_zero_word’s) when the criterion (1) or the maximum number of bins in the VCL NAL units for a coded picture is modified to
maxBins = (36 ÷ 3) * NumBytesInVclNalUnits + (RawMinCuBits * PicSizeInMinCbsY) ÷ 32.

Figure 3:  Percentage of padding bytes (cabac_zero_word’s) when the criterion (1) or the maximum number of bins in the VCL NAL units for a coded picture is modified to
maxBins = (40 ÷ 3) * NumBytesInVclNalUnits + (RawMinCuBits * PicSizeInMinCbsY) ÷ 32.