| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0279 |
| --- | --- |

| Title: | CE3-related: Unification of angular intra prediction for square and non-square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Shan Liu and Xiang Li | Email: | {leolzhao, xinzzhao, shanl, xlxiangli }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, two modifications are proposed to unify the angular intra prediction for square and non-square blocks. Firstly, angular prediction directions are slightly modified to cover diagonal directions of all block shapes. Secondly, it is proposed to keep all angular directions within the range between bottom-left diagonal direction and top-right diagonal direction for all block sizes. Simulation results reportedly show that, with almost no impact on the encoder or decoder run-time, average -0.06% BD-rate gain is achieved on top of VTM2.0.1 under all intra (AI) configuration.

Problem Statement

As shown in Figure 1, there are 85 angular direction modes together with Planar and DC modes in VTM2.0.1, where 20 angular directions are beyond the range between 45 degrees and -135 degrees, namely wide angles. The angular directions within 45 degrees (mode 66) to -135 degrees (mode 2) in clockwise direction was designed for square blocks. These diagonal directions (mode 2, 34, and 66) for square blocks are all included. However, the diagonal directions for non-square block shapes are not always covered. In addition, the angular directions for square blocks start from bottom-left diagonal direction to top-right diagonal direction whereas several angular directions for non-square blocks are still outside this direction range, which is not a unified design.

Figure 1: Intra prediction modes in VTM2.0.1

Proposed method

Firstly, it is proposed to modify the intra prediction angles so that the diagonal directions of all block shapes are included in the angular directions. In current VTM, the aspect ratio of a intra coding unit can only be M:1 or 1:M, where M can be1, 2, 4, 8, or 16, so the angular direction α with tan(α) equal to {2/32, 4/32, 8/32, 16/32, 32/32} are included into the available angular directions. To implement it, the modified values in angTable is shown in Table 1. With this modification, more integer-position samples are used as predictor so that interpolation error is further reduced.

Table 1: Modified angular direction table

| Original table | 0 | 1 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 49 | 54 | 60 | 68 | 79 | 93 | 114 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modified table | 0 | 1 | 2 | 3 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 | 73 | 86 | 102 | 128 |

Secondly, it is proposed to keep all available angular directions within the range between bottom-left diagonal direction and top-right diagonal direction for all block sizes. In current VTM2.0.1, when the aspect ratio of the coding unit is larger than four, there are certain angular directions outside the range between bottom-left and top-right diagonal direction, which is shown in Table 2. In the proposed method, these modes are mapped to wide angle modes within the range between bottom-left and top-right diagonal direction. Figure 3 (b) and (c) show two examples about how these modes are mapped, where Mode A and Mode B are the modes outside the diagonal direction range and the mapped mode of Mode A is also the diagonal direction. It is also shown that, with the proposed two modifications, all angular directions are from the bottom-left diagonal direction to top-right diagonal direction for all block shapes.

Table 2: Intra prediction modes outside the range between the bottom-left and top-right diagonal direction.

| Aspect ratio | Modes outside the diagonal direction range |
| --- | --- |
| W / H == 1/8 | Modes 55, 56 |
| W / H == 1/16 | Modes 53, 54, 55, 56 |
| W / H == 8 | Modes 12, 13 |
| W / H == 16 | Modes 12, 13, 14, 15 |

In addition, with the proposed method, the number of reference samples in the top reference row and left reference column can be restricted to 2 * width + 1 and 2 * height + 1 for all block shapes.

(a)

(b)

(c)

Figure 3 Illustrations of mode mapping process for modes outside the diagonal direction range

Experimental results

The proposed method is implemented on top of BMS2.0.1 [1] software. The simulations are performed on Linux cluster using VTM AI and RA configurations under the common test condition described in JVET-K1010 [2].

Table 3: Coding performance of 1st modification over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.03% | 100% | 99% | -0.02% | -0.02% | 0.02% | 99% | 99% |
| Class A2 | -0.06% | -0.04% | -0.11% | 101% | 101% | -0.01% | 0.01% | -0.11% | 100% | 100% |
| Class B | -0.08% | -0.16% | -0.06% | 98% | 98% | -0.01% | -0.08% | 0.01% | 100% | 100% |
| Class C | -0.09% | -0.07% | -0.03% | 100% | 104% | -0.01% | 0.13% | -0.08% | 100% | 100% |
| Class E | -0.06% | 0.06% | 0.02% | 98% | 98% |  |  |  |  |  |
| Overall | -0.06% | -0.06% | -0.04% | 99% | 100% | -0.01% | 0.00% | -0.04% | 100% | 100% |
| Class D | -0.01% | 0.15% | -0.08% | 99% | 94% | 0.05% | 0.06% | 0.30% | 100% | 103% |
| Class F | -0.04% | -0.07% | -0.03% | 100% | 103% | -0.03% | 0.10% | 0.04% | 99% | 101% |

Table 4: Coding performance of 1st + 2nd modifications over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | -0.04% | 97% | 93% | 0.00% | 0.03% | -0.01% | 96% | 96% |
| Class A2 | -0.06% | -0.09% | -0.10% | 97% | 96% | -0.03% | -0.17% | -0.11% | 96% | 95% |
| Class B | -0.09% | -0.15% | -0.10% | 96% | 95% | -0.03% | -0.12% | -0.17% | 98% | 98% |
| Class C | -0.10% | -0.11% | 0.06% | 99% | 98% | 0.02% | 0.04% | -0.05% | 99% | 96% |
| Class E | -0.06% | 0.04% | 0.00% | 97% | 97% |  |  |  |  |  |
| Overall | -0.06% | -0.08% | -0.04% | 97% | 96% | -0.01% | -0.06% | -0.09% | 97% | 97% |
| Class D | 0.01% | 0.17% | -0.01% | 99% | 89% | 0.00% | -0.20% | 0.11% | 100% | 100% |
| Class F | -0.05% | -0.11% | -0.16% | 99% | 100% | -0.01% | 0.04% | 0.02% | 99% | 99% |

Spec text changes

The following text describes the proposed method. It is based on VVC draft2 v6 [3] section 8.2.4.2. Modifications with regard to this proposal are highlighted in yellow.

8.2.4.2	Intra sample prediction

8.2.4.2.1	General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable whRatio is set equal to Min( Abs( Log2( nTbW / nTbH ) ), 2 ).

The variables refW and refH are derived as follows:

refW = ( nTbH > nTbW )  ?  ( nTbW + ( nTbH >> whRatio ) + Ceil( nTbH / 32 ) )  :  ( nTbW * 2 ) 	(8-16)

refH = ( nTbW > nTbH )  ?  ( nTbH + ( nTbW >> whRatio ) + Ceil( nTbW / 32 ) )  :  ( nTbH * 2 ) 	(8-17)

For the generation of the reference samples p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.2.4.2.2 is invoked with the sample location ( xTbCmp, yTbCmp ), the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.2.4.2.3 is invoked with the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

- The reference sample filtering process as specified in clause 8.2.4.2.4 is invoked with the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.2.4.2.5 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.2.4.2.6 is invoked with the transform block width nTbW, the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_CCLM, the corresponding intra prediction mode process specified in clause 8.2.4.2.8 is invoked with the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.2.4.2.7 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, and the reference sample array p as inputs, and the modified intra prediction mode predModeIntra and the predicted sample array predSamples as outputs.

When one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

Reference sample filtering process

Inputs to this process are:

- the (unfiltered) neighbouring samples refUnfilt[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable whRatio is set equal to Min( Abs( Log2( nTbW / nTbH ) ), 2 ).

The variable wideAngle is derived as follows:

- If all of the following conditions are true, wideAngle is set equal to 1.

- nTbW is greater than nTbH

- predModeIntra is greater than or equal to 2

- predModeIntra is less than ( whRatio > 1 ) ? 128 + 2 * whRatio : 8

- Otherwise, if all of the following conditions are true, wideAngle is set equal to 1.

- nTbH is greater than nTbW

- predModeIntra is less than or equal to 66

- predModeIntra is greater than ( whRatio > 1 ) ? 5660 – 2 * whRatio : 60

- Otherwise, wideAngle is set to 0.

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0:

- predModeIntra is equal to INTRA_DC,

- predModeIntra is equal to INTRA_PLANAR and nTbW * nTbH is less than or equal to 32,

- cIdx is not equal to 0.

- Otherwise, if predModeIntra is equal to INTRA_PLANAR  and nTbW * nTbH is greater than  32, filterFlag is set equal to 1.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ] or wideAngle is equal to 1, filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 20 | 14 | 2 | 0 | 20 | 0 |

For the derivation of the reference samples p[ x ][ y ] the following applies:

- If filterFlag is equal to 1, the filtered sample values p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 are derived as follows:

p[ −1 ][ −1 ] = ( refUnfilt[ −1 ][ 0 ] + 2 * refUnfilt[ −1 ][ −1 ] + refUnfilt[ 0 ][ −1 ] + 2 )  >>  2	(8-21)

p[ −1 ][ y ] = ( refUnfilt[ −1 ][ y + 1 ] + 2 * refUnfilt[ −1 ][ y ] + refUnfilt[ −1 ][ y − 1 ] + 2 )  >>  2
	for y = 0..refH − 2		(8-22)

p[ −1 ][ refH − 1 ] = refUnfilt[ −1 ][ refH − 1 ]		(8-23)

p[ x ][ −1 ] = ( refUnfilt[ x − 1 ][ −1 ] + 2 * refUnfilt[ x ][ −1 ] + refUnfilt[ x + 1 ][ −1 ] + 2 )  >>  2
	for x = 0..refW − 2 		(8-24)

p[ refW − 1 ][ −1 ] = refUnfilt[ refW − 1 ][ −1 ]		(8-25)

- Otherwise, the reference samples values p[ x ][ y ] are set equal to the unfiltered sample values refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1.

        - Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- the neighbouring samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1

Outputs of this process are the modified intra prediction mode predModeIntra and the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable whRatio is set equal to Min( Abs( Log2( nTbW / nTbH ) ), 2 ).

For non-square blocks (nTbW is not equal to nTbH), the intra prediction mode predModeIntra is modified as follows:

- If all of the following conditions are true, predModeIntra is set equal to ( predModeIntra + 65 ).

- nTbW is greater than nTbH

- predModeIntra is greater than or equal to 2

- predModeIntra is less than ( whRatio > 1 ) ? 128 + 2 * whRatio  : 8

- Otherwise, if all of the following conditions are true, predModeIntra is set equal to ( predModeIntra − 67 ).

- nTbH is greater than nTbW

- predModeIntra is less than or equal to 66

- predModeIntra is greater than ( whRatio > 1 ) ? 5660 – 2 * whRatio  : 60

Figure 8-1 – Intra prediction directions (informative)

Figure 8-1 illustrates the 8593 prediction directions, where the dashed directions are associated with the wide-angle modes that are only applied to non-square blocks.

Table 8-5 –  specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Table 8-5 – Specification of intraPredAngle

| predModeIntra |  |  |  |  |  |  |  |  |  |  |  |  |  | -14 | -13 | -12 | -11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle |  |  |  |  |  |  |  |  |  |  |  |  |  | 512 | 341 | 256 | 171 |
| predModeIntra | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| intraPredAngle | 114128 | 93102 | 7986 | 6873 | 6064 | 5457 | 4951 | 45 | 39 | 35 | 32 | 29 | 26 | 23 | 210 | 198 | 176 |
| predModeIntra | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| intraPredAngle | 154 | 132 | 110 | 98 | 76 | 54 | 3 | 2 | 1 | 0 | −1 | −2 | −3 | −54 | −76 | −98 | −110 |
| predModeIntra | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 |
| intraPredAngle | −132 | −154 | −176 | −198 | −210 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −210 | −198 | −176 | −154 | −132 |
| predModeIntra | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| intraPredAngle | −110 | −98 | −76 | −54 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 54 | 76 | 98 | 110 | 132 | 154 |
| predModeIntra | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 |
| intraPredAngle | 176 | 198 | 210 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 4951 | 5457 | 6064 | 6873 | 7986 | 93102 | 114128 |
| predModeIntra | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |  |  |  |  |

The inverse angle parameter invAngle is derived based on intraPredAngle as follows:

invAngle = Round256*32intraPredAngle		(8-33)

…...

- Conclusion

In this contribution, two modifications are proposed to unify the angular intra prediction for square and non-square blocks. The proposed method does not increase the encoding and decoding run time and average -0.06% BD-rate gain is achieved on top of VTM2.0.1 under AI configurations. It is proposed to adopt this proposal into VTM.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1001, Ljubljana, SI, 10–18 Jul. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).