| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0281 |
| --- | --- |

| Title: | CE4-related: Size constrain for inherited affine motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The inherited affine motion prediction requires extra storage for the control-point motion vectors (CPMVs) of the neighboring coded blocks. This contribution proposes size constrain for inherited affine motion prediction that reduces the memory size for CPMV storage. The average bd-rates when compares to VTM2.0.1 are 0.03%, 0.07%, -0.01%  in RA and -0.01%, 0.07%, -0.11% in LB.

# Introduction

The inherited affine motion prediction uses CPMVs of a neighboring affine coded block to derive the predicted CPMVs of current block. Therefore the CPMVs need to be stored for the inherited affine motion prediction. In the current VTM2.0.1, the CPMVs are stored mixed with the sub-block motion vectors. The CPMVs are stored in the top-left, top-right and bottom-left (if a PU uses 6-parameter affine motion models) sub-block locations of the sub-block motion vector field, overwriting the computed sub-block vectors for those locations. So, no additional buffer is used for the CPMV storage. However, a separate sub-block motion vector field is generated again for motion compensation. As discussed in [1], for a cost-sensitive decoder implementation, the sub-block motion vector field cannot be overwritten by CPMVs before it’s consumed by the motion compensation. Two methods are proposed in [1] to address this issue. One is to store CPMVs in a separate buffer, the other is use the same mixed MV field of CPMVs and sub-block vectors are used for motion compensation. The first method requires 768 more bytes than the second method in total memory size. The second method introduces nonnegligible loss in coding some sequences.

# Proposal

In this contribution, it’s propose to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than 16. So, if the width or height of a neighboring affine coded block is less than 16, the inherited affine motion prediction from the block is treated as unavailable.

The proposed method is applied on top of method 1 in [1], wherein CPMVs are stored in a separate buffer. The CPMV storage can then be in 16-pixel segment instead of 8-pixel segment. The memory size for CPMV storage is reduced by half.

However, noted that the proposed method can also be applied on top of current VTM2.0.1, the local buffer for CPMV is reduced to ¼ of the orignal. The local buffers for left/top-left/top neighboring CPMV contexts is reduced by half. If the proposed method is applied on top of method 2 in [1], local buffers for left/top-left/top neighboring CPMV contexts is reduced by half as well.

The summary of motion vector memory buffer usage for different methods is referenced here in Table 1, then summary after applying the proposed method is updated as in Table 2.

Table 1 summary of motion vector memory buffer usage for different methods in [1]

| Method | Local sub-block MV buffer (bytes) | Local CPMV buffer (bytes) | Local buffers for left/top-left/top neighboring MV contexts (bytes) | Local buffers for left/top-left/top neighboring CPMV contexts (bytes) | Total local memory size (bytes) |
| --- | --- | --- | --- | --- | --- |
| 128x128 CTU based VTM2.0.1 | 8,192 | 6,144 | 256 (left only) | 384 (left only) | 14,976 |
| 64x64 based VTM2.0.1 | 2,048 | 1,536 | 768 | 1,152 | 5,504 |
| 64x64 based clean-up method 1 | 2,048 |  | 768 | 1,152 | 3,968 |
| 64x64 based clean-up method 2 | 2,048 |  | 768 | 384 | 3,200 |

Table 2 Updated summary of motion vector memory buffer usage for different methods

| Method | Local sub-block MV buffer (bytes) | Local CPMV buffer (bytes) | Local buffers for left/top-left/top neighboring MV contexts (bytes) | Local buffers for left/top-left/top neighboring CPMV contexts (bytes) | Total local memory size (bytes) |
| --- | --- | --- | --- | --- | --- |
| 128x128 CTU based VTM2.0.1 | 8,192 | 1,536 | 256 (left only) | 192 (left only) | 10,176 |
| 64x64 based VTM2.0.1 | 2,048 | 384 | 768 | 576 | 3,776 |
| 64x64 based clean-up method 1 in [1] | 2,048 |  | 768 | 576 | 3392 |
| 64x64 based clean-up method 2 in [1] | 2,048 |  | 768 | 192 | 3,008 |

# Test results

The random access (RA) and low delay B (LB) results under common test condition are reported in Table 3. The overall average bd-rate are 0.03%, 0.07%, -0.01% in RA, and -0.01%, 0.07%, -0.11% in LB. The peak luma bd-rates loss in in RA is 0.16% when coding the sequence BQSquare. The bd-rate results in method 1 of [1] are are 0.01%, 0.05%, 0.01% in RA, and -0.03%, 0.13%, -0.09% in LD_B.

Table 3 Test result

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | -0.01% | 104% | 116% |
| Class A2 | 0.05% | 0.04% | -0.03% | 98% | 99% |
| Class B | 0.02% | -0.01% | -0.05% | 83% | 95% |
| Class C | 0.04% | 0.14% | 0.05% | 82% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.07% | -0.01% | 89% | 99% |
| Class D | 0.05% | 0.10% | -0.09% | 81% | 84% |
| Class F (optional) | 0.01% | 0.05% | 0.05% | 81% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.16% | -0.09% | 103% | 119% |
| Class C | 0.02% | 0.11% | -0.09% | 96% | 98% |
| Class E | -0.08% | 0.39% | -0.16% | 82% | 96% |
| Overall | -0.01% | 0.07% | -0.11% | 95% | 106% |
| Class D | 0.04% | 0.57% | -0.43% | 81% | 85% |
| Class F (optional) | -0.06% | 0.51% | 0.73% | 81% | 92% |

# References

[1] Minhua Zhou and Brian Heng, “CE4-related: A clean up for affine mode”, JVET-L0047, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).